FMUSER IPTV HD STB Android 7.1 OS Dual-band Wi-Fi 10/100M Ethernet Input HDMI 2.0 Output 4K H.265 Decoding IPTV Set-Top Box for Hotel Hospital IPTV Systems

△ Android 7.1.2

△ HDMI 2.0

△ Low Cost

△ Personalized User Interface (Hotel/Hospital/Community…)

△ Advertisement System (ADS) Imbedded

Product overview

The FMUSER IPTV HD STB is an Android 7.1-based set-top box with 4K H.265/HEVC decoding and HDMI 2.0 output. It processes IPTV streams via 10/100M Ethernet or dual-band Wi-Fi, supporting Widevine L1 DRM and custom EPG interfaces. The compact device integrates ad systems and runs commercial apps, ideal for hospitality TV systems and community IPTV deployments.

Key feature

Android 7.1 flexibility: Supports Netflix, Skype, and third-party apps via Google Play

Specifications

| Hardware | CPU | RK3229, 28nm Quad core Cortex A7, High cost performance 4K OTT Box solution |
| --- | --- | --- |
|  | GPU | ARM Mali-400 GPU |
|  | HDMI | Including an HDMI 2.0 for HDMI 2.0 for 4K YUV420@60Hz with HDCP 1.4/2.2 |
|  |  | Support 4K 10-bit H.265/H.264/VP9 video playback and display |
|  | Memory | 1GB DDRIII |
|  | Flash | 8GB Nand Flash |
|  | WiFi | 2.4G，802.11 b/g/n, |
|  | Wired network | Ethernet GMAC 10/100/100M Ethernet controller |
|  |  | Support 10/100/1000 data transfer rate and RGMII interface |
|  |  | Support 10/100Mbps data transfer rate and RGMII interface |
|  | IR | Infra Remote control |
|  | Power Supply | DC input 5V/2A |
| Software | OS | Android 7.1 |
|  | 3D | Hardward 3D graphics acceleration |
|  | Video decoding | 4K VP9 Video decoder, 4K 10-bits H.265/H.264 video decoder |
|  |  | 1080P other video decoders (VC-1, MPEG-1/2/4, VP6/8) |
|  |  | Support conversion from BT.2020 to Rec.709/Rec.601 |
|  |  | Video post processor: de-interlace |
|  |  | H.263/H.264 2160P@ 24fps VC-1 ,MPEG - 1 1080p @ 60fps MPEG- 2 2160P @ 24fps MPEG-4 1080P @ 60fps AVS 1080p @ 60fps MVC 2160P @ 24fps WMV,3GP,MKV,ASF,3G2M4V,MJPEG,RV8,RV9,RV10 1080P@60fps |
|  |  | Support HEVC decoder |
|  |  | Support Main/Main10 HEVC/H.265 decoder, 4K @ 60FPS |
|  |  | Support 4096x2304 resolution |
|  |  | Support HDMI 720P-2160P FUII 3D HD( 2160P, 3840 * 2160 ), 4K+2 K |
|  |  | Support flash11.1 |
|  | Video Decoder format | H.264,H.265,VP8,MVC(1080P) |
|  | Video format | Supports DIVD/DIVX/REAL8 / 9 / 10, RV, RM, RMVB, PMP, FLV, MP4, M4V, VOB, VP6, VP8 2160P @ 24fps |
|  | Audio format | Supports MP1, MP2, MP3, WMA, OGG, AAC, M4A , FLAC, APE, AMR, RA, WAV, etc |
|  | Picture format | Supports JPG, JPEG, BMP, GIF, PNG, JFIF etc Resolution 8192x8192 |
|  | HDD file system | FAT16 / FAT32 / NTFS |
|  | Support Subtitle | SRT, SMI, SUB, SSA, IDX+USB |
|  | XBMC/KODI | Support |
|  | DRM | Widewine Level1, DRM PlayReady ,Verimatrix, built in Trustzone, HDCP 2.2/1.4 |
|  | Support language | Chinese, English, Germany, Japanese, Korea etc. multilateral languages |
| Stream In/Out | 1*HDMI 2.0 | Built-in HDMI 2.0 transmitter including both controller and PHY with CEC and HDCP, 4K @ 60FPS max resolution output |
|  | 1* SD Card | to 64GB |
|  | 4*USB HOST | 2High speed USB 2.0,support USB DISK and USB HDD |
|  | 1*SPDIF | SPDIF output |
|  | 1*RJ45 | Ethernet:10/100M, standard RJ-45 |
|  | 1*Power input | 5V/2A |
|  | Accessories | Gift box Power charger adapter HDMI cable (optional) IR remote control (optional) User's Manual |
| Applications | Online | Browse all video websites, support Netflix, Hulu, Flixster, youtube etc |
|  | Apps | Apps download freely form android market, amazon appstore etc |
|  | Medium | Local Media playback, support HDD, U disck ,SD card |
|  | Talk online | Support SKYPE video call, MSN, facebook, twitter, QQ etc |
|  | Others | Support WiFi hotpiont, could be as a wireless router Support DLNA, Google TV Remote, LAN, 3D Moive, Email, office suit etc |
| Packing | Dimension | 118*118*25 mm |
|  | Approx weight | 400g |

System Construction

Advantages

Personalized EPG Interface

* Personalized EPG interface for hotel, hospital, Community and other industries.

* System wallpaper is changeable and EPG content is editable.

* Various business modes blend in IPTV system

* Support modular interface and convenient to docking with third part manage software

Advertisement System—Value added Services

* Hotel, hospital and community version can support set up advertisement business to increase revenues.

* Ad modes include booting ad, Menu ad (only for hospital and community version), VOD ad, pictures and text ad etc.

Mass HD Programs

* 1080P/60Hz full HD video

* Mass resource of Video on Demand including movies, entertainments, factual shows and etc

* Live programs synchronized broadcast

Strength and After-sale Services

* FMUSER IPTV system is with solid quality based on FMUSER’s own DTV head-end equipment

* The whole system is self research and developed, which makes it easy for the after-sale maintenances

* Skilled after-sale supporting team

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227