FUQ-72XX Series FM Stereo Broadcast Transmitter 10W-10kW Adjustable Output FPGA Processed LDMOS Amplification GB-T4311-2000 GY-T169-2001 Compliant Dual-Transmitter Redundancy Coaxial Switching

Product overview

The FUQ-72XX series delivers FPGA-processed FM stereo signals with LDMOS amplification, featuring adjustable 10W–10kW RF output and AGC stability (±0.1dB). Its dual-transmitter redundancy with coaxial switching ensures 24/7 operation, compliant with GB/T4311-2000/GY/T169-2001 standards. The 4U rack-mount design supports NMS control for broadcast centers and emergency alert systems.

Key feature

Integrated lightning protection (10kV surge resistance)

Technical Specifications

| S/No. | Item |  | Unit | Technical Index |  | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Output Power |  | W | 500 |  |  |
| 2 | Frequency |  | MHz | 1kW≤ | 65.5~87; 87~108 |  |
|  |  |  |  | 1kW＞ | 87～108 |  |
| 3 | Frequency deviation |  | Hz | ≤500 |  |  |
| 4 | Sparious Radiation |  | dB | ≤-70 |  |  |
| 5 | parasitic amplitude modulation noise |  | dB | -60 |  |  |
| 6 | Pilot frequency deviation |  | Hz | ±1 |  |  |
| 7 | S signal 38kHz Residual component |  | dB | ＜-40 |  |  |
| 8 | Deviation (100% modulation) |  | kHz | ±75 |  |  |
| 9 | pre-emphasis |  | µs | 50 |  |  |
| 10 | Distortion (100% modulation) | L | ％ | ＜0.2 |  |  |
|  |  | R |  |  |  |  |
| 11 | Frequency response (without emphasis, de-emphasis) | L | dB | ±0.2 |  |  |
|  |  | R |  |  |  |  |
| 12 | Frequency response (with emphasis, de-emphasis) | L | dB | ±0.2 |  |  |
|  |  | R |  |  |  |  |
| 13 | SNR (100% modulation) | L | dB | ≥75 |  |  |
|  |  | R |  |  |  |  |
| 14 | L/R Separation | L R | dB | ＞70 |  |  |
|  |  | R L |  |  |  |  |
| 15 | L/R level diffrence |  | dB | ＜0.1 |  |  |

Specifications and Environment Conditions

| Item | Item | Technical Requirements |  | Remark |
| --- | --- | --- | --- | --- |
| Output and Input Specifications | RDS input | BNC interface(57KHz) |  |  |
|  | Input interface | XLR, AES/EBU(Optional) |  |  |
|  | Output interface | L29-50K |  |  |
| Environment condition | Working temperature | -20～+50℃ |  |  |
|  | Storage temperature | -30～+75℃ |  |  |
|  | Relatively humidity | ＜95% (25℃ no condensation) |  |  |
|  | Cooling mode | inside forced air cooling |  |  |
|  | atm press | 86～106kPa |  |  |
|  | power supply | AC, 100～264V/50Hz |  |  |
|  | machine room | Less dust, shock-free |  |  |
|  | Demission (L×W×H) | 10~30W | 482×420×44mm |  |
|  |  | 100W | 482×420×88mm |  |
|  |  | 300~1kW | 482×560×176mm |  |
|  |  | 3kW | 600×800×1080mm |  |
|  |  | 5kW | 600×1050×1695mm |  |
|  |  | 10kW | 650×1050×1695mm |  |

System Principle

Order Information

| DFT-7211: 10W | DFT-7212: 100W | DFT-7213: 1kW | DFT-7214: 10kW |
| --- | --- | --- | --- |
| DFT-7231: 30W | DFT-7232: 300W | DFT-7233: 3kW |  |
| DFT-7251: 50W | DFT-7252: 500W | DFT-7253: 5kW |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227