FUQ-72XX Series FM Stereo Broadcast Transmitter 1x RF Output FPGA Digital Modulation LDMOS Amplifier ≤0.2% Distortion GB/T4311-2000 Standard High Fidelity Transmission for FM Radio and Emergency Alert Systems

Product overview

The FUQ-72XX Series is a professional FM stereo broadcast transmitter with 1x RF output and LDMOS amplifier technology. It processes audio signals via FPGA-based digital modulation (DAC) and delivers 10W-10kW adjustable power with ≤0.2% distortion. The rack-mountable unit features air-cooled operation and complies with GB/T4311-2000/GY/T169-1 standards. Ideal for FM radio stations, emergency broadcast systems, and community radio networks requiring high-fidelity transmission.

Key feature

Universal PSU: Supports AC100–264V input and -20°C to +50°C operation for harsh environments.

Technical Specifications

| S/No. | Item |  | Unit | Technical Index |  | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Output Power |  | W | 500 |  |  |
| 2 | Frequency |  | MHz | 1kW≤ | 65.5~87; 87~108 |  |
|  |  |  |  | 1kW＞ | 87～108 |  |
| 3 | Frequency deviation |  | Hz | ≤500 |  |  |
| 4 | Sparious Radiation |  | dB | ≤-70 |  |  |
| 5 | parasitic amplitude modulation noise |  | dB | -60 |  |  |
| 6 | Pilot frequency deviation |  | Hz | ±1 |  |  |
| 7 | S signal 38kHz Residual component |  | dB | ＜-40 |  |  |
| 8 | Deviation (100% modulation) |  | kHz | ±75 |  |  |
| 9 | pre-emphasis |  | µs | 50 |  |  |
| 10 | Distortion (100% modulation) | L | ％ | ＜0.2 |  |  |
|  |  | R |  |  |  |  |
| 11 | Frequency response (without emphasis, de-emphasis) | L | dB | ±0.2 |  |  |
|  |  | R |  |  |  |  |
| 12 | Frequency response (with emphasis, de-emphasis) | L | dB | ±0.2 |  |  |
|  |  | R |  |  |  |  |
| 13 | SNR (100% modulation) | L | dB | ≥75 |  |  |
|  |  | R |  |  |  |  |
| 14 | L/R Separation | L R | dB | ＞70 |  |  |
|  |  | R L |  |  |  |  |
| 15 | L/R level diffrence |  | dB | ＜0.1 |  |  |

Specifications and Environment Conditions

| Item | Item | Technical Requirements |  | Remark |
| --- | --- | --- | --- | --- |
| Output and Input Specifications | RDS input | BNC interface(57KHz) |  |  |
|  | Input interface | XLR, AES/EBU(Optional) |  |  |
|  | Output interface | L29-50K |  |  |
| Environment condition | Working temperature | -20～+50℃ |  |  |
|  | Storage temperature | -30～+75℃ |  |  |
|  | Relatively humidity | ＜95% (25℃ no condensation) |  |  |
|  | Cooling mode | inside forced air cooling |  |  |
|  | atm press | 86～106kPa |  |  |
|  | power supply | AC, 100～264V/50Hz |  |  |
|  | machine room | Less dust, shock-free |  |  |
|  | Demission (L×W×H) | 10~30W | 482×420×44mm |  |
|  |  | 100W | 482×420×88mm |  |
|  |  | 300~1kW | 482×560×176mm |  |
|  |  | 3kW | 600×800×1080mm |  |
|  |  | 5kW | 600×1050×1695mm |  |
|  |  | 10kW | 650×1050×1695mm |  |

System Principle

Order Information

| DFT-7211: 10W | DFT-7212: 100W | DFT-7213: 1kW | DFT-7214: 10kW |
| --- | --- | --- | --- |
| DFT-7231: 30W | DFT-7232: 300W | DFT-7233: 3kW |  |
| DFT-7251: 50W | DFT-7252: 500W | DFT-7253: 5kW |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227