FMUSER FUQ-200A 10G IP QAM Processor 64-128 DVB-C QAM Channels IP-to-QAM Conversion 3x SFP+-SFP Ports EN300 429 ITU-T J.83A-B-C Multiplexer Scrambler Modulator for Broadcast Systems Product Overview

The FMUSER FUQ-200C integrates multiplexer, scrambler, and modulator functions, delivering up to 128 DVB-C QAM channels via RF ports. It processes input signals through SFP+ ports, supporting TS over UDP/RTP and multicast protocols. Housed in a 1U chassis, it features web-based management for easy control. Ideal for broadcast systems requiring high-density signal processing.The device is equipped with 1 (or 2) 64ch QAM card and characterized with dual RF output ports on each card to broaden the bandwidth for QAM carriers. Its high density helps operators to save cost to the most extent.

Key feature

High-Density Design: Features a 1U chassis with plug-in cards for up to 128 non-adjacent carrier outputs.

Advanced Processing: Supports multiplexing, scrambling with 6 CAS Simul-cryption, PCR adjusting, PID remapping, and PSI/SI editing.

Flexible Interfaces: Includes 3 SFP+ (10G)/SFP (1G) ports for TS over UDP/RTP and multicast.

Web-Based Management: Enables easy monitoring and configuration via a network interface.

Inner Principle Chart (two cards)

Carrier Setting Illustration (per card)

Specifications

| Input | Input | Max 512 IP in per card through SFP+/SFP ports 1-3 |  |
| --- | --- | --- | --- |
|  | Transmission Rate | Max 9600Mbps for each 10G SFP+/SFP input |  |
|  | Transport Protocol | TS over UDP/RTP, unicast and multicast, IGMP V2/V3 |  |
| Mux | Max PIDs Remapping | 256 per output channel |  |
|  | Functions | PID remapping (auto/manually optional) |  |
|  |  | PCR accurate adjusting |  |
|  |  | PSI/SI table automatically generating |  |
| Scrambling | Max simulcrypt CA | 6 |  |
|  | Scramble Standard | ETR289, ETSI 101 197, ETSI 103 197 |  |
|  | Connection | Local/remote connection |  |
| Modulation | Modulation Standard | EN300 429/ITU-T J.83A/B/C |  |
|  | Constellation | J.83A | Constellation :16/32/64/128/256QAM |
|  |  |  | Bandwidth :8M |
|  |  | J.83B/C | Constellation :64/256QAM |
|  |  |  | Bandwidth :6M |
|  | QAM Channel | 64/128 non-adjacent carrier outputs, 384Mbps bandwidth for each RF port |  |
|  | Symbol Rate | 3600~7000Ksps, 1ksps stepping 5057Ksps (J.83B, 64QAM) ; 5361Ksps (J.83B, 256QAM) |  |
|  | Constellation | 16, 32, 64 , 128, 256QAM |  |
|  | FEC | RS (204, 188) |  |
| RF Output | Interface (per card) | 2 F type output ports for 64 carriers, 75Ω Carrier 1~32 out thru RF1, 33~64 thru RF2 |  |
|  | RF Range | 50~960MHz, 1kHz stepping |  |
|  | Output Level | -20dBm~+10dBm(87~117dbµV), 0.1dB stepping |  |
|  | MER | ≥ 40dB |  |
| TS output | Per Daughter QAM Card | 64 IP output over UDP/RTP/RTSP, unicast/multicast, through SFP+/SFP ports 1-3 |  |
| System | Web-based Network management |  |  |
| General | Dimension | 420mm×440mm×44.5mm (WxLxH) |  |
|  | Temperature | 0~45℃(operation), -20~80℃(storage) |  |
|  | Power Supply | AC 100V±10%, 50/60Hz; or AC 220V±10%, 50/60Hz Redundancy power supply (optional as per order) |  |
|  | Consumption | 50W(1 daughter QAM card)/75W(2 daughter QAM cards) |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227