FMUSER FUQ-200B QAM Modulator 256 IP Input 96 QAM Output 10G SFP+-GE Interfaces DVB-C EN300-429 ITU-T J.83A-B-C Multiplex Scrambler Modulation for Broadcast Systems

Product Overview

The FMUSER FUQ-200B integrates multiplex, scrambler, and modulator functions, outputting 48/96 non-adjacent DVB-C QAM carriers via 2/4 RF ports and SFP+/GE ports. It processes signals through multiplexing, scrambling, and modulating in a 1U chassis with modular design and web-based management. Ideal for broadcast operators to save costs in high-density signal distribution.The device is equipped with 1 (or 2) 48ch QAM card and characterized with dual RF output ports on each card to broaden the bandwidth for QAM carriers. Its high density helps operators to save cost to the most extent.

Key feature

Integrates multiplex, scrambler, and modulator with 6 CAS Simul-cryption., Features 2/4 RF outputs and multiple interfaces including SFP+/SFP/GE ports., Outputs 48/96 non-adjacent DVB-C QAM carriers., Supports web-based network management for easy control.

Inner Principle Chart (per card)

Carrier Setting Illustration (per card)

Specifications

| Input | Input | Max 256*4 IP in through GE ports 1-4 Max 256*3 IP in through SFP+/SFP ports 1-3 |  |
| --- | --- | --- | --- |
|  | Transmission Rate | Max 960Mbps for each GE input, and Max 9600Mbps for each 10G SFP+/SFP input |  |
|  | Transport Protocol | TS over UDP/RTP, unicast and multicast, IGMP V2/V3 |  |
| Mux | Max PIDs Remapping | 256 per output channel |  |
|  | Functions | PID remapping (auto/manually optional) |  |
|  |  | PCR accurate adjusting |  |
|  |  | PSI/SI table automatically generating |  |
| Scrambling | Max simulcrypt CA | 6 |  |
|  | Scramble Standard | ETR289, ETSI 101 197, ETSI 103 197 |  |
|  | Connection | Local/remote connection |  |
| Modulation | Modulation Standard | EN300 429/ITU-T J.83A/B/C |  |
|  | Constellation | J.83A | Constellation :16/32/64/128/256QAM |
|  |  |  | Bandwidth :8M |
|  |  | J.83B/C | Constellation :64/256QAM |
|  |  |  | Bandwidth :6M |
|  | QAM Channel | 48/96 non-adjacent carrier outputs, 384Mbps bandwidth for each RF port |  |
|  | Symbol Rate | 3600~7000Ksps, 1ksps stepping 5057Ksps (J.83B, 64QAM) ; 5361Ksps (J.83B, 256QAM) |  |
|  | Constellation | 16, 32, 64 , 128, 256QAM |  |
|  | FEC | RS (204, 188) |  |
| RF Output | Interface (per card) | 2 F type output ports for 48 carriers, 75Ω Carrier 1~24 out thru RF1, 25~48 thru RF2 |  |
|  | RF Range | 50~960MHz, 1kHz stepping |  |
|  | Output Level | -20dBm~+10dBm(87~117dbµV), 0.1dB stepping |  |
|  | MER | ≥ 40dB |  |
| TS output | Per Daughter QAM Card | 48 IP output over UDP/RTP/RTSP, 2*100/1000M self-adaption, Ethernet Ports, unicast/multicast IP 1~24 out thru Data1, IP 25~48 out thru Data2. |  |
| System | Network management software (NMS) supporting |  |  |
| General | Demission | 420mm×440mm×44.5mm (WxLxH) |  |
|  | Weight | 5kg |  |
|  | Temperature | 0~45℃(operation), -20~80℃(storage) |  |
|  | Power Supply | AC 100V±10%, 50/60Hz ; or AC 220V±10%, 50/60Hz |  |
|  | Consumption | 15.4W |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227