FMUSER FUQ-200E QAM Processor 112 Channels 10G-SFP+-1G-SFP GE Input DVB-C EN300-429 ITU-T J.83 A-B-C Output Multiplexer Scrambler Modulator Web Network Management Cable TV Headend

Product Overview

The FMUSER FUQ-200E is a 10G IP QAM processor integrating multiplexer, scrambler, and modulator functions. It processes signals through multiplexing, scrambling, and modulation, supporting up to 112 DVB-C QAM channels with 3 SFP+(10G)/SFP(1G) ports and 2 GE ports. Housed in a 1U chassis with modular card slots, it features web-based network management and is ideal for cable television headends and broadcast systems requiring high-density processing.This processor is equipped with 2 card slots to load 32ch or 48ch or 64ch QAM card. Its high density helps operators to save cost to the most extent.

Key feature

Enables web-based network management for remote monitoring and control.

Inner Principle Chart (Example: one 64qam card + one 48qam card)

Carrier Setting Illustration (Example: 64qam card)

Specifications

| Input | Input from Front Panel | Max 512 IP in per card through SFP+/SFP ports 1-3 and 2 GE ports |  |  |
| --- | --- | --- | --- | --- |
|  | Input from Daughter Card | 2*512 IP in from 2 GE ports (SFP port optional)--32/48 QAM |  |  |
|  | Transmission Rate | Max 9600Mbps for each 10G SFP+/SFP input |  |  |
|  | Transport Protocol | TS over UDP/RTP, unicast and multicast, IGMP V2/V3 |  |  |
| Mux | Max PIDs Remapping | 256 per output channel |  |  |
|  | Functions | PID remapping (auto/manually optional) |  |  |
|  |  | PCR accurate adjusting |  |  |
|  |  | PSI/SI table automatically generating |  |  |
| Scrambling | Max simulcrypt CA |  | 6 |  |
|  | Scramble Standard |  | ETR289, ETSI 101 197, ETSI 103 197 |  |
|  | Connection |  | Local/remote connection |  |
| Modulation | Modulation Standard |  | EN300 429/ITU-T J.83A/B/C |  |
|  | Constellation |  | J.83A | Constellation :16/32/64/128/256QAM |
|  |  |  |  | Bandwidth :8M |
|  |  |  | J.83B/C | Constellation :64/256QAM |
|  |  |  |  | Bandwidth :6M |
|  | QAM Channel |  | 32/48/64/96/112 non-adjacent carriers output, 384Mbps bandwidth for each RF port |  |
|  | Symbol Rate |  | 3600~7000Ksps, 1ksps stepping 5057Ksps (J.83B, 64QAM) ; 5361Ksps (J.83B, 256QAM) |  |
|  | Constellation |  | 16, 32, 64, 128, 256QAM |  |
|  | FEC |  | RS (204, 188) |  |
| RF Output | Interface (per card) | 1 or 2 F type output ports for 32/48/64 carriers card, 75Ω. 32ch qam card : all Carrier out thru one RF port 32ch/48ch qam card : Carrier 1~16/1~24 out thru RF1, 17~32/25~48 thru RF2 64ch qam card : Carrier 1~32 out thru RF1, 33~64 thru RF2 |  |  |
|  | RF Range | 50~960MHz, 1kHz stepping |  |  |
|  | Output Level | -20dBm~+10dBm(87~117dbµV), 0.1dB stepping |  |  |
|  | MER | ≥ 40dB |  |  |
| TS output | Per Daughter QAM Card | 32/48/64 IP output over UDP/RTP/RTSP, unicast/multicast, through SFP+/SFP ports 1-3 or GE ports RJ45 |  |  |
| System | Web-based Network management |  |  |  |
| General | Dimension | 420mm×440mm×44.5mm (WxLxH) |  |  |
|  | Temperature | 0~45℃(operation), -20~80℃(storage) |  |  |
|  | Power Supply | AC 100V±10%, 50/60Hz ; or AC 220V±10%, 50/60Hz |  |  |
|  | Consumption | 50W(1 daughter QAM card)/75W(2 daughter QAM cards) |  |  |

Order Guide

| Options Available | Combinations |
| --- | --- |
| 32ch QAM out | 32qam card * 1 |
| 48ch QAM out | 48qam card * 1 |
| 64ch QAM out | 64qam card * 1 |
| 96ch QAM out | 64qam card * 1 + 32qam card *1 |
| 112ch QAM out | 64qam card * 1 + 48qam card *1 |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227