FMUSER FUQ-230 Modular Headend Processor 128 IP Inputs 3 Module Slots DVB-C-DVB-T-ATSC-ISDB-T ASI-S2X-USB Multi-Standard Demodulation Remultiplexing for Terrestrial Cable Satellite Networks Product overview

The FMUSER FUQ-230 is a 1U modular headend processor with 128 IP inputs, 1 ASI, 1 DVB-S/S2/S2X, and 1 USB interface. It processes signals via demodulation, descrambling, and remultiplexing using 3 swappable modules. Web-based NMS control enables remote management for cable headends, terrestrial transmitters, and IPTV distribution.

Key feature

MPTS over UDP/RTP/RTSP with 256 PIDs per channel

Principle Chart

Chassis Specifications

| Input | 3 Modules inputs(see the module specification) 1 ASI in for re-multiplexing 128 IP(SPTS/MPTS) inputs over UDP and RTP, RJ45, 1000M/100M DATA 1 , Unicast/Multicast 1 DVB-S/S2/S2X tuner in for re-multiplexing 1 USB Player input for re-mux (“xxx.ts” video) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Multiplexing | Maximum PID Remapping | 256 input per channel |  |  |  |
|  | Function | A/V PID Filtering |  |  |  |
|  |  | PID remapping ( automatically or manually) |  |  |  |
|  |  | Accurate PCR adjusting |  |  |  |
|  |  | Generate PSI/ SI table automatically |  |  |  |
| Modulation | DVB-C | QAM Channel |  | 16 non-adjacent carriers output (maximum bandwidth 192MHz) |  |
|  |  | Standard |  | EN300 429/ITU-T J.83A/B |  |
|  |  | MER |  | ≥40db |  |
|  |  | RF frequency |  | 50~960MHz, 1KHz step |  |
|  |  | RF output level |  | -20~+3dbm, 0.1db step |  |
|  |  | Symbol Rate |  | 5.0Msps~7.0Msps, 1ksps stepping |  |
|  |  |  |  | J.83A | J.83B |
|  |  | Constellation |  | 16/32/64/128/256QAM | 64/256 QAM |
|  |  | Bandwidth |  | 8M | 6M |
|  | DVB-T | Standard |  | EN300744 |  |
|  |  | FFT mode |  | 2K,4K, 8K |  |
|  |  | Bandwidth |  | 6M, 7M, 8M |  |
|  |  | Constellation |  | QPSK, 16QAM, 64QAM |  |
|  |  | Guard Interval |  | 1/4, 1/8, 1/16, 1/32 |  |
|  |  | FEC |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |
|  |  | MER |  | ≥42 dB |  |
|  |  | RF frequency |  | 50~960MHz, 1KHz step |  |
|  |  | RF out |  | 8 non-adjacent carriers output (maximum bandwidth 192MHz) |  |
|  |  | RF output level |  | -20~ +3dBm, 0.1db step |  |
|  | ATSC | Standard |  | ATSC A/53 |  |
|  |  | Bandwidth |  | 6M |  |
|  |  | Constellation |  | 8VSB |  |
|  |  | FEC |  | RS(208 188)+Trellis |  |
|  |  | MER |  | ≥40dB |  |
|  |  | ACL |  | -55 dBc |  |
|  |  | RF frequency |  | 50~960MHz, 1KHz step |  |
|  |  | RF out |  | 8 non-adjacent carriers output (maximum bandwidth 192MHz) |  |
|  |  | RF output level |  | -20~+3dbm (for all carriers), 0.5db stepping |  |
|  | ISDB-T | Standard |  | ARIB STD-B31 |  |
|  |  | Bandwidth |  | 6M |  |
|  |  | Constellation |  | QPSK, 16QAM, 64QAM |  |
|  |  | Guard Interval |  | 1/32, 1/16, 1/8, 1/4 |  |
|  |  | Transmission Mode |  | 2K, 4K, 8K |  |
|  |  | Code rate |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |
|  |  | MER |  | ≥40dB |  |
|  |  | RF frequency |  | 50~960MHz, 1KHz step |  |
|  |  | RF out |  | 6 non-adjacent carriers output (maximum bandwidth 192MHz) |  |
|  |  | RF output level |  | -20dBm~+3dBm, 0.1dB stepping |  |
| Stream output | 1 ASI output as mirror of one of RF output carriers 16 MPTS output over UDP and RTP/RTSP as mirror of 16 DVB-C carriers, 8 MPTS output over UDP and RTP/RTSP as mirror of 8 DVB-T/ATSC carriers(Optional) 6 MPTS output over UDP and RTP/RTSP as mirror of 6 ISDB-T carriers(Optional) 1*100M/1000M Base-T Ethernet interface, DATA 2 |  |  |  |  |
| System function | Network management (WEB) |  |  |  |  |
|  | Chinese and English language |  |  |  |  |
|  | Ethernet software upgrade |  |  |  |  |
| Miscellaneous | Dimension (W×L×H) |  | 482mm×380mm×44mm |  |  |
|  | Environment |  | 0~45℃(work)；-20~80℃（Storage） |  |  |
|  | Power requirements |  | AC 110V± 10%, 50/60Hz, AC 220 ± 10%, 50/60Hz |  |  |

Module Specifications

DX902A 2 Tuner Descrambling Module

Stream in:

2 Tuner input, F Type,

Stream out: 1 MPTS output to DHP230 chassis for re-multiplexing

DVB-CI:
2 independent common interface slots

Standard:  DVB-S/S2/S2X;

DVB-S

Input Frequency: 950-2150MHz

Symbol Rate: QPSK 1~45Msps

Signal Strength: -65~ -25dBm

FEC Demodulation: 1/2, 2/3, 3/4, 5/6, 7/8

DVB-S2

Input Frequency: 950-2150MHz

Symbol rate: QPSK/8PSK 1~45Msps; 16APSK 1~45 Msps;  32APSK1~32 Msps

FEC Demodulation: 1/2, 2/3, 3/4,5/6,7/8, 4/5,5/6,8/9, 9/10

DVB-S2X

Input Frequency: 950-2150MHz

Symbol rate: QPSK/8PSK/16APSK 0.5~45 Msps;

8APSK/32APSK: 0.5~40Msps

FEC Demodulation:

QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20

8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10, 23/36, 25/36, 13/18

8APSK: 5/9-L, 26/45-L

16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10,1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36, 2/3-L, 25/36, 13/18, 7/9, 77/90

32APSK: 3/4, 4/5, 5/6, 8/9, 2/3-L, 32/45, 11/15, 7/9

Support Diseqc function

Multiplexing:

Maximum PID Remapping: 256 output pids

Function: PID remapping (automatically or manually), Accurate PCR adjusting, generate PSI/ SI table automatically

Descrambling:

CAM/CI Quantity: 2

BISS Mode: Mode 1, Mode E; 32 BISS Keys

DX942A 4 frequencies Descrambling Module

Stream in:

4 frequencies input(each RF in interface for 2 frequencies locking), F Type,

Stream out:  1 MPTS output to DHP230 chassis for re-multiplexing

DVB-CI: 2 independent common interface slots

Standard: DVB-C (J.83 A/C)/J.83B/ DVB-T/DVB-T2/ISDB-T switchable

Standard: DVB-C (J.83 A/C); J.83B

Input Frequency: 60MHz~890MHz

Symbol rate: 1000~9000Ksps

Constellation: 16/32/64/128/256 QAM; 64/256 QAM for J.83B

Standard: DVB-T/T2

Frequency In: 60MHz~890MHz

Bandwidth: 5/6/7/8M bandwidth

PLP supported for DVB-T2

Standard: ISDB-T

Input Frequency: 60-890MHz

Multiplexing:

Maximum PID Remapping: 256 output pids

Function: PID remapping (automatically or manually), generate PSI/ SI table automatically

Descrambling:

CAM/CI Quantity: 2

BISS Mode: Mode 1, Mode E; 32 BISS Keys

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227