FMUSER FUQ-2500B Analog RF Modulator 32-64 Channel IP-HDMI-AV Input PAL-NTSC-SECAM Output HEVC-H.265-H.264-MPEG-2 A2-NICAM728 OSD Insertion Broadcast Systems

Product overview

- The FMUSER FUQ-2500B converts IP/HDMI/AV inputs to analog RF outputs for up to 64 channels. It processes signals with full digital processing for high SFDR performance, supporting A2 and NICAM728 audio standards and PAL/NTSC/SECAM modulation including OSD insertion. Applications include television broadcasting for PAL, NTSC, or SECAM systems.

Key feature

- - Supports A2 and NICAM728 audio standards with dual-channel mono and stereo modes.

- - Ensures high SFDR performance through full digital processing for superior RF output.

- - Provides OSD customization with logo/caption insertion, adjustable scrolling, and multilingual support per channel.

- - Features hybrid input flexibility for 16-64 IP channels or HDMI/AV + IP combinations.

- - Delivers multi-standard output with 32/64 carriers within 400MHz for PAL, NTSC, or SECAM broadcasts.

Support A2 and NICAM728 mono and stereo audio, up to dual channels

Audio options available:

- Mono L, Mono R, Mono L+R

- A2 Dual Mono (L, R), A2 Stereo

- NICAM Dual Mono (L, R), NICAM Stereo

Knowledge about A2 and NICAM 728 audio:

NICAM 728: It is transmission of two-channel digital sound with terrestrial television systems B, G, H, l, K1 and L

A2 Stereo: Known as "two-channel sound", it is an analog television sound transmission system used in Germany, Austria, Australia, Switzerland, NetherlaFU and some other countries that use or used PAL-B or PAL-G. .It relies on two separate FM carriers.

A2 Audio Spectral Diagram                                        NICAM Audio Spectral Diagram

Support High SFDR Performance

Full Digital Processing, Super RF out performance

Support OSD (Logo&Caption) insertion for each channel, easy operation

Support Multiple-modulation Standard

32 (or 16/48/64) standard PAL or NTSC or SECAM TV programs output via 32 (or 16/48/64) non-adjacent or adjacent carriers output within 400MHz

Other Key feature

Support to convert 32 (or 16/48/64) IP channels (or 4 HDMI/AV+28 IP channels, or 8 HDMI/AV+24 IP channels) to analog RF out

Suport CC/Subtitle/Teletext

Support BISS De-scrambling

Support program info modify, multiple audio select l

2 (or 4) GE ports for IP MPTS/SPTS input, Max 840Mbps for each GE input

Support HEVC/H.265, H.264/AVC, MPEG-2 TS Decoding

32 (or 16/48/64) standard PAL or NTSC or SECAM TV programs output via 32 (or 16/48/64) non-adjacent or adjacent carriers output within 400MHz

Redundancy power supply for option

Compact body, Freight saving

Support Web-based Network management

Specifications

| Input | Interface/rate | 32(or 16/48/64) IP (or 4 HDMI/AV+28 IP, or 8 HDMI/AV+24 IP) input 2 /4 GE ports (max 64 IP input) Max 840Mbps for each GE input |
| --- | --- | --- |
|  | Stream | UDP, UDP / RTP, 1-7 packets (SPTS/MPTS) |
|  | Transport Protocol | UDP/RTP, unicast and multicast, IGMP V2/V3 |
|  | Packet Length | 188 / 204 Bytes |
| Decoding Parameters | Video | HEVC/H.265, H.264/AVC, MPEG-2 |
|  | Audio | MPEG-1/2 Layer 1/2, (HE-)AAC |
|  | Audio Channel | Mono L, Mono R, Mono L+R A2 Dual Mono (L, R), A2 Stereo NICAM Dual Mono (L, R), NICAM Stereo (Note: when applying dual audio, one TV program with dual audio will occupy 2 frequencies, which means the total output channels will reduce respectively.) |
|  | Data | CC, Teletext, Subtitles, DVB Subtitling |
|  | Resolutions | HEVC/H.265: 1080@60P, 1080@60I, 1080@50P, 1080@50I, 720@60P, 720@50P H.264/AVC: 1080@60I, 1080@50P, 1080@50I, 1080@30P, 1080@25P, 720@60P, 720@50P, 576@50I, 480@60I MPEG2: 1080@60I, 1080@50I, 720@60P, 720@50P, 576@50I, 480@60I |
|  | Aspect ratio | 4:3 |
| Modulation Parameters | Number of channels | 32 (or 16/48/64) non-adjacent or adjacent carriers output |
|  | Connectors | 75Ω, F-jack |
|  | Frequency range | 43.25 – 951.252MHz |
|  | Output Bandwidth | 400MHz |
|  | Output level | -5 ~ 2dBm (102 ~ 109dBμV) |
|  | Return loss | ≥ 14dB |
|  | Spurious frequency dist. | ≥ 60dB |
|  | Residual carrier accuracy | 1% |
|  | TV standard | PAL B/G/D/K/M/N/I, NTSC M, SECAM |
|  | Video-signal to noise ratio | ≥ 60dB |
|  | Carrier to noise ratio | ≥ 60dB (when single carrier on) ≥ 55.5dB (when all 32 carriers on) |
| Network Interface | Management | 1 x 100 Base-T Ethernet (RJ 45) |
|  | Data | 2 x 1000 Base-T Ethernet (RJ 45) |
| Others | Image resolution output | 480i/576i |
|  | OSD | Logo : JPG, BMP, PNG |
|  |  | Caption : support multiple languages, scrolling direction and speed customized |
|  | Output volume adjustment | 0 - 100 % |
| General | Demission | 483mm×330mm×44.5mm (1U) 483mm×330mm×88mm (2U) |
|  | Temperature | 0~45℃(operation), -20~80℃(storage) |
|  | Power Supply | AC100V±10%, AC 220V±10%, 50/60Hz Redundancy power supply (Optional) |

Order Guide

| Options | Configuration |
| --- | --- |
| Option 1 | 16 or 32 IP input, 16 or 32 analog channel outputs |
| Option 2 | 4 HDMI+28 IP input, 32 analog channel outputs |
| Option 3 | 8 HDMI+24 IP input, 32 analog channel outputs |
| Option 4 | 4 AV+28 IP input, 32 analog channel outputs |
| Option 5 | 8 AV+24 IP input, 32 analog channel outputs |
| Option 6 | 48 or 64 IP input, 48 or 64 analog channel outputs |

System Construction Illustration

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227