FMUSER FUQ-2500E Agile Modulator 1×CVBS Video 1×L-R Audio Input 1×RF Output PAL-NTSC-SECAM Standards Configurable RF 47-870MHz Hotel Community Entertainment Product overview

The FMUSER FUQ-2500E Agile Modulator offers RF modulation with 1 audio (L/R) and 1 video (CVBS) input, plus configurable RF output. It processes signals using video input AGC for stability. This unit features a 19-inch 1U rack-mount design and keyboard/LCD operation with online upgrade. Ideal for hotels, communities, and entertainment venues.

Key feature

- Supports 1 audio (L/R) and 1 video (CVBS) input for versatile connectivity.

- Configurable RF output (47-870MHz) with test port and ±5kHz accuracy.

- Features video input AGC and compatibility with PAL B/G, PAL D/K, NTSC, SECAM standards.

- Enables keyboard and LCD operation with online upgrade capability.

- Integrates 19-inch 1U rack-mount design for easy installation.

Principle Chart

Specifications

| Input | 1* audio (L/R) input and 1*video (CVBS) |  |  |
| --- | --- | --- | --- |
| Video | A/V ration |  | -10~-20dB |
|  | Input level |  | 1VPP |
|  | Video clamping capacity |  | ≥26dB |
|  | Color / light delay difference |  | ≤45nS |
|  | Differential gain |  | ≤2% (Modulation 87.5%) |
|  | Differential phase |  | ≤2° (Modulation 87.5%) |
|  | SNR |  | ≥50dB |
|  | Image carrier drift |  | ≤5kHz |
|  | 2T-K impulse ratio |  | ≤4% |
|  | Flatness in band |  | ≤1dB |
| Audio | Pre-emphasis of audio |  | 50μs |
|  | Input level |  | 0.7V |
|  | SNR |  | ≥60dB |
|  | Flatness in band |  | ≤1dB |
|  | Distortion degree |  | ≤0.5% |
|  | Audio modulation frequency offset |  | ± 50kHz |
|  | V/A interval |  | According to TV System standard |
| RF Output | Interface |  | 1 F typed output port, 75Ω |
|  | Frequency range |  | 47~870MHz, 1kHz step |
|  | TV system standard |  | PAL B/G, PAL D/K, NTSC, SECAM |
|  | Output return loss |  | ≥14dB |
|  | Output level |  | ≥113dBμV |
|  | Output level adjustable range |  | 0~16dB |
|  | Error of video/audio carrier interval |  | ≤5 kHz |
|  | Carrier to noise ratio (C/N) |  | ≥62dB |
|  | Output suppression |  | ≥60dB (≥130dBuV testing) |
|  | Flatness in band |  | ≤1dB |
|  | Frequency accuracy |  | ≤5kHz |
|  | Adjacent Frequency suppression |  | ≥45dB |
| System function | Keyboard and LCD operation |  |  |
|  | Multiple languages |  |  |
|  | Ethernet software upgrade |  |  |
| Miscellaneous | Dimension(W×L×H) | 482*256*44mm |  |
|  | Environment | -10~45℃(work); -20~80℃ (storage） |  |
|  | Power requirements | AC 90-264V, 50Hz |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227