FMUSER FUQ-3504 IP Multiplexer 4 ASI Bidirectional Inputs-Outputs 768 UDP-RTP IP Inputs 4 MPTS UDP-RTP Outputs PID Re-mapping PSI-SI Auto-Generation for CATV Headend Product Overview

The FUQ-3504 IP Multiplexer provides high-density MPTS multiplexing with 4 bidirectional ASI inputs and 768 IP streams via UDP/RTP, outputting to 4 MPTS streams. It processes signals through PID remapping, PCR adjustment, and PSI/SI auto-generation. Controlled via web-based NMS over a dedicated 10/100M port, this 1RU rack-mountable unit is designed for digital broadcast headends and CATV systems.

Key feature

- Supports 4 bidirectional ASI ports and 768 IP inputs via UDP/RTP.

- Performs PID remapping, PCR adjustment, and PSI/SI auto-generation.

- Includes large buffer memory to prevent overflow and bypass output capability.

- Features web-based NMS for centralized monitoring and control.

Principle Chart

Specifications

| Input / Output | 4 bi-direction ASI ports: max 4 ASI input/output, BNC 75Ω 3 bi-direction Data ports (RJ45) 768 IP inputs over UDP/RTP (256 IP inputs thru GE1, GE2 and DATA port respectively) 4 MPTS outputs over UDP/RTP thru GE1, GE2 and DATA port 100/1000Mbps self-adaption for all data ports |  |
| --- | --- | --- |
|  | Packet format | 204/188 self-adaption for input, 204/188 is settable for output |
|  | Output TS Mode | bytes/packets |
| Re-multiplex | Max PID numbers | 748 PID numbers for GE1 |
|  |  | 748 PID numbers for GE 2 |
|  |  | 748 PID numbers for DATA port and 4 ASI ports together |
|  | Functions | PID re-mapping |
|  |  | PCR accurate adjusting |
|  |  | Automatic generating PSI/SI table |
|  | PID transparent | PID transparent and mapping achievable |
| System | Web management:10/100M NMS port |  |
|  | Language: English and Chinese |  |
|  | Ethernet software upgrade |  |
| General | Dimensions | 482mm×300mm×44mm (WxLxH) |
|  | Weight | 3.5kg |
|  | Temperature | 0~45℃(operation), -20~80℃(storage) |
|  | Power supply | AC 110V±10%, 50/60Hz Or AC 220V±10%, 50/60Hz |
|  | Consumption | ≤40W |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227