FMUSER FUQ-3514B Modulator Dual DVB-ASI-SMPTE-310 Inputs RF Output 50-860MHz ATSC A-53 8-VSB Modulation Adaptive DPD IP Remote Control for ATSC Broadcasting and Set-top Box Testing Product Overview

The FMUSER FUQ-3514F modulator processes dual-channel DVB-ASI and SMPTE-310 inputs, delivering RF output from 50MHz to 860MHz. It employs ATSC 8-VSB modulation compliant with ATSC A/53 standard. Features adaptive digital pre-distortion (DPD) and remote control via IP and front panel. Ideal for ATSC broadcasting networks and set-top box testing applications.

Key feature

Supports ATSC A/53 compliance for reliable digital broadcasting.

Features dual-channel DVB-ASI/SMPTE-310 inputs with automatic switch-over.

Delivers RF output from 50MHz to 860MHz with 1Hz stepping.

Integrates adaptive digital pre-distortion (DPD) for signal optimization.

Enables remote control via IP and front panel for monitoring and upgrades.

Specifications

| Input |  | Self-adapted 2 Asi/SMPTE 310M input 2 ASI ports input, one for hot backup, BNC interface |
| --- | --- | --- |
|  |  | Dual IP input ports, one for hot backup , over UDP/RTP, Unicast/Multicast, GE ports |
|  |  | 1 channel 10MHz reference clock input |
| Modulation | Standard | ATSC A/53 |
|  | Constellation | 8VSB |
|  | Bandwidth | 6 MHz |
|  | FEC | RS(208 188)+Trellis |
| RF output | Port | N Type, 50Ω impedance |
|  | RF range | 50~860Mhz,1Hz stepping |
|  | Output Level ATT | -20dBm~6dBm,0.1dB stepping |
|  | Shoulder Level | 60dB |
|  | MER | ≥ 40dB |
| DPD | Non-linear DPD | over 10dB ACPR improvement (normally) |
|  | linear DPD | over 10dB non-flatness adjustment(normally) |
| System |  | LCD display & keyboard operation, NMS, Chinese & English operation display |
|  |  | Software upgraded through network |
| General | Demission(WxLxH) | 482mm×455mm×44.5mm |
|  | Weight | 6.0kg |
|  | Temperature | 0~45℃( operation),-20~80℃( storage) |
|  | Power Supply | AC 110V±10%,50/60Hz or AC 220V±10%, 50/60Hz |
|  | Consumption | 25W |

Principle chart

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227