FUQ-3515 ASI TS Switcher 3 ASI Inputs 2 ASI Outputs 188 TS Packet Format BNC Interface Auto-Failback Manual Switching Broadcast Headend Redundancy Solution

Product Overview

The FUQ-3515 ASI TS Switcher provides 1 main + 2 backup ASI inputs and 2 independent ASI outputs for transport stream redundancy. It performs automatic failback switching based on TS packet integrity monitoring (bitrate/sync/continuity). This 1RU rack-mount unit supports web-based control and firmware upgrades. Ideal for broadcast headend redundancy and cable TV signal backup systems.

Key feature

WEB-based management: Remote control via HTTP/HTTPS with upgrade support

Specifications

| Input | 3 ASI input: ASI 3 is main stream, ASI 2 is secondary stream (backup 1), ASI 1 is third stream (back up 2) |  |
| --- | --- | --- |
| Output | 2 ASI output: ASI 4 output bypass, ASI 5 loop out ASI 3 |  |
| Interface | BNC interface |  |
| TS Package Format | 188 |  |
| ASI in/out bitrate | 1~210Mbps |  |
| System | language | English/Chinese |
|  | Remote Control | web-server management (WEB) |
|  | Upgrade | Web upgrade |
| General | Size | 482mmx410mmx44mm |
|  | Temprature | 0~45℃ (working); -20~80 (Storage) |
|  | Power Supply | 110VAC±10%, 50/60Hz 220VAC±10%, 50/60Hz |
|  | Consumption | 15W |

ADD: No.10 and NO.12 Wuxing Fourth Road, Wuhou District, Chengdu, Sichuan, P.R.China 蜀ICP备17000710号-1

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227