FMUSER FUQ-3516R ISDB-T Modulator Dual ASI-Dual IP Inputs ARIB STD-B31 6MHz Bandwidth Network Control SFN-MFN Broadcasting STB Testing Product overview

The FMUSER FUQ-3516R ISDB-T Modulator delivers terrestrial digital television broadcasting, featuring dual ASI and dual IP inputs. It modulates signals compliant with ARIB STD-B31 standards, supporting single frequency network (SFN) and multi-frequency network (MFN). With network-based control via WEB interface, LCD, and keyboard, it enables easy management. Ideal for broadcasting network setup and set-top box testing in South American markets like Brazil and Argentina.

Key feature

Enables network control via WEB, LCD, and keyboard for easy upgrades.

Specifications

| Input | 2 ASI ports input, one for hot backup, BNC interface |  |  |
| --- | --- | --- | --- |
|  | Dual IP input ports over UDP/RTP, Unicast/Multicast, GE ports |  |  |
|  | 1 channel 10MHz reference clock input |  |  |
|  | 1 RF input for DPD -15.2dbm~+5dbm |  |  |
| Modulation | Standard |  | ARIB STD-B31 |
|  | Mode |  | mode 1(2k), mode 2(4k), mode 3(8k) |
|  | Constellation |  | DQPSK, QPSK, 16QAM, 64QAM |
|  | External coding |  | RS (204，188) |
|  | Internal coding |  | Convolution (1/2, 2/3, 3/4, 5/6, 7/8) |
|  | Guard interval |  | 1/4, 1/8, 1/16, 1/32 |
|  | Hierarchy mode |  | A、A+B、A+B+C |
|  | Time domain interlacing |  | mode 1: 0, 4, 8, 16 mode 2: 0, 2, 4, 8 mode 3: 0, 1, 2, 4 |
|  | Bandwidth |  | 6MHz |
| RF Out | Connector |  | N Type, 50ΩImpedance |
|  | RF range |  | 50~999Mhz，1Hz step |
|  | Output level ATT |  | -20dbm~+5dbm，0.1db step |
|  | MER |  | ≥ 42db |
| Non-linear DPD | over 10db ACPR improvement (normally) |  |  |
| linear DPD | over 10db non-flatness adjustment(normally) |  |  |
| System | LCD display & keyboard operation, WEB, Chinese & English operation display |  |  |
|  | Supporting software upgrading through WEB |  |  |
| General | Demission (W*L*H) | 440mm×414mm×44.5mm |  |
|  | Weight | 4.9kg |  |
|  | Temperature | 0~45℃(operation),-20~80℃(storage) |  |
|  | Power supply | AC 100V~240V,50/60Hz |  |
|  | Consumption | 25W |  |

Principle chart

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227