FMUSER FUQ-3517X Modulator Dual 2 ASI 2 IP Inputs DVB-T-T RF Output Non-linear DPD Web Control Broadcast and LabsModulator

Product overview

The FMUSER FUQ-3517X modulator delivers DVB-T/(T) modulation with dual ASI and IP inputs and RF output. It processes signals using non-linear DPD to enhance spectrum efficiency. Featuring web-based control and a high-stability oscillator, it supports broadcast applications for fixed/mobile devices and DVB-T/(T) STBs.

Principle chart

Key feature

Supports dual ASI and IP inputs over UDP protocol.

Integrates non-linear DPD enhancing MER (>43dB) and shoulder attenuation (>56dB).

Provides web network management for remote upgrades, PID remapping, and MFN net mode control.

Includes constant temperature crystal oscillator with 0.1ppm stability.

Supports DVB-T/(T) RF output in 150–999MHz range with configurable guard intervals and constellations.

Specifications

| Input | TS input over 2 ASI and 2 IP port (UDP) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 10MHz reference clock input and loop out, BNC interface |  |  |  |
|  | 1PPS input and loop out, BNC interface |  |  |  |
| Multiplexing(option) | Maximum PID Remapping | 128 input per channel |  |  |
|  | Function | PID remapping (automatically or manually) |  |  |
|  |  | Accurate PCR adjusting |  |  |
|  |  | Generate PSI/SI table automatically |  |  |
| Modulation | DVB-(T) | Standard |  | EN302 755 |
|  |  | Mode |  | Mode A: single-PLP; |
|  |  | PLP Constellation |  | QPSK, 16QAM, 64QAM, 256QAM (Normal or Rotated) |
|  |  | L1 Post Constellations |  | BPSK, QPSK, 16QAM, 64QAM |
|  |  | FEC Length |  | Short(16K), Normal (64K) |
|  |  | FEC Rate |  | 1/2, 3/5, 2/3, 3/4, 4/5, 5/6 |
|  |  | Pilot Pattern |  | PP1 - PP8 |
|  |  | Guard Interval |  | 1/128, 1/32, 1/16, 19/256, 1/8, 19/128, 1/4 |
|  |  | FFT Mode |  | 1k, 2k, 4k 8k, 16k, 32k (normal or extended) |
|  |  | Bandwidth |  | 5MHz, 6MHz, 7MHz, 8MHz |
|  |  | Net Mode |  | MFN |
|  | DVB-T | Standard |  | EN300 744 |
|  |  | FFT |  | 2K, 8K |
|  |  | Bandwidth |  | 5MHz, 6MHz, 7MHz, 8MHz |
|  |  | Constellation |  | QPSK, 16QAM, 64QAM |
|  |  | Guard interval |  | 1/4, 1/8, 1/16, 1/32 |
|  |  | FEC |  | 1/2, 2/3, 3/4, 5/6, 7/8 |
|  |  | Net Mode |  | MFN |
| RF Out | Connector | N Type, 50ΩImpedance |  |  |
|  | RF range | 150~999Mhz, 1hz stepping |  |  |
|  | Output level ATT | -15~+5 dBm, 0.1db stepping |  |  |
|  | MER | > 43db |  |  |
|  | Shoulder Level | >56dB |  |  |
| System | LCD display, keyboard and web Network management |  |  |  |
|  | Supporting software upgrading through network |  |  |  |
| General | Demission (W*L*H) |  | 482mm×410mm×44mm |  |
|  | Temperature |  | 0~45℃(operation),-20~80℃(storage) |  |
|  | Power supply |  | AC 220V±10%,50/60Hz |  |

Shoulder Level with DPD enabled:

MER:

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227