FMUSER FUQ-3532AB Encoder Modulator 32 CVBS-256 IP Inputs 8 MPTS IP-8 RF Outputs MPEG-4 AVC-H.264 DVB-C-T Encoding Modulation for DTV Broadcasting

Product Overview

The FMUSER FUQ-3532AB integrates encoding, multiplexing, and modulating functions, supporting 32 CVBS inputs, 1 DVB-C tuner, 1 ASI, and 256 IP inputs, with 8 MPTS outputs via IP and 8 RF outputs. It processes signals by encoding with MPEG4 AVC/H.264, multiplexing, and modulating for transmission. Housed in a standard 1U case, it offers web-based management for real-time monitoring and updates. Ideal for DTV broadcasting systems, enabling cost-effective multi-channel distribution in cable and IPTV networks.DX3532SA also supports 1 DVB-C tuner input, 1 ASI and 256 IP input and 8 non-adjacent carriers output through RF output interface and data port output. The device is also characterized with high integrated level, high performance and low cost. This is very adaptable to newly generation DTV broadcasting system.

Key feature

Integrated Encoding and Modulating: Combines encoding and modulating in a compact 1U case for efficient processing.

Versatile Input Support: Accepts 32 CVBS, 1 DVB-C tuner, 1 ASI, and 256 IP inputs over UDP/RTP.

Advanced Output Capabilities: Generates 8 MPTS outputs via IP and 8 RF outputs for flexible broadcasting.

Stream Management Tools: Enables PID remapping, PCR adjustment, PSI/SI editing, and logo insertion.

Web-Based Control: Features real-time monitoring and easy updates via web interface.

Principle Chart

DVB-C RF Out

DVB-T RF Out

Technical specification

| Input | 32 CVBS inputs , DB15-To-RCA interface |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1ASI input port |  |  |  |  |
|  | 256 IP (100/1000M)input over UDP and RTP protocol |  |  |  |  |
|  | 1 DVB-C Tuner input |  |  |  |  |
| Video | Resolution | 720×576i (PAL-B, D, G, H, I), 720×480i (NTSC-M, J) |  |  |  |
|  | Encoding | MPEG4 AVC/H.264（HP/MP/BP） |  |  |  |
|  | Bit-rate | 1Mbps~5Mbps each channel |  |  |  |
|  | Rate Control | CBR/VBR |  |  |  |
|  | GOP Structure | IPP(P frame is adjustable) |  |  |  |
| Audio | Encoding | MPEG-1 Layer 2 |  |  |  |
|  | Sampling rate | 48KHz |  |  |  |
|  | Resolution | 24-bit |  |  |  |
|  | Audio Gain | 0-255 adjustable |  |  |  |
|  | Bit-rate | 64/128/192/224/256/320/384Kbps each channel |  |  |  |
| Modulation | DVB-C | QAM Channel: 8 non-adjacent carriers output (maximum bandwidth 192MHz) Standard: EN300 429/ITU-T J.83A/B MER: ≥40dB RF frequency: 50~960MHz, 1KHz step RF output level: -20~+3dBm (87~110 dBµV), 0.1dB step Symbol Rate: 5.0Msps~7.0Msps, 1ksps step |  |  |  |
|  |  |  |  | J.83A | J.83B |
|  |  | Constellation |  | 16/32/64/128/256QAM | 64/256 QAM |
|  |  | Bandwidth |  | 8M | 6M |
|  | DVB-T | Standard |  | EN300744 |  |
|  |  | FFT mode |  | 2K, 4K, 8K |  |
|  |  | Bandwidth |  | 6M, 7M, 8M |  |
|  |  | Constellation |  | QPSK, 16QAM, 64QAM |  |
|  |  | Guard Interval |  | 1/4, 1/8, 1/16, 1/32 |  |
|  |  | FEC |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |
|  |  | MER |  | ≥40 dB |  |
|  |  | RF frequency |  | 50~960MHz, 1KHz step |  |
|  |  | RF out |  | 8*COFDM DVB-T non-adjacent carriers output (maximum bandwidth 192MHz) |  |
|  |  | RF output level |  | -20~+3 dBm (87~110 dBµV), 0.1dB step |  |
| Scrambling Parameters | Max simulscrypt CA | 4 |  |  |  |
|  | Scramble Standard | ETR289, ETSI 101 197, ETSI 103 197 |  |  |  |
|  | Connection | Local/remote connection |  |  |  |
| Stream output | IP (8*MPTS) output over UDP and RTP, 100/1000 Base-T Ethernet interface (Multicast/unicast) |  |  |  |  |
| System function | Network management (WEB) |  |  |  |  |
|  | Ethernet software upgrade |  |  |  |  |
| Miscellaneous | Dimension(W×L×H) |  | 482mm×328mm×44mm |  |  |
|  | Environment |  | 0~45℃(work)；-20~80℃（Storage） |  |  |
|  | Power requirements |  | AC 110V± 10%, 50/60Hz, AC 220 ± 10%, 50/60Hz |  |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227