FMUSER FUQ-3902B DVB-S2X Receiver 2 Tuner Inputs 32 IP Inputs UDP-RTP IP Output CAM-CI BISS Descrambling Remote Web NMS for Broadcast Engineers

Product Overview

The FMUSER FUQ-3902B is a DVB-S2X receiver that integrates demodulation, descrambling, and remuxing, featuring 2 RF tuner inputs and 32 IP inputs supporting UDP/RTP, with IP output. It converts satellite signals into IP streams through efficient processing. This compact unit enables remote control via web-based NMS management. Ideal for broadcast facilities, satellite reception, and IP streaming applications.

Key feature

- Supports dual DVB-S/S2/S2X tuner inputs for versatile reception.

- Features 32 IP inputs with UDP/RTP support for remuxing.

- Integrates decryption via CAM/CI slots and BISS.

- Enables remote monitoring and control through web-based NMS.

- Allows high PID mapping with up to 256 mappings per input channel.

Principle Chart

Technical specification

| Input | 2*RF (DVBS/S2/S2X), F type 32*IP input for re-mux (UDP/RTP) through data(1000M/100M) ports |  |  |  |
| --- | --- | --- | --- | --- |
| Tuner | DVB-S | Input Frequency |  | 950-2150MHz |
|  |  | Symbol Rate |  | QPSK 0.5Msps~45Msps |
|  |  | Signal Strength |  | -65~ -25dBm |
|  |  | FEC Demodulation |  | 1/2, 2/3, 3/4, 5/6, 7/8 |
|  | DVB-S2 | Input Frequency |  | 950-2150MHz |
|  |  | Symbol Rate |  | QPSK/8PSK/16APSK 0.5Msps~45Msps 32APSK0.5Msps~40 Msps |
|  |  | FEC Demodulation |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9 |
|  | DVB-S2X | Input Frequency |  | 950-2150MHz |
|  |  | Symbol Rate |  | QPSK/8PSK/16APSK: 0.5~45 Msps; 8APSK/32APSK: 0.5~40 Msps |
|  |  | FEC Demodulation |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10, 23/36, 25/36, 13/18 8APSK: 5/9-L, 26/45-L 16APSK:2/3,3/4,4/5, 5/6, 8/9, 9/10,1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36, 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 2/3-L, 32/45, 11/15, 7/9 |
| Multiplexing | Maximum PID Remapping | 256 PIDs per input channel |  |  |
|  | Function | PID remapping (automatically or manually), Accurate PCR adjusting, generate PSI/ SI table automatically |  |  |
| Descrambling | CAM/CI Quantity | 2 |  |  |
|  | BISS Mode | Mode 1, Mode E; up to 120Mbps |  |  |
| Output | 1*MPTS over UDP and RTP, 1000M/100M Base-T Ethernet interfaces (unicast / multicast) |  |  |  |
| System function | Network management (WEB) |  |  |  |
|  | English language |  |  |  |
|  | Ethernet software upgrade |  |  |  |
| Miscellaneous | Dimension(W×L×H) |  | 482mm×328mm×44mm |  |
|  | Environment |  | 0~45℃(work)；-20~80℃（Storage） |  |
|  | Power requirements |  | AC 110V± 10%, 50/60Hz, AC 220 ± 10%, 50/60Hz |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227