FMUSER FUQ-3902B DVB-S-S2-S2X Demodulator Descrambler IP Multiplexer 2 Tuner Inputs 32 IP Stream Outputs UDP-RTP Web-NMS Headend IPTV

Product Overview

The FMUSER FUQ-3902B is a professional DVB-S/S2/S2X demodulator, descrambler, and IP multiplexer with 2 tuner inputs (950-2150MHz) and 32 IP stream outputs (UDP/RTP). It demodulates satellite signals, decrypts via CAM/CI or BISS, and remuxes up to 32 channels into a single MPTS output. This 1RU rack-mount unit supports DiSEqC control and web-based NMS, ideal for broadcast headends and IPTV distribution.

Key feature

Web-based NMS for remote configuration and monitoringPrinciple Chart

Technical specification

| Input | 2*RF (DVBS/S2/S2X), F type 32*IP input for re-mux (UDP/RTP) through data(1000M/100M) ports |  |  |  |
| --- | --- | --- | --- | --- |
| Tuner | DVB-S | Input Frequency |  | 950-2150MHz |
|  |  | Symbol Rate |  | QPSK 0.5Msps~45Msps |
|  |  | Signal Strength |  | -65~ -25dBm |
|  |  | FEC Demodulation |  | 1/2, 2/3, 3/4, 5/6, 7/8 |
|  | DVB-S2 | Input Frequency |  | 950-2150MHz |
|  |  | Symbol Rate |  | QPSK/8PSK/16APSK 0.5Msps~45Msps 32APSK0.5Msps~40 Msps |
|  |  | FEC Demodulation |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9 |
|  | DVB-S2X | Input Frequency |  | 950-2150MHz |
|  |  | Symbol Rate |  | QPSK/8PSK/16APSK: 0.5~45 Msps; 8APSK/32APSK: 0.5~40 Msps |
|  |  | FEC Demodulation |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10, 23/36, 25/36, 13/18 8APSK: 5/9-L, 26/45-L 16APSK:2/3,3/4,4/5, 5/6, 8/9, 9/10,1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36, 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 2/3-L, 32/45, 11/15, 7/9 |
| Multiplexing | Maximum PID Remapping | 256 PIDs per input channel |  |  |
|  | Function | PID remapping (automatically or manually), Accurate PCR adjusting, generate PSI/ SI table automatically |  |  |
| Descrambling | CAM/CI Quantity | 2 |  |  |
|  | BISS Mode | Mode 1, Mode E; up to 120Mbps |  |  |
| Output | 1*MPTS over UDP and RTP, 1000M/100M Base-T Ethernet interfaces (unicast / multicast) |  |  |  |
| System function | Network management (WEB) |  |  |  |
|  | English language |  |  |  |
|  | Ethernet software upgrade |  |  |  |
| Miscellaneous | Dimension(W×L×H) |  | 482mm×328mm×44mm |  |
|  | Environment |  | 0~45℃(work)；-20~80℃（Storage） |  |
|  | Power requirements |  | AC 110V± 10%, 50/60Hz, AC 220 ± 10%, 50/60Hz |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227