FMUSER FUQ-3904C Receiver Card 4 Tuner Inputs 5 MPTS UDP-RTP Outputs DVB-S-S2-S2X PID Remapping DiSEqC Support Broadcast IPTV Integration

Product overview

The FMUSER FUQ-3904C is a professional receiver card supporting DVB-S/S2/S2X standards, featuring four tuner inputs and five MPTS outputs. It processes satellite signals through multiplexing and PID remapping. Managed via web-based NMS for remote control. Ideal for broadcast integration and satellite reception systems.

Key Features

Includes DiSEqC support for enhanced satellite dish functionality.

Inner Principle Chart (Per Card)

Specifications

| Input / Output | 4 Tuner input, F Type 5 MPTS (4 MPTS out thru bypassing, 1 MPTS multiplexing) out over UDP/RTP thru Data port, 100/1000Mself-adaption, unicast/multicast |  |  |  |
| --- | --- | --- | --- | --- |
| Mux | Max number of managing PID: 256 (for 1 MPTS out thru multiplexing) |  |  |  |
|  | Functions | PID remapping (automatically/ manually) |  |  |
|  |  | PID pass-through |  |  |
| Tuner Section | DVB-S | Input Frequency |  | 950-2150MHz |
|  |  | Symbol Rate |  | 0.5~45 Msps |
|  |  | Constellation |  | QPSK |
|  |  | FEC |  | 1/2, 2/3, 3/4, 5/6, 7/8 |
|  | DVB-S2 | Input Frequency |  | 950-2150MHz |
|  |  | Symbol Rate |  | QPSK/8PSK/16APSK: 0.5~45 Msps 32APSK:0.5~40 Msps |
|  |  | Constellation |  | QPSK/8PSK/16APSK/32APSK |
|  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9,9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9 |
|  |  | Max input bitrate per tuner |  | ≤170 Mbps |
|  | DVB-S2X | Symbol Rate |  | QPSK/8PSK/16APSK: 0.5~45 Msps; 8APSK/32APSK: 0.5~40 Msps |
|  |  | Constellation |  | QPSK/8PSK/16APSK/8APSK/32APSK |
|  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10, 23/36, 25/36, 13/18 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10,1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36, 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 2/3-L, 32/45, 11/15, 7/9 |
|  |  | Max input bitrate per tuner |  | ≤170 Mbps |
|  | Signal Strength | -65 ~ -25dBm |  |  |
| System | Web management:100M NMS port |  |  |  |
|  | Language: English and Chinese |  |  |  |
|  | Ethernet software upgrade |  |  |  |
| General | Dimensions |  | 482mm×328mm×44mm (WxLxH) |  |
|  | Weight |  | 3.5kg |  |
|  | Temperature |  | 0~45℃(operation), -20~80℃(storage) |  |
|  | Power supply |  | AC 110V±10%, 50/60Hz Or AC 220V±10%, 50/60Hz |  |
|  | Consumption |  | ≤40W |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227