FMUSER FUQ-41013K TransModulator 2-In 2-Out DVB-S-S2-S2X to DVB-T Dual Tuners CAM Slots EN300744 Advanced Signal Processing Broadcast Systems

Product overview

The FMUSER FUQ-41013K TransModulator converts satellite signals to terrestrial formats with dual DVB-S/S2/S2X tuners and 2 CAM slots. It processes signals through 2 groups multiplexing and 2 groups DVB-T modulating, including PCR adjusting and PID remapping. Managed via web-based NMS for remote control, it suits broadcasters in TV transmission and terrestrial network deployments.

Key feature

Features dual DVB-S/S2/S2X tuners with 2 CAM slots for secure reception.

Supports 2 groups multiplexing and 2 groups DVB-T modulating for flexible output.

Includes accurate PCR adjusting, PID remapping, PSI/SI editing, and A/V PID filtering.

Enables web-based NMS for remote monitoring and upgrades.

Principle Chart

Tech Specifications

| Input | 2 DVB-S/S2/S2X Tuners |  |  |  |
| --- | --- | --- | --- | --- |
| Tuner Section | DVB-S/S2/S2X Tuner | DVB-S | Input Frequency | 950~2150MHz |
|  |  |  | Symbol rate | 0.5~45Msps |
|  |  |  | Signal Strength | - 65- -25dBm |
|  |  |  | FEC | 1/2, 2/3, 3/4, 5/6, 7/8 |
|  |  |  | Constellation | QPSK |
|  |  |  | Max input bitrate | ≤129 Mbps |
|  |  | DVB-S2 | Input Frequency | 950~2150MHz |
|  |  |  | Symbol rate | QPSK/8PSK /16APSK: 0.5~45 Msps 32APSK: 0.5~40Msps; |
|  |  |  | FEC | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10 |
|  |  |  | Constellation | QPSK, 8PSK, 16APSK, 32APSK |
|  |  |  | Max input bitrate | ≤129 Mbps |
|  |  | DVB-S2X | Input Frequency | 950~2150MHz |
|  |  |  | Symbol rate | QPSK/8PSK /16APSK: 0.5~45 Msps 8APSK: 0.5~40Msps 32APSK: 0.5~40Msps |
|  |  |  | FEC | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36, 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 2/3-L, 32/45, 11/15, 7/9 |
|  |  |  | Constellation | QPSK, 8PSK, 8APSK, 16APSK, 32APSK |
|  |  |  | Max input bitrate | ≤129 Mbps |
| Descrambling | CAM/CI |  | 2 sets |  |
| Multiplexing | Maximum PID Remapping |  | 256 per input channel |  |
|  | Function |  | A/V PID filtering |  |
|  |  |  | PID Remapping |  |
|  |  |  | Accurate PCR adjusting |  |
|  |  |  | Generate PSI/SI table automatically |  |
| Modulation | DVB-T |  | Standard | EN300744 |
|  |  |  | FFT mode | 2K |
|  |  |  | Bandwidth | 6M, 7M, 8M |
|  |  |  | Constellation | QPSK, 16QAM, 64QAM |
|  |  |  | Guard Interval | 1/4, 1/8, 1/16, 1/32 |
|  |  |  | Code Rate | 1/2, 2/3, 3/4, 5/6, 7/8 |
|  |  |  | MER | ≥42 dB |
|  |  |  | RF frequency | 50~960MHz, 1KHz step |
|  |  |  | RF out | 2*DVB-T non-adjacent carriers combined output |
|  |  |  | RF output level | -28~ -3 dBm (79~104 dbµV), 0.1db step |
| System | Web-based NMS support |  |  |  |
|  | Ethernet software upgrading |  |  |  |
| General | Temperature |  | 0~45℃(Operation) ; -20~80℃(Storage) |  |
|  | Power |  | AC 100V±1050/60Hz; AC 220V±10%, 50/60HZ |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227