FMUSER FUQ-41086D HD IRD 8 Tuner Inputs DVB-S-S2-S2X DVB-S2-S2X Outputs UDP-RTP Dual Mode BISS Decryption Diseqc Broadcast IPTV

Product overview

The FMUSER FUQ-41086D HD IRD supports up to 24 tuner inputs for DVB-S/S2/S2X, DVB-C/T, and ISDB-T standards. It processes signals in Passthrough mode for 8 MPTS output or Mux mode for 2 MPTS / 512 SPTS over UDP/RTP. Managed via web-based NMS, it includes features like Biss decryption and Diseqc function. Ideal for broadcast and IPTV systems.

Key feature

Supports expandable tuner inputs for up to 24 tuners via optional cards, compatible with DVB-S/S2/S2X, DVB-C/T, and ISDB-T.

Features dual output modes: Passthrough mode for 8 MPTS or Mux mode for 2 MPTS / 512 SPTS over UDP/RTP unicast/multicast.

Enables advanced PID handling including remapping and pass-through (up to 256 PIDs per channel) exclusively in Mux mode.

Includes security with Biss decryption (64 keys per tuner) and Diseqc function, managed via NMS.

Inner Principle Chart (Per Card/8 Input):

Version 1: Passthrough Mode

Version 2: Mux Mode (MPTS and SPTS output Switchable)

Specifications (Per Card/8 Input)

| Input / Output | 8 Tuner input (DVB-S/S2/S2X or/and DVB-C/T/T2/ISDB-T), F Type 8 MPTS out thru bypassing over UDP/RTP thru Data port (Version 1) 100/1000Mself-adaption, unicast/multicast |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Mux (Version 2) | Max number of managing PID: 256 (for 2 MPTS out thru multiplexing) or 512 SPTS out |  |  |  |  |
|  | Functions | PID remapping (automatically/ manually) |  |  |  |
|  |  | PID pass-through |  |  |  |
| Tuner Section | DVB-S | Input Frequency |  | 950-2150MHz |  |
|  |  | Symbol Rate |  | 0.5~45 Msps |  |
|  |  | Constellation |  | QPSK |  |
|  |  | FEC |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |
|  | DVB-S2 | Input Frequency |  | 950-2150MHz |  |
|  |  | Symbol Rate |  | QPSK/8PSK/16APSK: 0.5~45 Msps 32APSK:0.5~40 Msps |  |
|  |  | Constellation |  | QPSK/8PSK/16APSK/32APSK |  |
|  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9,9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9 |  |
|  |  | Max input bitrate per tuner |  | ≤170 Mbps |  |
|  | DVB-S2X | Symbol Rate |  | QPSK/8PSK/16APSK: 0.5~45 Msps; 8APSK/32APSK: 0.5~40 Msps |  |
|  |  | Constellation |  | QPSK/8PSK/16APSK/8APSK/32APSK |  |
|  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10, 23/36, 25/36, 13/18 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10,1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36, 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 2/3-L, 32/45, 11/15, 7/9 |  |
|  |  | Max input bitrate per tuner |  | ≤170 Mbps |  |
|  |  | Signal Strength |  | -65 ~ -25dBm |  |
|  | DVB-C | Input Frequency |  | 60 ~890 MHz |  |
|  |  | Standard |  | J.83A, J.83C | J.83B |
|  |  | Constellation |  | 16/32/64/128/256 QAM | 64/256 QAM |
|  | DVB-T/T2 | Input Frequency |  | 60 ~890 MHz |  |
|  |  | Bandwidth |  | 6/7/8 M |  |
|  | ISDB-T | Input Frequency |  | 60 ~890 MHz |  |
| System | Web management:100M NMS port |  |  |  |  |
|  | Language: English and Chinese |  |  |  |  |
|  | Ethernet software upgrade |  |  |  |  |
| General | Dimensions |  | 482mm×328mm×44mm (WxLxH) |  |  |
|  | Weight |  | 3.5kg |  |  |
|  | Temperature |  | 0~45℃(operation), -20~80℃(storage) |  |  |
|  | Power supply |  | AC 110V±10%, 50/60Hz Or AC 220V±10%, 50/60Hz |  |  |
|  | Consumption |  | ≤25W (1card inside) ≤50W (2 cards inside) ≤75W (3 cards inside) |  |  |

Order Guide

| DX3908-8 | 8 FTA DVB-S/S2/S2X Tuner Input, Version 1 or Version 2 |
| --- | --- |
| DX3908-16 | 16 FTA DVB-S/S2/S2X Tuner Input, Version 1 or Version 2 |
| DX3908-24 | 24 FTA DVB-S/S2/S2X Tuner Input, Version 1 or Version 2 |
| DX3928-8 | 8 FTA DVB-C/T/ISDB-T Tuner Input, Version 1 or Version 2 |
| DX3928-16 | 16 FTA DVB-C/T/ISDB-T Tuner Input, Version 1 or Version 2 |
| DX3928-24 | 24 FTA DVB-C/T/ISDB-TTuner Input, Version 1 or Version 2 |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227