FMUSER FUQ-4109Y HD IRD Multi-Source Input 1RF DVB-C-DVB-S-S2-ISDB-T, 1ASI 1IP Input MPEG-2 H.264 Decoding ISDB-T Multiplexing CI Descrambling SNMP-Web Management for Broadcast Systems

Product overview

The FUQ-4109Y Series HD IRD decodes TS signals from ASI, RF tuner (supporting DVB-C/DVB-S/S2/ISDB-T), or IP sources into audio/video. It processes signals through HD decoding compliant with MPEG-2 and H.264 standards, and performs ISDB multiplexing for ASI and IP outputs. The device includes a CI descramble module and supports management via SNMP/web, LCD, and keyboard control. Ideal for broadcast systems requiring reliable signal decoding and multiplexing in headend or distribution environments.

Key Feature

Supports multi-source inputs including RF (DVB-C/DVB-S/S2/ISDB-T), ASI, and IP

Delivers high-definition decoding up to 1080i with MPEG-2 and H.264 compliance

Performs ISDB-T multiplexing for ASI and IP outputs

Features CI descrambling with DVB-CSA module via PCMCIA interface

Offers integrated management via SNMP/web, LCD, and keyboard control

Principle Chart

Specifications

| Input | 1RF Input (DVB-C/DVB-S/S2/ISDB-T optional) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 1ASI Input (support 188/204 package format) |  |  |  |
|  | 1 IP MPTS Input (100Mbps, RJ45 Port) over UDP/RTP protocol |  |  |  |
|  | GPS 10Mhz Clock Input |  |  |  |
|  | GPS 1PPS Signal Input |  |  |  |
| Demodulating Section | DVB-C | Standard |  | J.83A(DVB-C), J.83B, J.83C |
|  |  | Input Frequency |  | 30 MHz~1000 MHz |
|  |  | Constellation |  | 16/32/64/128/256 QAM |
|  | DVB-S | Input Frequency |  | 950-2150MHz |
|  |  | Symbol rate |  | 1-45Msps |
|  |  | Signal Strength |  | - 65- -25dBm |
|  |  | Constellation |  | 1/2, 2/3, 3/4, 5/6, 7/8 QPSK |
|  | DVB-S2 | Input Frequency |  | 950-2150MHz |
|  |  | Symbol rate |  | 1~45 Msps |
|  |  | Code rate |  | 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 |
|  |  | Constellation |  | QPSK, 8PSK (16 APSK, 32 APSK on request) |
|  |  | Roll off |  | 0.20, 0.25, 0.35 |
|  | ISDB-T | Input Frequency |  | 30-1000MHz |
| ASI Output | ASI 1&2 output BTS after multiplexing, ASI 3&4 for tuner pass-through |  |  |  |
| IP Output | 1 IP MPTS output (100M, RJ45 Port) over UDP/RTP protocol |  |  |  |
| Video Output | 1xCVBS(BNC): 576i@25fps, 480i@29.97fps |  |  |  |
|  | 1xYPbPr(BNC): 1080i@25, 29.97, 30fps; 720p@50, 59.94fps |  |  |  |
|  | 1xHD/SD-SDI(BNC): 1080i@25, 29.97, 30fps; 720p@50, 59.94fps; 576i@25fps; 480i@29.97fps |  |  |  |
|  | 1 x HDMI: 1080i@25, 29.97, 30fps; 720p@50, 59.94fps |  |  |  |
| Audio Output | Stereo unbalanced audio: BNC interface |  |  |  |
|  | Stereo balanced audio: XLR interface |  |  |  |
|  | Digital audio: SPDIF |  |  |  |
| Video Decoding | MPEG-2, H.264 |  |  |  |
| Audio Decoding | MPEG1 layer 2, AC3, HE-AAC (v2), LC-AAC |  |  |  |
| ISDB-T multiplexing | Support 256 PID mapping per channel (Automatically or manually) |  |  |  |
|  | PCR accurate adjusting |  |  |  |
|  | Generate PSI/SI table automatically |  |  |  |
| ISDB-T BTS parameters | Modes：mode1(2k), mode2(4k), mode3(8k) |  |  |  |
|  | Guard Interval：1/4,1/8,1/16,1/32 |  |  |  |
|  | Code Rate：1/2,2/3,3/4,5/6,7/8 |  |  |  |
|  | Constellations: DQPSK/QPSK/16QAM/64QAM |  |  |  |
|  | Layer Mode：A, A+B, A+B+C |  |  |  |
|  | Number of Segment: 1-13 |  |  |  |
|  | Bandwidth：6MHz,7MHz,8MHz |  |  |  |
| SFN parameters | Maximum time delay：0ms ～ 1000ms |  |  |  |
|  | Device delayed time shifting：-1000ms ～ +1000ms |  |  |  |
|  | Device ID:0 ～ 30 |  |  |  |
| DVB-CI Parameters | Descrambler: DVB-CSA |  |  |  |
|  | Smart Card interface: ISO7816 |  |  |  |
|  | Interface Card Separation: PCMCIA |  |  |  |
| System | Support SNMP/Web management, LCD + keyboard control |  |  |  |
|  | English and Chinese interface conversion |  |  |  |
|  | Ethernet software upgrade |  |  |  |
| General | Dimensions |  | 482mm*455mm*44.5mm |  |
|  | Approx weight |  | 3.2kg |  |
|  | Power |  | <20W(Max) |  |
|  | Temperature |  | 0~45℃(Operating), -20~80℃(Storage) |  |

Order Information:

FU-41091 DVB-C HD IRD;

FU-41092 DVB-S HD IRD;

FU-41095 DVB-S2 HD IRD;

FU-41096 ISDB-T HD IRD;

Technical Comparison:

|  |  | FU-408X /FU-400X | FU-4109X | R&S®AVG050 ISDB-T BTS Gateway |
| --- | --- | --- | --- | --- |
| Stream In | IP input | Over ,UDP, RTP (100 Base-T) | Over UDP/RTP (100 Base-T) | Over IPv4,UDP, RTP (1000 Base-T) |
| Stream Out | ASI out | ASI 1&2 mirrored output BTS after multiplexing; ASI 3&4 for tuner pass-through | ASI 1&2 mirrored output BTS after multiplexing; ASI 3&4 for tuner pass-through | ASI 1&2 mirrored output for BTS in an MPEG-2 transport stream with ancillary data |
|  | IP out | 8 SPTS output over UDP, 100M Base-T (-FU-408X) 32 SPTS output over UDP, 1000M Base-T (-FU-400X) | 1 IP MPTS output over UDP/RTP (1000 Base-T) | 2 × IP streams with individual IP addresses, UDP ports and FEC configuration in line with SMPTE 2022-1/2 |
| Decryption | BISS | Yes | N/A | Yes |
| Reference | 10MHz REF | N/A | Yes | Yes |
|  | 1PPS | N/A | Yes | N/A |
| ISDB-T Parameters | Modulation Constellation | N/A | DQPSK/QPSK/16QAM/64QAM | QPSK/16QAM/64QAM |
|  | PSI/SI |  | PAT, PMT, NIT, SDT, TOT, TDT | PAT, PMT, NIT, SDT, TOT, BIT |
|  | SFN Adapter |  | Yes | Yes |
| Decoding | Standard | Support H.264 and MPEG2 decode and output V/A via various interfaces | Support H.264 and MPEG2 decode and output V/A via various interfaces | N/A |
|  | LCD Monitor | Yes | Yes |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227