FMUSER FUQ-4214 IP Multiplexer 512 IP Inputs 2 ASI 2 IP Outputs UDP SPTS-MPTS ISO13818 EN300468 Multiplex Insert CATV Systems Product overview

The FMUSER FUQ-4214 IP Multiplexer processes transport streams with 512 IP inputs (UDP SPTS/MPTS) and delivers 2x2 ASI ports plus 2 IP outputs. It multiplexes programs while inserting EPG, CA, and data services with PSI/SI rebuilding. This 1RU rack-mount unit features web-based NMS and front-panel controls. Ideal for CATV integration and DVB digital headends requiring reliable TS processing.

Key feature

- Multiplexes 512 UDP inputs with SPTS/MPTS support and PID remapping (up to 2048 PIDs/channel)

- Delivers 2 independent ASI output groups (4 ports total) + 2 IP outputs over UDP

- Ensures broadcast compliance with ISO13818 & EN300468 including PCR adjustment

- Supports EPG/CA insertion, PSI/SI editing, and huge buffer memory for stability

- Provides web-based NMS, LCD, and keyboard control interfaces

Principle Chart

Specifications

| Input | IP (RJ45,GE) | 512 x MPTS/SPTS, Unitcast/Multicast, UDP Protocol IGMP V2/V3 |
| --- | --- | --- |
| Re-multiplex | PID re-mapping (auto or manually available) |  |
|  | PCR accurate adjusting |  |
|  | Automatic generating PSI/SI table |  |
|  | PID passthrough |  |
|  | Support multiplex maximum 2048 PIDs each output channel |  |
| Output port | ASI | 2 x independent channels of ASI output (each channel has 2 ports) (up to 210Mbps) |
|  | IP(RJ45,GE) | 2 x IP (Unitcast/Multicast), UDP Protocol (up to 230Mbps) |
| NMS port | Ethernet port | GE |
| General | Dimension | 482mmx410mmx44mm(WxLxH) |
|  | Weight | 2.0kg |
|  | Temperature | 0~45℃(operation), -20~80℃(storage) |
|  | Power supply | 110VAC±10%，50/60Hz; 220VAC±10%，50/60Hz |
|  | Consumption | <25w |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227