FMUSER FUQ-4216B Dual-Channel TS Re-Multiplexer 12 ASI Input to ASI-IP Output ISDB-T-ISDB-Tb PSI-SI SFN Support Network Managed for Broadcast Product overview

The FUQ-4216B is a TS re-multiplexer compliant with ISDB-T/ISDB-Tb standards, handling up to 12 inputs with ASI and IP outputs. It processes transport streams through re-multiplexing, supporting PSI/SI table editing and SFN support. Featuring network control via NMS/SNMP, it is ideal for professional broadcast systems in Japan and South America.

Key feature

Supports ISDB-T/ISDB-Tb standards for compliant broadcasting.

Features dual independent re-multiplexing channels for simultaneous processing.

Enables PSI/SI table editing and generation with descriptor insertion.

Provides IP and ASI outputs over UDP protocol.

Allows remote network management via NMS/SNMP for upgrades.

Specifications

| Signal inputting | 12 ASI input |
| --- | --- |
|  | 188/204Byte transmission stream packet |
|  | TS packet mode and suddenly code rate mode |
|  | GPS 10mhz reference clock inputting |
|  | GPS 1pps signal inputting |
| Re-multiplexing | 2 separate channels TS re-multiplexer |
|  | PCR accurate adjusting |
|  | PSI/SI table automatically generating |
| Modulation parameter | mode：mode1(2k) mode2(4k) mode3(8k) |
|  | Guard interval：1/4 1/8 1/16 1/32 |
|  | Coding rate：1/2 2/3 3/4 5/6 7/8 |
|  | constellation：DQPSK QPSK 16QAM 64QAM |
|  | Layer：A A+B A+B+C |
|  | Bandwidth: 6MHz 7MHz 8MHz |
| SFN parameter | Maximum time delay：0ms ～ 1000ms |
|  | Off set：-1000ms ～ +1000ms |
|  | Device ID ：01 ～ 30 |
| Output | 2 ASI output(BNC) |
|  | 2 IP output(UDP protocol) |
|  | RS encoding output supporting |
|  | GPS 1pps and GPS 10Mhz loop-out(BNC) |
| General | Demission：44mmx482mmx410mm |
|  | Temperature：0～45℃(operation) -20～80℃（storage） |
|  | Power supply：220VAC±10%，50/60Hz，10W |

Principle Chart

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227