FMUSER FUQ-4218K Tuner Input Multiplexer 16 Tuners 2 ASI Inputs DVB-S-S2-S2X-C-T ISDB-T ATSC IP Outputs 4 MPTS BISS Descrambling CATV Headend System Product overview

The FUQ-4218K Tuner Input Multiplexer processes 16 tuner inputs (configurable for DVB-S/S2/S2X, DVB-C/T, ISDB-T, or ATSC) and 2 ASI inputs, multiplexing them into 4 IP outputs (MPTS). It features BISS descrambling, PID re-mapping, and PSI/SI table generation for optimized signal delivery. With web-based management and 2 GE mirrored ports, it suits CATV broadcasting and headend consolidation projects.

Key feature

Web-Based Control: Remote configuration via intuitive browser interface

Working Principle

Specifications

| Input | 16 tuners input, (DVB-C, DVB-T/(T), ISDBT, S/S2/S2X, ATSC optional), F type 2 ASI input, BNC interface |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Tuner Section |  | DVB-C | Input Frequency |  |  | 30 ~1000 MHz |
|  |  |  | Standard |  |  | J.83A(DVB-C), J.83B, J.83C |
|  |  |  | Constellation |  |  | 16/32/64/128/256 QAM |
|  |  | DVB-T/(T) | Input Frequency |  |  | 30 ~1000 MHz |
|  |  |  | Bandwidth |  |  | 6/7/8 M |
|  | Version 1 | DVB-S | Input Frequency |  |  | 950~2150MHz |
|  |  |  | Symbol rate |  |  | 1~45 Msps |
|  |  |  | FEC |  |  | 1/2, 2/3, 3/4, 5/6, 7/8 |
|  |  |  | Constellation |  |  | QPSK |
|  |  |  | Signal Strength |  |  | - 65~ -25dBm |
|  |  |  | Max input bitrate |  |  | ≤120 Mbps |
|  |  | DVB-S2 | Input Frequency |  |  | 950~2150MHz |
|  |  |  | Symbol rate |  |  | 1~45 Msps |
|  |  |  | FEC |  |  | 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 |
|  |  |  | Constellation |  |  | QPSK, 8PSK |
|  |  |  | Max input bitrate |  |  | ≤120 Mbps |
|  | Version 2 (New) | DVB-S | Input Frequency |  |  | 950~2150MHz |
|  |  |  | Symbol rate |  |  | 0.5~45Msps |
|  |  |  | Signal Strength |  |  | - 65- -25dBm |
|  |  |  | FEC |  |  | 1/2, 2/3, 3/4, 5/6, 7/8 |
|  |  |  | Constellation |  |  | QPSK |
|  |  |  | Max input bitrate |  |  | ≤120 Mbps |
|  |  | DVB-S2 | Input Frequency |  |  | 950~2150MHz |
|  |  |  | Symbol rate |  |  | QPSK/8PSK /16APSK :0.5~45 Msps 32APSK: 0.5~40Msps; |
|  |  |  | FEC |  |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10 |
|  |  |  | Constellation |  |  | QPSK, 8PSK, 16APSK, 32APSK |
|  |  |  | Max input bitrate |  |  | ≤120 Mbps |
|  |  | DVB-S2X | Input Frequency |  |  | 950~2150MHz |
|  |  |  | Symbol rate |  |  | QPSK/8PSK /16APSK :0.5~45 Msps 8APSK/32APSK: 0.5~40Msps |
|  |  |  | FEC |  |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36 , 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 2/3-L, 32/45, 11/15, 7/9 |
|  |  |  | Constellation |  |  | QPSK, 8PSK, 8APSK, 16APSK, 32APSK |
|  |  |  | Max input bitrate |  |  | ≤120 Mbps |
|  |  | ISDB-T | Input Frequency |  |  | 30~1000MHz |
|  |  | ATSC | Input Frequency |  |  | 50 MHz~960MHz |
|  |  |  | Bandwidth |  |  | 6M |
|  |  |  | Constellation |  |  | 8VSB |
| Re-multiplex |  | PID re-mapping |  |  |  |  |
|  |  | PCR accurate adjusting |  |  |  |  |
|  |  | Automatic generating PSI/SI table |  |  |  |  |
| PID |  | Output range |  |  | 0x0000—0x1FFF |  |
|  |  | PID transparent |  |  | Any PID transparent and mapping achievable |  |
|  |  | Amount of output PID per input |  |  | 128 |  |
| BISS descrambling |  | Mode 1, Mode E (Up to 850Mbps) (descramble individual program) |  |  |  |  |
| Output |  | 4 IP (MPTS) output over UDP and RTP/RTSP protocol through GE1 and GE2 port, Unicast and Multicast |  |  |  |  |
| System |  | Remote management |  | Web-server Management |  |  |
|  |  | Language |  | English |  |  |
|  |  | Upgrade |  | USB, web management |  |  |
| Miscellaneous |  | Dimension |  | 482mm×410mm×44mm (W×L×H) |  |  |
|  |  | Approx weight |  | 3.5kg |  |  |
|  |  | Environment |  | 0~45℃(work)；-20~80℃（Storage） |  |  |
|  |  | Power requirements |  | 100~240VAC, 50/60Hz |  |  |
|  |  | Power consumption |  | 20W |  |  |

Tuner Input Options:

- 8 DVB-C Tuners + 8 ATSC Tuners

- 8 DVB-S2/S2X Tuners + 8 DVB-(T) Tuners

- 8 DVB- S2/S2X Tuners + 8 ISDB-T Tuners

- 16 DVB-S/S2/S2X/DVB-C/(T) /ISDB-T/ATSC Tuners

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227