FMUSER FUQ-4322AB HD Encoder 2 Inputs HDMI-HD-SDI 3 Outputs ASI-IP HEVC-H.265 H.264 Encoding OSD Insertion Broadcasting Live Streaming

Product overview

The FMUSER FUQ-4322AB is a professional HD encoder supporting HEVC/H.265 and H.264 video encoding. It accepts inputs from HDMI and HD-SDI sources and outputs via ASI or IP with OSD insertion. Control is managed through web management. Ideal for broadcasting applications like live streaming and television transmission.Multiple audio and video encoding formats are available to meet customers’ various requirements. It supports 2 stereo (4 mono) to mix with the video to form a stream output. The encoded program will finally output in TS through ASI/IP port.

Key feature

- Supports HEVC/H.265 and H.264 HD video encoding with B-frame support.

- Features OSD insertion for QR codes, logos, and captions.

- Offers flexible audio encoding supporting 2 stereos or 4 mono channels with MPEG-1 Layer II, HE-AAC, and LC-AAC.

- Provides multiple output options including ASI and IP over UDP/RTP/RTSP.

Inner Principle Chart

- Supports HEVC/H.265 and H.264 HD video encoding with B-frame support.

- Features OSD insertion for QR codes, logos, and captions.

- Offers flexible audio encoding supporting 2 stereos or 4 mono channels with MPEG-1 Layer II, HE-AAC, and LC-AAC.

- Provides multiple output options including ASI and IP over UDP/RTP/RTSP.

Specifications

| Video | Interface | 1×HDMI, 1×HD-SDI |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Resolution | Input |  | Output |  |
|  |  |  |  | MPEG-4 AVC/H.264 HD | HEVC/H.265 HD |
|  |  | 1920×1080p@60/59.94 |  | 1920×1080p@60/59.94 | 1920×1080p@60/59.94 |
|  |  | 1920×1080p@50 |  | 1920×1080p@50 | 1920×1080p@50 |
|  |  | 1920×1080i@60/59.94 |  | 1920×1080i@60 1920×1080p@60/30 | 1920×1080p@60 |
|  |  | 1920×1080i @50 |  | 1920×1080i@50 1920×1080p@50/25 | 1920×1080p@50 |
|  |  | 1280×720p@60/59.94 |  | 1280×720p@60/59.94 | 1280×720p@60/59.94 |
|  |  | 1280×720p@50 |  | 1280×720p@50 | 1280×720p@50 |
|  | Encoding | HEVC/H.265 HD; MPEG-4 AVC/H.264 HD |  |  |  |
|  | Bitrate Range | HEVC/H.265: 0.5 – 20.00 Mbps MPEG-4 AVC/H.264: 4 – 20.00 Mbps |  |  |  |
|  | Rate Control | CBR/VBR |  |  |  |
|  | GOP Structure | IPPP, IBBP |  |  |  |
|  | Chroma | 4:2:0 |  |  |  |
| Audio | Interface | 2×XLR, 2×AES, 2×RCA, 1×SDI (support maximum 2 stereos synchronous processing), 1×HDMI, |  |  |  |
|  | Encoding | MPEG-1 Layer II, HE-AAC (v1&v2), LC-AAC, DD AC3 passthrough |  |  |  |
|  | Sampling rate | 48KHz |  |  |  |
|  | Bit-rate | 48Kbps~384Kbps |  |  |  |
| Stream Input |  | 2 ASI Input for remux |  |  |  |
| Stream output |  | 2×ASI output ports, BNC interface |  |  |  |
|  |  | 1 IP (MPTS) over UDP / RTP /RTSP, 1000 Base-T Ethernet interface*2 with independent IP addresses (multicast/unicast) |  |  |  |
| System function |  | LCD/keyboard and web management |  |  |  |
|  |  | Language: English |  |  |  |
|  |  | Ethernet software upgrade |  |  |  |
| General |  | Dimensions | 482mm×405mm×44.5mm (W × D × H) |  |  |
|  |  | Temperature | 0~45℃(Operation), -20~80℃(Storage) |  |  |
|  |  | Power | AC110V±10%, 50/60Hz; AC 220V±10%,50/60Hz |  |  |
|  |  | Consumption | 15W |  |  |

- Supports HEVC/H.265 and H.264 HD video encoding with B-frame support.

- Features OSD insertion for QR codes, logos, and captions.

- Offers flexible audio encoding supporting 2 stereos or 4 mono channels with MPEG-1 Layer II, HE-AAC, and LC-AAC.

- Provides multiple output options including ASI and IP over UDP/RTP/RTSP.

Technical Comparison

| - Supports HEVC/H.265 and H.264 HD video encoding with B-frame support. - Features OSD insertion for QR codes, logos, and captions. - Offers flexible audio encoding supporting 2 stereos or 4 mono channels with MPEG-1 Layer II, HE-AAC, and LC-AAC. - Provides multiple output options including ASI and IP over UDP/RTP/RTSP. - Supports HEVC/H.265 and H.264 HD video encoding with B-frame support. - Features OSD insertion for QR codes, logos, and captions. - Offers flexible audio encoding supporting 2 stereos or 4 mono channels with MPEG-1 Layer II, HE-AAC, and LC-AAC. - Provides multiple output options including ASI and IP over UDP/RTP/RTSP. | FU-4322B | FU-4322A | FU-4322HV |
| --- | --- | --- | --- |
| Chipset | Magnum | Magnum | Fujitsu |
| Video Encoding | MPEG-2 HD/SD; MPEG-4 AVC/H.264 HD/SD | MPEG-2 HD/SD; MPEG-4 AVC/H.264 HD/SD | HEVC/H.265 HD MPEG-4 AVC/H.264 HD |
| Bit Rate | 0.20 – 60.00 Mbps | 0.25Mbps~60.5Mbps | HEVC/H.265: 0.5 – 20.00 Mbps MPEG-4 AVC/H.264: 4 – 20.00 Mbps |
| HDMI Resolution | HD | HD/SD | HD |
| Audio Encode | MPEG-1 Layer II, HE-AAC (v1&v2), LC-AAC, DD AC3 (2.0), DD AC3 passthrough (for SDI in) | MPEG-1 Layer II, HE-AAC (v1&v2), LC-AAC; DD AC3 passthrough | MPEG-1 Layer II, HE-AAC (v1&v2), LC-AAC, DD AC3 passthrough (for SDI and HDMI) |
| Audio Channel | Up to 4 stereos | Up to 2 stereos | Up to 2 stereos |
| Rate Control | CBR | CBR/VBR | CBR/VBR |
| Aspect Ratio | 16:9, 4:3, 14:9 | 16:9, 4:3, 1:1, 2.35:1 | - Supports HEVC/H.265 and H.264 HD video encoding with B-frame support. - Features OSD insertion for QR codes, logos, and captions. - Offers flexible audio encoding supporting 2 stereos or 4 mono channels with MPEG-1 Layer II, HE-AAC, and LC-AAC. - Provides multiple output options including ASI and IP over UDP/RTP/RTSP. |
| Chroma | 4:2:0, 4:2:2 | 4:2:0, 4:2:2 | 4:2:0 |
| One-seg | N/A | Yes | N/A |
| Video Buffer | Yes | N/A | N/A |
| ASI In & Mux | N/A | 1 ASI input for remux | 2 ASI input for remux |
| IP out | 1SPTS over UDP/RTP | MPTS/8*SPTS over UDP/RTP, | 1 MPTS over UDP / RTP /RTSP |
| Low Delay | 150ms, 200ms, 350ms, 650ms | 1400ms - 5000ms, 1ms stepping | N/A |
| Closed Caption | EIA 608 and EIA 708 | EIA 608 and EIA 708 | EIA 608 |
| OSD Function | Yes | N/A | Yes |
| Management | Web-server | Web-server, SNMP | Web-server |

- Supports HEVC/H.265 and H.264 HD video encoding with B-frame support.

- Features OSD insertion for QR codes, logos, and captions.

- Offers flexible audio encoding supporting 2 stereos or 4 mono channels with MPEG-1 Layer II, HE-AAC, and LC-AAC.

- Provides multiple output options including ASI and IP over UDP/RTP/RTSP.

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227