FMUSER FUQ-4322B Professional 4K Encoder Dual HDMI 2.0 Inputs 4 ASI Outputs IP MPTS UDP-RTP H.265 HEVC H.264 Broadcast Streaming

Product Overview

The FMUSER FUQ-4322B is a 4K encoder featuring dual HDMI 2.0 inputs with hot redundancy, supporting H.265/HEVC and H.264/AVC video encoding. It outputs 1 TS via 4 ASI ports and 1 MPTS over IP with real-time bit-rate monitoring. Controlled via web-based NMS, it is ideal for broadcast and streaming applications.

Key feature

Supports dual HDMI 2.0 inputs with hot redundancy and loop-out for 1080P/4K conversion.

Encodes video in H.265/HEVC and H.264/AVC formats with CBR control (1–23Mbps).

Outputs via 4 ASI ports and 1 MPTS over IP (UDP/RTP, unicast/multicast).

Features broadcast enhancements including logo insertion, captioning, and QR code insertion.

(1080P and 4K convert to each other)

Specifications

| Input | 1×HDMI input (2.0) with 2 HDMI port, HDCP 2.2/1.4 |  |  |  |
| --- | --- | --- | --- | --- |
|  | 2 ASI input, BNC interface |  |  |  |
|  | 32 IP input over UDP and RTP protocol |  |  |  |
| Video Input | Resolution |  |  | Chroma |
|  | 3840×2160_60P,3840×2160_59.94P, 3840×2160_50P,3840×2160_30P, 3840×2160_29.96P,3840×2160_25P, 3840×2160_24P,1920×1080_60P, 1920×1080_59.94P,1920×1080_50P, 1920×1080_30P ,1920×1080_25P 1920×1080_24P, |  |  | RGB, YCbCr 4:4:4, YCbCr 4:2:2, YCbCr 4:2:0 (2160P_50/60Hz only) |
| Video Encoding | Encoding | H.265/HEVC, H.264/AVC |  |  |
|  | Chroma | 4:2:2, 4:2:0 |  |  |
|  | Bit-rate | 1Mbps~23Mbps |  |  |
|  | Rate Control | CBR |  |  |
| Audio Encoding | Encoding | MPEG-1 Layer 2, LC-AAC |  |  |
|  | Sampling rate | 48KHz |  |  |
|  | Bit-rate | 64Kbps~256Kbps each channel |  |  |
| Multiplexing | 2 ASI input multiplexed with local 1 channel of TS |  |  |  |
|  | PID remapping (automatically or manually) |  |  |  |
|  | Accurate PCR adjusting |  |  |  |
|  | Generate PSI/SI table automatically |  |  |  |
| Stream output | 1*ASI output through 4 BNC interfaces |  |  |  |
|  | 1 MPTS over UDP/RTP, 1000M/100M Base-T Ethernet interface (unicast/ multicast) |  |  |  |
| System function | LCD/keyboard operating, web NMS supporting |  |  |  |
|  | Ethernet software & hardware upgrade |  |  |  |
| Miscellaneous | Dimension (W× L× H) |  | 482mm×455mm×44mm |  |
|  | Approx weight |  | 4kg |  |
|  | Temperature |  | 0~45℃(work), -20~80℃（Storage） |  |
|  | Power |  | AC 100V-220V±10%, 50/60Hz |  |
|  | Consumption |  | 25W |  |

Principle Chart

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227