FMUSER FUQ-4353 Dual-Channel Encoder HDMI-SDI-YPbPr-CVBS Inputs ASI-IP Outputs MPEG-2 H.264 Encoding Web NMS Broadcast IPTV Solution

Product Overview

The FMUSER FUQ-4353 is a dual-channel encoder supporting HD/SD MPEG-2 & H.264 video and MP2/AAC/AC3 audio. It accepts pluggable HDMI/3G-SDI/YPbPr+CVBS inputs and delivers ASI re-muxed outputs (MPTS/SPTS) or IP streams (UDP/RTP). The 1RU rack-mount unit features PSI/SI editing and web-based NMS for broadcast and IPTV workflows.

Key feature

Centralized Control: Web GUI and SNMP v2 for remote management via network

Technical Specifications

| HDMI Input | Video | Input | HDMI*1 (with one for backup) |
| --- | --- | --- | --- |
|  |  | Encoding | MPEG2; MPEG4 AVC/H.264 |
|  |  | Bitrate | 1-19.5Mbps |
|  |  | Resolution | 1920*1080_60P, 1920*1080_50P, (-for MPEG4 AVC/H.264 only) 1920*1080_60i, 1920*1080_50i, 1280*720_60p, 1280*720_50P 720*480_60i, 720*576_50i |
|  |  | Low Delay | Normal, Mode 1, Mode 2 |
|  |  | Chroma | 4:2:0 |
|  |  | Aspect Ratio | 16:9,4:3 |
|  | Audio | Encoding | MPEG1 Layer II;MPEG 2-AAC; MPEG 4-AAC and Dolby Digital AC3 2.0 |
|  |  | Sample rate | 48KHz |
|  |  | Bitrate | 64/96/128/ 192/256/320kbps |
| HDMI/YPbPr/CVBS 3-in-1 Input (under developing) | Video (HDMI) | Encoding | MPEG2; MPEG4 AVC/H.264 |
|  |  | Input | HDMI*1 |
|  |  | Bitrate | 1-19.5Mbps |
|  |  | Resolution | 1920*1080_60P, 1920*1080_50P, -for MPEG4 AVC/H.264 only) 1920*1080_60i, 1920*1080_50i, 1280*720_60p, 1280*720_50P 720*480_60i, 720*576_50i |
|  |  | Low Delay | Normal, Mode 1, Mode 2 |
|  |  | Chroma | 4:2:0 |
|  |  | Aspect Ratio | 16:9,4:3 |
|  | Audio (HDMI) | Encoding | MPEG1 Layer II ,MPEG2-AAC, MPEG4-AAC and Dolby Digital AC3 2.0 |
|  |  | Input | HDMI*1 |
|  |  | Sample rate | 48KHz |
|  |  | Bitrate | 64/96/128/ 192/256/320kbps |
|  | Video (YpbPr/ CVBS) | Encoding | MPEG2; MPEG4 AVC/H.264 |
|  |  | Input | YpbPr*1 / CVBS *1 |
|  |  | Bitrate | 1-19.5Mbps |
|  |  | Resolution | CVBS: 720x576_50i (PAL); 720x480_60i (NTSC) YpbPr: 1920*1080_60i, 1920*1080_50i; 1280*720_60p, 1280*720_50P |
|  |  | Low Delay | Normal, Mode 1, Mode 2 |
|  |  | Chroma | 4:2:0 |
|  |  | Aspect Ratio | 16:9, 4:3 |
|  | Audio (YpbPr/ CVBS) | Encoding | MPEG1 Layer II; MPEG2-AAC; MPEG4-AAC and Dolby Digital AC3 2.0 |
|  |  | Interface | 1*Stereo/2*mono |
|  |  | Sample rate | 48KHz |
|  |  | Bit rate | 64/96/128/ 192/256/320kbps |
| SDI Input; Video; Encoding; MPEG2; MPEG4 AVC/H.264. Input; SDI*1. Bitrate; 1-19.5Mbps. Resolution; 1920*1080_60P, 1920*1080_50P, (-for MPEG4 AVC/H.264 only) 1920*1080_60i, 1920*1080_50i, 1280*720_60p, 1280*720_50P 720*480_60i, 720*576_50i. Low Delay; Normal, Mode 1, Mode 2. Chroma; 4:2:0. Aspect Ratio; 16:9,4:3. Audio; Encoding; MPEG1 Layer II ,MPEG2-AAC, MPEG4-AAC and Dolby Digital AC3 2.0. Sample rate; 48KHz. Bitrate; 64/96/128/ 192/256/320kbps. |

| General | System | Local interface | LCD + control buttons |
| --- | --- | --- | --- |
|  |  | Remote management | Web NMS |
|  |  | Stream In | 1 ASI in for re-multiplexing |
|  |  | Stream Out | 2 ASI out (BNC type, same one TS, mirror as MPTS or SPTS) |
|  |  |  | IP (1 MPTS & 2 SPTS) over UDP, RTP/RTSP (RJ45, 1000M) |
|  |  | DATA Port | 1000M |
|  |  | NMS interface | RJ45, 100M |
|  |  | Language | English |
|  | Physical Specification | Power supply | 100~240VAC, 50/60Hz |
|  |  | Dimensions | 482*300*44mm |
|  |  | Operation temp | 0~45℃ |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227