FMUSER FUQ-4353B HD-SD Encoder 4 Input Channels SDI-HDMI-ASI-IP Interfaces MPEG-2 H.264 Encoding ASI Re-multiplexing Broadcast Headend System

Product Overview

The FMUSER FUQ-4353B is a professional HD/SD encoder supporting MPEG-2 and H.264 video encoding with four SDI or HDMI inputs. It processes signals by encoding and re-multiplexing into MPTS or SPTS, outputting via ASI and IP. Featuring a modular drawer design, it is ideal for broadcast, cable TV, and streaming applications.HDMI inputs supporting MPEG‐2 and MPEG‐4 AVC/H.264 video encoding and

MPEG 2, MPEG 2‐AAC, MPEG 4‐AAC and DD AC3 audio encoding. The 4 encoded programs will output through ASI and IP ports in MPTS or SPTS, and there is also one ASI in port for re-multiplexing.

It adopts inner drawer‐type structural design which greatly facilitates the change of encoding modules if needed.

Key feature

Supports MPEG2 HD/SD and MPEG4 AVC/H.264 video encoding with resolution downscale conversion

Multiple audio encoding options: MPEG2, AAC, and AC3, including dialog normalization

Flexible input configurations: 4x SDI, 4x HDMI, or mixed inputs

Advanced features: low delay, closed caption support, and web-based NMS management

Principle Chart

Technical Specifications

Encoding Section

Video

| Encoding | MPEG2 & MPEG4 AVC/H.264 |
| --- | --- |
| Input | SDI*4 or HDMI*4 + 1 ASI in |
| Resolution | 1920*1080_60P, 1920*1080_50P, (-for MPEG4 AVC/H.264 only) 1920*1080_60i, 1920*1080_50i, 1280*720_60p, 1280*720_50P 720*480_60i, 720*576_50i |
| Support object resolution (for downscale conversion) | 1920*1080_60P, 1920*1080_50P, (-for MPEG4 AVC/H.264 only) 1440*1080_60i, 1440*1080_50i, 1280*720_60p, 1280*720_50P 720*480_60i, 720*576_50i |
| Bit Rate | 1~19.5Mbps |
| Chroma Sample | 4:2:0 |
| Aspect Ratio | 16:9, 4:3 |

Audio

| encoding | MPEG 2, MPEG2-AAC, MPEG4-AAC, Dolby Digital AC3 (2.0) |
| --- | --- |
| Dialog Normalization Applicable for DD AC3 encoding only) | -31 ~ -1 dB |
| Sample rate | 48KHz |
| Bit rate | 64kbps, 96kbps,128kbps, 192kbps, 256kbps, 320kbps |

System

| Local interface | LCD + control buttons |
| --- | --- |
| Remote management | Web NMS |
| Low Delay Mode | Normal, mode 1, mode 2, Manual |
| output | 2*ASI out (BNC type, mirror ports/the same one TS); IP (1 MPTS & 4 SPTS) over UDP, RTP/RTSP (RJ45, 1000M) |
| NMS interface | RJ45, 100M |
| Language | English |

General

| Power | AC 100V~240V |
| --- | --- |
| Dimensions | 482*400*44mm |
| Operation temperature | 0~45℃ |

Technical Comparision

|  | The Former Version | The Current Version（V2） |
| --- | --- | --- |
| ASI in | No | Yes |
| Bit rate Mode | CBR/VBR option | CBR |
| Audio Group/Pair option-SDI | No | Yes |
| Video Bit Rate | 0.5~19.5Mbps for H.264 encoding 1~19.5Mbps for MPEG-2 encoding | 1~19.5Mbps |
| Low Delay | Normal/Mode 1/Mode 2 | Normal/Mode 1/Mode 2/Manual |
| Character Encoding option | No | Yes |
| Output Protocol | UDP, RTP | UDP, RTP/RTSP |
| DATA Port | 100M port | 1000M port |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227