FMUSER FUQ-4353C 4-Channel Encoder 4x HDMI-YPbPr-CVBS Input 2x ASI-IP Output MPEG2 MPEG4 H.264 Dual-Output EIA608 Closed Captioning Digital TV Headend IPTV System

Product Overview

The FMUSER FUQ-4353C is a professional 4-in-1 HD encoder featuring 4x HDMI/YPbPr/CVBS inputs and dual-output via ASI (2x) + IP (UDP/RTP/RTSP). It performs real-time MPEG2/MPEG4 H.264 encoding with AC3 passthrough and EIA608 Closed Captioning support. The 1RU rack-mount unit offers web-based NMS control and firmware upgrades. Ideal for digital TV headends, IPTV systems, and satellite uplink applications.

Key feature

1. Multi-standard encoding: Supports MPEG2 & H.264 with HD-to-SD downscaling (1080i/720p/576i)

2. Flexible audio handling: AC3 2.0/5.1/7.1 passthrough + MPEG4-AAC/MP2 encoding

3. Dual-output architecture: 2x ASI outputs (SPTS/MPTS) + IP streaming (UDP/RTP/RTSP)

4. Broadcast features: EIA608 Closed Captioning, LCN support, low-latency mode

5. Remote management: Web GUI + SNMP v2/v3 for NMS integration

Principle Chart

Specifications

| Encoding Section |  |  |
| --- | --- | --- |
| Video (HDMI) |  |  |
| Encoding | MPEG2; MPEG4 AVC/H.264 |  |
| Interface | HDMI*4 |  |
| Resolution | Input | Output |
|  | 1920*1080_60P, 1920*1080_50P (For MPEG 4 AVC/H.264 only), 1920*1080_60i, 1920*1080_50i, 1280*720_60p, 1280*720_50P | 1920*1080_60i, 1920*1080_50i, 1440*1080_60i, 1440*1080_50i 1280*720_60p, 1280*720_50P, 720*480_60i, 720*576_50i |
| Low Delay | Normal, Mode 1, Mode 2 |  |
| Audio (HDMI) |  |  |
| Encoding | MPEG1 Layer II; MPEG2-AAC; MPEG4-AAC; DD AC3 (2.0) Optional; DD AC3 (2.0/5.1/7.1) passthrough |  |
| Interface | HDMI*4 /SPDIF*4 |  |
| Sample rate | 48KHz |  |
| Bit rate | 64/96/128/ 192/256/320kbps |  |
| Video (CVBS/YPbPr) |  |  |
| Encoding | MPEG2; MPEG4 AVC/H.264 |  |
| Interface | CVBS/YPbPr*4 (RCA) |  |
| Resolution | CVBS: 720*576_50i, 720*480_60i |  |
|  | YPbPr Input | YPbPr Output |
|  | 1920*1080_60i, 1920*1080_50i; 1280*720_60p, 1280*720_50P | 1920*1080_60i, 1920*1080_50i, 1440*1080_60i, 1440*1080_50i 1280*720_60p, 1280*720_50P, 720*480_60i, 720*576_50i |
| Low Delay | Normal, Mode 1, Mode 2 |  |
| Audio (L/R) |  |  |
| Encoding | MPEG1 Layer II; MPEG2-AAC; MPEG4-AAC; DD AC3 (2.0) Optional |  |
| Interface | 4*Stereo/8*mono/4*SPDIF |  |
| Sample rate | 48KHz |  |
| Bit rate | 64/96/128/ 192/256/320kbps |  |

| System |  |  |
| --- | --- | --- |
| Local interface | LCD + control buttons |  |
| Remote management | Web NMS |  |
| Stream Out | 2 ASI mirrored out (BNC type, 100M); IP (MPTS&4 SPTS) over UDP, RTP/RTSP out (RJ45, 100M) |  |
| NMS interface | RJ45, 100M |  |
| Aspect Ratio | 4:3; 16:9 |  |
| Language | English |  |
| General |  |  |
| Power supply |  | AC 100V~240V |
| Dimensions |  | 482*400*44mm |
| Weight |  | 4.5 kg |
| Operation temperature |  | 0~45℃ |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227