FUQ-4359 Transcoder 6 Tuner-ASI Inputs 8 IP Inputs DVB-S-S2-C-T-ISDB-T-ATSC to IP-ASI Outputs Bidirectional H.264-MPEG2 Transcoding BISS CC Support for Broadcast Operators

Product overview

The FUQ-4359 transcoder performs bidirectional conversion between H.264 and MPEG-2 formats, supporting HD/SD video. It includes 6 tuner or ASI inputs and 8 IP inputs, with outputs via IP or ASI. Managed via web-based NMS, it handles signal transcoding efficiently. Ideal for broadcast operators in television and media distribution.

Key feature

Supports bidirectional transcoding between H.264 and MPEG-2 for SD/HD programs.

Features flexible I/O with 6 tuner/ASI inputs, 8 IP inputs, 8 SPTS and 1 MPTS over IP/ASI outputs.

Includes BISS descrambling and closed captioning support.

Enables remote control and monitoring via web-based NMS.

Principle Chart (FU-4359)

Transcoding Mode Example

Technical Specifications

| Stream In | MPTS/SPTS over UDP/RTP, 1000M Base-T Ethernet Interface/ SFP interface |  |  |
| --- | --- | --- | --- |
|  | 6 * (DVB-S/S2/C/T/ISDB-T/ATSC) Tuners; 6 * ASI (optional) |  |  |
| BISS Descramble | Maximum 8 programs |  |  |
| Video | Resolution | 1920x1080P,1920x1080i,1280x720P, 720x576i, 720x480i 480x576, 544x576, 640x576, 704x576 |  |
|  | Trans-coding | 4*MPEG2 HD → 4*MPEG2/H.264 HD ; 4*MPEG2 HD → 4*MPEG2/H.264 SD ; 8 *MPEG2 SD → 8 *MPEG2/H.264 SD |  |
|  |  | 4* H.264 HD → 4*MPEG2/H.264 HD ; 4* H.264 HD → 4*MPEG2/H.264 SD ; 8* H.264 SD → 8 *MPEG2/H.264 SD |  |
|  | Rate Control | CBR/VBR |  |
| Audio | Trans-coding | MPEG-1 Layer 2, AAC and AC3(Optional) any-to-any or pass through |  |
|  | Sampling rate | 48KHz |  |
|  | Bit Rate | 64Kbps-384Kbps |  |
| Stream Out | 8*SPTS & 1*MPTS over UDP/RTP/RTSP, 1000M Base-T Ethernet Interface (UDP/RTP uni-cast / multicast) /SFP interface |  |  |
|  | 1*ASI (as a copy of one of the 8 SPTS or the MPTS) output, BNC interface |  |  |
| System Function | LCD & Key board control; web NMS management |  |  |
|  | Ethernet software upgrade |  |  |
| General | Dimensions |  | 430mm×405mm×45mm(WxDxH) |
|  | Approx weight |  | 4.8 Kg |
|  | Temperature range |  | 0~45℃(Operation), -20~80℃(Storage) |
|  | Power requirements |  | AC 110V±10%, 50/60Hz or AC 220V±10%,50/60Hz |
|  | Power consumption |  | 20W |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227