FMUSER FUQ-44105B 16-Channel DVB-C QAM Modulator with 16 DVB-C Tuners 512 IP Inputs 16 RF Outputs ETR289 Scrambling Multiplexing for CATV IPTV Headend

Product overview

The FMUSER FUQ-44105B is a high-density QAM modulator supporting 16x DVB-C tuner inputs, 2x ASI (BNC), and 512x IP streams via GE1 (UDP/RTP). It performs multiplexing, ETR289-compliant scrambling, and 16-256QAM modulation, delivering 16x RF outputs (50-960MHz) and 1x MPTS IP mirror (UDP/RTP/RTSP). The 1U rack-mount unit features MER ≥40dB and web-based control. Ideal for CATV digitization, IPTV redistribution, and multi-standard headend deployments.FU-44105C is also characterized with high integrated level, high performance and low cost. It supports dual power supply (optional). This is very adaptable to newly generation CATV broadcasting system.

Key feature

Broad Compliance: Supports DVB-C/T/S/S2/S2X, ATSC, ISDB-T, and J.83A/B/C standards

Working Principle

Specifications

| Input | 16 DVB-C (DVB-T/-(T)/-S/-S2/-S2X, ATSC, ISDB-T Optional) FTA Tuner |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 512 IP input through GE1 over UDP and RTP protocol |  |  |  |  |  |
|  | 2 ASI input, BNC interface |  |  |  |  |  |
| Tuner Section | Multi-mode tuners switchable (New) | DVB-C | Standard |  | J.83A(DVB-C), J.83B, J.83C |  |
|  |  |  | Frequency In |  | 60~890MHz |  |
|  |  |  | Constellation |  | 16/32/64/128/256 QAM |  |
|  |  | DVB-T/(T) | Frequency In |  | 60~890MHz |  |
|  |  |  | Bandwidth |  | 6/7/8 M bandwidth |  |
|  |  | ISDB-T | Frequency In |  | 60~890MHz |  |
|  | Version 1 | DVB-S/S2 | Frequency In |  | 950~2150MHz |  |
|  |  |  | Symbol rate |  | QPSK 1~45Mbauds 8PSK 1~45Mbauds |  |
|  |  |  | Code rate |  | 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 |  |
|  |  |  | Constellation |  | QPSK, 8PSK |  |
|  | Version 2 (New) | DVB-S | Frequency In |  | 950~2150MHz |  |
|  |  |  | Symbol rate |  | 0.5~45Msps |  |
|  |  |  | Signal Strength |  | - 65~-25dBm |  |
|  |  |  | FEC |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |
|  |  |  | Constellation |  | QPSK |  |
|  |  |  | Max input bitrate |  | ≤129 Mbps |  |
|  |  | DVB-S2 | Frequency In |  | 950~2150MHz |  |
|  |  |  | Symbol rate |  | QPSK/8PSK /16APSK: 0.5~45 Msps 32APSK: 0.5~40Msps; |  |
|  |  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10 |  |
|  |  |  | Constellation |  | QPSK, 8PSK, 16APSK, 32APSK |  |
|  |  |  | Max input bitrate |  | ≤129 Mbps |  |
|  |  | DVB-S2X | Frequency In |  | 950-2150MHz |  |
|  |  |  | Symbol rate |  | QPSK/8PSK /16APSK: 0.5~45 Msps 8APSK/32APSK: 0.5~40Msps |  |
|  |  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36 , 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10, 2/3-L, 32/45, 11/15, 7/9 |  |
|  |  |  | Constellation |  | QPSK, 8PSK, 8APSK, 16APSK, 32APSK |  |
|  |  |  | Max input bitrate |  | ≤129 Mbps |  |
|  |  | ATSC | Frequency In |  | 30~1000MHz |  |
|  |  |  | Bandwidth |  | 6M |  |
|  |  |  | Constellation |  | 8VSB |  |
| Multiplexing | Maximum PID Remapping |  | 360 output per channel |  |  |  |
|  | Function |  | PID remapping (automatically or manually) |  |  |  |
|  |  |  | Accurate PCR adjusting |  |  |  |
|  |  |  | Generate PSI/SI table automatically |  |  |  |
| Scrambling Parameters | Max simulscrypt CA |  | 4 |  |  |  |
|  | Scramble Standard |  | ETR289, ETSI 101 197, ETSI 103 197 |  |  |  |
|  | Connection |  | Local/remote connection |  |  |  |
| Modulation | QAM Channel |  | 16 non-adjacent carriers output |  |  |  |
|  | Standard |  | EN300 429/ITU-T J.83A/B |  |  |  |
|  | MER |  | ≥40db |  |  |  |
|  | RF frequency |  | 50~960MHz, 1KHz step |  |  |  |
|  | RF output level |  | -20~+10dbm(87~107 dbµV),0.1db step |  |  |  |
|  | Symbol Rate |  | 5.0Msps~7.0Msps, 1ksps stepping |  |  |  |
|  |  |  | J.83A |  |  | J.83B |
|  | Constellation |  | 16/32/64/128/256QAM |  |  | 64/256 QAM |
|  | Bandwidth |  | 8M |  |  | 6M |
| Stream out | 16 RF output (F type interface) |  |  |  |  |  |
|  | 1 IP (MPTS) output over UDP and RTP/RTSP (GE1 only), as mirror of one carrier |  |  |  |  |  |
| System | Network management (WEB) |  |  |  |  |  |
|  | Chinese and English language |  |  |  |  |  |
|  | Ethernet software upgrade |  |  |  |  |  |
| General | Dimension(W*D*H) |  |  | 482mm×300mm×44.5mm |  |  |
|  | Temperature |  |  | 0~45℃(Operation) ; -20~80℃(Storage) |  |  |
|  | Power |  |  | AC 100V±1050/60Hz; AC 220V±10%, 50/60HZ |  |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227