FMUSER FUQ-44105B QAM Modulator 16-24 DVB-S2 Tuner 128 IP 1 ASI Input 16 RF 16 IP Output DVB-S2 IP ASI QAM Multiplexing Scrambling EN300 429 ITU-T J.83A-B For CATV BroadcastingProduct overview

The FMUSER FUQ-44105B is a QAM modulator with integrated multiplexing and scrambling. It accepts inputs from up to 24 DVB-S2 tuners, 128 IP sources via GE1, and ASI, and outputs 16 RF carriers and 16 IP (MPTS) streams via GE2. The device processes signals through multiplexing, scrambling to ETR289/ETSI standards, and QAM modulation. Managed via web interface, it is ideal for modern CATV broadcasting systems.FU-44105C is also characterized with high integrated level, high performance and low cost. It supports dual power supply (optional). This is very adaptable to newly generation CATV broadcasting system.

Key feature

Supports up to 24 DVB-S2 tuners, 128 IP inputs over UDP/RTP, and ASI, with TS file playback via USB.

Delivers 16 non-adjacent RF outputs and 16 IP (MPTS) outputs for versatile distribution.

Integrates multiplexing, scrambling to ETR289/ETSI standards, and QAM modulation with MER≥40dB.

Enables PSI/SI editing, PCR adjusting, and web management for configuration.

Offers optional redundant power supply for enhanced reliability.

Working Principle

Specifications

| Input | 24 or 16 DVB-S/S2/S2X FTA Tuner input as per order |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 128 IP input through GE1 over UDP and RTP protocol |  |  |  |  |
|  | 1 ASI input, BNC interface |  |  |  |  |
|  | TS files input and playing via the USB disk (FAT 32) |  |  |  |  |
|  | Tuner | DVB-S | Frequency In |  | 950~2150MHz |
|  |  |  | Symbol rate |  | 0.5~45Msps |
|  |  |  | Signal Strength |  | - 65~-25dBm |
|  |  |  | FEC |  | 1/2, 2/3, 3/4, 5/6, 7/8 |
|  |  |  | Constellation |  | QPSK |
|  |  |  | Max input bitrate |  | ≤129 Mbps |
|  |  | DVB-S2 | Frequency In |  | 950~2150MHz |
|  |  |  | Symbol rate |  | QPSK/8PSK /16APSK: 0.5~45 Msps 32APSK: 0.5~40Msps; |
|  |  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10 |
|  |  |  | Constellation |  | QPSK, 8PSK, 16APSK, 32APSK |
|  |  |  | Max input bitrate |  | ≤129 Mbps |
|  |  | DVB-S2X | Frequency In |  | 950-2150MHz |
|  |  |  | Symbol rate |  | QPSK/8PSK /16APSK: 0.5~45 Msps 8APSK/32APSK: 0.5~40Msps |
|  |  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36 , 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10, 2/3-L, 32/45, 11/15, 7/9 |
|  |  |  | Constellation |  | QPSK, 8PSK, 8APSK, 16APSK, 32APSK |
|  |  |  | Max input bitrate |  | ≤129 Mbps |
| Multiplexing | Max PID Remapping |  | 360 output per channel |  |  |
|  | Function |  | PID remapping (automatically or manually) |  |  |
|  |  |  | Accurate PCR adjusting |  |  |
|  |  |  | Generate PSI/SI table automatically |  |  |
| Scrambling Parameters | Max simulscrypt CA |  | 4 |  |  |
|  | Scramble Standard |  | ETR289, ETSI 101 197, ETSI 103 197 |  |  |
|  | Connection |  | Local/remote connection |  |  |
| Modulation | QAM Channel |  | 16 non-adjacent carriers output |  |  |
|  | Standard |  | EN300 429/ITU-T J.83A/B |  |  |
|  | MER |  | ≥40db |  |  |
|  | RF frequency |  | 50~960MHz, 1KHz step |  |  |
|  | RF output level |  | -20~+10dbm(87~107 dbµV),0.1db step |  |  |
|  | Symbol Rate |  | 5.0Msps~7.0Msps, 1ksps stepping |  |  |
|  |  |  | J.83A |  | J.83B |
|  | Constellation |  | 16/32/64/128/256QAM |  | 64/256 QAM |
|  | Bandwidth |  | 8M |  | 6M |
| Stream out | 16 RF output (F type interface) |  |  |  |  |
|  | 16 IP (MPTS) output over UDP and RTP/RTSP (thru GE2), 2 ASI output(one as mirror) |  |  |  |  |
| System | Network management (WEB) |  |  |  |  |
|  | Chinese and English language |  |  |  |  |
|  | Ethernet software upgrade |  |  |  |  |
| General | Dimension (W*D*H) |  |  | 482mm×230mm×44mm |  |
|  | Temperature |  |  | 0~45℃(Operation) ; -20~80℃(Storage) |  |
|  | Power |  |  | AC 100V±1050/60Hz; AC 220V±10%, 50/60HZ |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227