FMUSER FUQ-44105B V3 QAM Modulator 24-16 Tuner 128 IP 2 ASI Input DVB-S-S2-S2X-C-T-T2-ISDB-T Input 16 DVB-C 16 IP 2 ASI Output BISS ETSI Scrambling CATV BroadcastingProduct overview

The FMUSER FUQ-44105D v3 is a QAM modulator supporting 24/16 tuner inputs, 128 IP inputs via DATA1, and 2 ASI inputs, with outputs of 16 DVB-C carriers and 16 IP (MPTS) streams. It processes signals by modulating into QAM format with optional BISS scrambling. Featuring web-based management for remote control, it is ideal for CATV broadcasting systems, ensuring reliable transmission.FU-44105C v3 is also characterized with high integrated level, high performance and low cost. This is very adaptable to a new generation CATV broadcasting system.

Key feature

Supports 24/16 FTA multi-mode tuner inputs for DVB-S/S2/S2X, DVB-C/T/T2, or ISDB-T with loop-out.

Features 128 IP inputs, 2 ASI inputs, and USB TS file playback for flexible source integration.

Outputs 8 non-adjacent DVB-T carriers and 8 IP (MPTS) outputs, plus 2 TS outputs via ASI.

Ensures MER ≥40dB with multiplexing, PCR adjusting, and PSI/SI editing via web control.

Working Principle

Specifications

| Input | 24 or 16 FTA DVB-S/S2/S2X or DVB-C/T/T2/ISDB-T multi-mode switchable tuner inputs as per order |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 128 IP inputs through DATA1 over UDP and RTP protocol |  |  |  |  |
|  | 2 ASI inputs, BNC interface |  |  |  |  |
|  | TS files input by playing via the USB disk (FAT 32, USB 2.0) |  |  |  |  |
|  | Tuner option 1 | DVB-S | Frequency In |  | 950~2150MHz |
|  |  |  | Symbol rate |  | 0.5~45Msps |
|  |  |  | Signal Strength |  | - 65~-25dBm |
|  |  |  | FEC |  | 1/2, 2/3, 3/4, 5/6, 7/8 |
|  |  |  | Constellation |  | QPSK |
|  |  |  | Max input bitrate |  | ≤129 Mbps |
|  |  | DVB-S2 | Frequency In |  | 950~2150MHz |
|  |  |  | Symbol rate |  | QPSK/8PSK /16APSK: 0.5~45 Msps 32APSK: 0.5~40Msps; |
|  |  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10 |
|  |  |  | Constellation |  | QPSK, 8PSK, 16APSK, 32APSK |
|  |  |  | Max input bitrate |  | ≤129 Mbps |
|  |  | DVB-S2X | Frequency In |  | 950-2150MHz |
|  |  |  | Symbol rate |  | QPSK/8PSK /16APSK: 0.5~45 Msps 8APSK/32APSK: 0.5~40Msps |
|  |  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36 , 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10, 2/3-L, 32/45, 11/15, 7/9 |
|  |  |  | Constellation |  | QPSK, 8PSK, 8APSK, 16APSK, 32APSK |
|  |  |  | Max input bitrate |  | ≤129 Mbps |
|  | Tuner option 2 | DVB-C | Standard |  | J.83A(DVB-C), J.83B, J.83C |
|  |  |  | Frequency In |  | 50~960MHz |
|  |  |  | Constellation |  | 16/32/64/128/256 QAM |
|  |  | DVB-T/T2 | Frequency In |  | 50~960MHz |
|  |  |  | Bandwidth |  | 6/7/8 M bandwidth |
|  |  | ISDB-T | Frequency In |  | 50~960MHz |
| Multiplexing | Max PID Remapping |  | 256 outputs per channel |  |  |
|  | Function |  | PID remapping (automatically or manually)/CA Filter |  |  |
|  |  |  | A/V PID Filter |  |  |
|  |  |  | Accurate PCR adjusting |  |  |
|  |  |  | Generate PSI/SI table automatically |  |  |
| Scrambling Parameters (optional) | Max simulscrypt CA |  | 4 |  |  |
|  | Scramble Standard |  | ETR289, ETSI 101 197, ETSI 103 197 |  |  |
|  | Connection |  | Local/remote connection |  |  |
| Modulation | QAM Channel |  | 16 non-adjacent carriers output, within 192MHz bandwidth |  |  |
|  | Standard |  | EN300 429/ITU-T J.83A/B |  |  |
|  | MER |  | ≥40db |  |  |
|  | RF frequency |  | 50~960MHz, 1KHz step |  |  |
|  | RF output level |  | -20~+3dbm(87~110 dbµV),0.1db step |  |  |
|  | Symbol Rate |  | 3600ksps~6952ksps, 1ksps stepping |  |  |
|  |  |  | J.83A |  | J.83B |
|  | Constellation |  | 16/32/64/128/256QAM |  | 64/256 QAM |
|  | Bandwidth |  | 8M |  | 6M |
| Stream out | 16 carriers output (F type interface) |  |  |  |  |
|  | 16 IP (MPTS) output over UDP and RTP/RTSP (thru DATA 2), 2 ASI outputs(2 TS as mirror) |  |  |  |  |
| System | Network management (WEB) |  |  |  |  |
|  | Chinese and English language |  |  |  |  |
|  | Ethernet software upgrade |  |  |  |  |
| General | Dimension (W*D*H) |  |  | 482mm×270mm×44mm |  |
|  | Temperature |  |  | 0~45℃(Operation) ; -20~80℃(Storage) |  |
|  | Power |  |  | AC 100V±1050/60Hz; AC 220V±10%, 50/60HZ |  |

Major Technical Comparison

| FU-44105C | v1 | v2 | v3 |
| --- | --- | --- | --- |
| LCD/KEY Feature | NO | NO | Yes |
| ASI inputs | 2 | 1 | 2 |
| ASI outputs | NO | 2 for one TS | 2 for two TS |
| Tuner inputs | 16 | 16 or 24 | 16 or 24 (FTA DVB-S/S2/S2X or DVB-C/T/T2/ISDB-T multi-mode switchable Tuner inputs with loop out) |
| RF loop out | Yes | No | Yes |
| IP outputs | 1 MPTS output | 16 MPTS output | 16 MPTS output |
| USB | N/A | Playing TS | Playing TS |
| DATA Ports | IP inputs/outputs via GE1 | IP inputs via DATA 1 IP outputs via DATA 2 | IP inputs via DATA 1 IP outputs via DATA2 |
| Reboot in Web after firmware upgrading | N/A | N/A | Yes |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227