FMUSER FUQ-44105R DVB-T Modulator 24-16 Tuner 256 IP 1 ASI Input 8 RF 8 IP Output EN300 744 QPSK-16QAM-64QAM BISS Descrambling For Digital Broadcasting Headend

Product overview

The FMUSER FUQ-44105R modulator processes 24/16 FTA DVB-S/S2/S2X tuner inputs, 256 IP inputs via GE ports, and ASI input. It performs multiplexing and DVB-T modulation to deliver 8 RF carriers and 8 IP MPTS streams. Controlled via web management with optional dual power supply, this rack-mountable device supports broadcast redundancy in digital terrestrial television systems.

FU-44105T is also characterized with high integrated level, high performance and low cost. It supports dual power supply (optional). This is very adaptable to newly generation broadcasting system.

Key feature

- Supports 24/16 FTA DVB-S/S2/S2X tuners, 256 UDP/RTP IP inputs, and ASI input

- Generates 8 non-adjacent DVB-T RF outputs with 8 mirrored IP MPTS streams

- Performs BISS descrambling (Version B), multiplexing, and PSI/SI editing

- Provides web-based control for configuration and monitoring

- Optional redundant power supply ensures continuous operation

Working Principle

Specifications

| Input | 24 or 16 FTA DVB-S/-S2/-S2X Tuner input |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 256 IP (GE1&GE2) input over UDP and RTP protocol |  |  |  |  |  |
|  | 1 ASI input, BNC interface |  |  |  |  |  |
|  | TS files input and playing via the USB disk (FAT 32) |  |  |  |  |  |
|  | Tuners | DVB-S |  |  | Frequency In | 950~2150MHz |
|  |  |  |  |  | Symbol rate | 0.5~45Msps |
|  |  |  |  |  | Signal Strength | - 65~-25dBm |
|  |  |  |  |  | FEC | 1/2, 2/3, 3/4, 5/6, 7/8 |
|  |  |  |  |  | Constellation | QPSK |
|  |  |  |  |  | Max input bitrate | ≤120 Mbps |
|  |  | DVB-S2 |  |  | Frequency In | 950~2150MHz |
|  |  |  |  |  | Symbol rate | QPSK/8PSK /16APSK: 0.5~45 Msps 32APSK: 0.5~40Msps; |
|  |  |  |  |  | FEC | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10 |
|  |  |  |  |  | Constellation | QPSK, 8PSK, 16APSK, 32APSK |
|  |  |  |  |  | Max input bitrate | ≤120 Mbps |
|  |  | DVB-S2X |  |  | Frequency In | 950-2150MHz |
|  |  |  |  |  | Symbol rate | QPSK/8PSK /16APSK: 0.5~45 Msps 8APSK/32APSK: 0.5~40Msps |
|  |  |  |  |  | FEC | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36 , 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10, 2/3-L, 32/45, 11/15, 7/9 |
|  |  |  |  |  | Constellation | QPSK, 8PSK, 8APSK, 16APSK, 32APSK |
|  |  |  |  |  | Max input bitrate | ≤120 Mbps |
| Multiplexing | Maximum PID Remapping |  | 256 output per channel |  |  |  |
|  | Function |  | PID remapping (automatically or manually) |  |  |  |
|  |  |  | A/V PID filtering (not for FU-44105T version B) |  |  |  |
|  |  |  | Accurate PCR adjusting |  |  |  |
|  |  |  | Generate PSI/SI table automatically |  |  |  |
| Modulation | Standard |  | EN300 744 |  |  |  |
|  | FFT |  | 2K 4K 8K |  |  |  |
|  | Bandwidth |  | 6M, 7M, 8M |  |  |  |
|  | Constellation |  | QPSK, 16QAM, 64QAM |  |  |  |
|  | Guard interval |  | 1/4, 1/8, 1/16, 1/32 |  |  |  |
|  | FEC |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |  |  |
|  | MER |  | ≥42 dB |  |  |  |
|  | RF out |  | 8 non-adjacent carrier output |  |  |  |
|  | RF frequency |  | 50~960MHz |  |  |  |
|  | RF output level |  | -20~+10dBm, 0.1dB step |  |  |  |
| Stream out | 8 DVB-T RF output (F type interface) |  |  |  |  |  |
|  | 8 IP (MPTS) output over UDP and RTP/RTSP (GE1 only), as mirror of carriers 2 ASI output, one as mirror |  |  |  |  |  |
| System | Network management (WEB) |  |  |  |  |  |
|  | Chinese and English language |  |  |  |  |  |
|  | Ethernet software upgrade |  |  |  |  |  |
| General | Dimension(W*D*H) |  |  | 482mm×230mm×44mm |  |  |
|  | Temperature |  |  | 0~45℃(Operation) ; -20~80℃(Storage) |  |  |
|  | Power |  |  | AC 100V±1050/60Hz; AC 220V±10%, 50/60HZ |  |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227