FMUSER FUQ-44105R V3 DVB-T Modulator 24-16 Tuner Inputs 128 IP 2 ASI Input Multi-Standard DVB-S-S2-S2X-C-T-T2-ISDB-T Input 8 DVB-T 8 IP 2 ASI Output Integrated Multiplexing PSI-SI Editing Broadcasting Headend

Product overview

The FMUSER FUQ-44105R V3 is a DVB-T modulator with 24/16 FTA tuner inputs for DVB-S/S2/S2X, DVB-C/T/T2, or ISDB-T, plus 128 IP inputs and 2 ASI inputs. It modulates signals into 8 non-adjacent DVB-T carriers and outputs 8 IP (MPTS) and 2 TS via ASI. Controlled via web management, this device supports multiplexing and PSI/SI editing. Ideal for broadcast, IPTV, and cable TV systems.FU-44105T is also characterized with high integrated level, high performance and low cost. This is very adaptable to a new generation broadcasting system.

Key feature

Integrated demodulating, re-multiplexing, BISS descrambling, and modulating in one unit.

Supports 4 tuner inputs for DVB-S/S2/S2X or DVB-C/T/T2/ISDB-T standards.

Provides 4 DVB-C RF or 2 DVB-T RF outputs for versatile distribution.

Enables PID pass-through for seamless program identifier transmission.

Features LCD/keyboard and web-based NMS for control and upgrades.

Working Principle

Specifications

| Input | 24 or 16 FTA DVB-S/-S2/-S2X or DVB-C/T/T2/ISDB-T Multi-Mode switchable tuner inputs |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 128 IP inputs over UDP and RTP protocol |  |  |  |  |  |
|  | 2 ASI inputs, BNC interface |  |  |  |  |  |
|  | TS files input by playing ts via the USB disk (FAT 32, USB 2.0) |  |  |  |  |  |
|  | Tuner Option 1 | DVB-S |  |  | Frequency In | 950~2150MHz |
|  |  |  |  |  | Symbol rate | 0.5~45Msps |
|  |  |  |  |  | Signal Strength | - 65~-25dBm |
|  |  |  |  |  | FEC | 1/2, 2/3, 3/4, 5/6, 7/8 |
|  |  |  |  |  | Constellation | QPSK |
|  |  |  |  |  | Max input bitrate | ≤120 Mbps |
|  |  | DVB-S2 |  |  | Frequency In | 950~2150MHz |
|  |  |  |  |  | Symbol rate | QPSK/8PSK /16APSK: 0.5~45 Msps 32APSK: 0.5~40Msps; |
|  |  |  |  |  | FEC | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10 |
|  |  |  |  |  | Constellation | QPSK, 8PSK, 16APSK, 32APSK |
|  |  |  |  |  | Max input bitrate | ≤120 Mbps |
|  |  | DVB-S2X |  |  | Frequency In | 950-2150MHz |
|  |  |  |  |  | Symbol rate | QPSK/8PSK /16APSK: 0.5~45 Msps 8APSK/32APSK: 0.5~40Msps |
|  |  |  |  |  | FEC | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36 , 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10, 2/3-L, 32/45, 11/15, 7/9 |
|  |  |  |  |  | Constellation | QPSK, 8PSK, 8APSK, 16APSK, 32APSK |
|  |  |  |  |  | Max input bitrate | ≤120 Mbps |
|  | Tuner Option 2 | DVB-C |  |  | Standard | J.83A(DVB-C), J.83B, J.83C |
|  |  |  |  |  | Frequency In | 50~960MHz |
|  |  |  |  |  | Constellation | 16/32/64/128/256 QAM |
|  |  | DVB-T/T2 |  |  | Frequency In | 50~960MHz |
|  |  |  |  |  | Bandwidth | 6/7/8 M bandwidth |
|  |  | ISDB-T |  |  | Frequency In | 50~960MHz |
| Multiplexing | Maximum PID Remapping |  | 256 output per channel |  |  |  |
|  | Function |  | PID remapping (automatically or manually)/CA Filter |  |  |  |
|  |  |  | A/V PID filtering |  |  |  |
|  |  |  | Accurate PCR adjusting |  |  |  |
|  |  |  | Generate PSI/SI table automatically |  |  |  |
| Modulation | Standard |  | EN300 744 |  |  |  |
|  | FFT |  | 2K 4K 8K |  |  |  |
|  | Bandwidth |  | 6M, 7M, 8M |  |  |  |
|  | Constellation |  | QPSK, 16QAM, 64QAM |  |  |  |
|  | Guard interval |  | 1/4, 1/8, 1/16, 1/32 |  |  |  |
|  | FEC |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |  |  |
|  | MER |  | ≥40 dB |  |  |  |
|  | RF out |  | 8 non-adjacent carrier output within 192MHz bandwidth |  |  |  |
|  | RF frequency |  | 50~960MHz |  |  |  |
|  | RF output level |  | -20~+3dBm, 0.1dB step |  |  |  |
| Stream out | 8 DVB-T carriers output (F type interface) |  |  |  |  |  |
|  | 8 IP (MPTS) output over UDP and RTP/RTSP (DATA2, as mirror of carriers) 2 ASI outputs (2 TS as mirror) |  |  |  |  |  |
| System | Network management (WEB) |  |  |  |  |  |
|  | Chinese and English language |  |  |  |  |  |
|  | Ethernet software upgrade |  |  |  |  |  |
| General | Dimension(W*D*H) |  |  | 482mm×270mm×44mm |  |  |
|  | Temperature |  |  | 0~45℃(Operation) ; -20~80℃(Storage) |  |  |
|  | Power |  |  | AC 100V±1050/60Hz; AC 220V±10%, 50/60HZ |  |  |

Major Technical Comparison

| FU-44105T | v1 | v2 | v3 |
| --- | --- | --- | --- |
| LCD/KEY Feature | NO | NO | Yes |
| ASI inputs | 2 | 1 | 2 |
| ASI outputs | NO | 2 for one TS | 2 for two TS |
| Tuner inputs | 16 | 16 or 24 | 16 or 24 (FTA DVB-S/S2/S2X or DVB-C/T/T2/ISDB-T Multi-Mode switchable tuner inputs with loop out) |
| RF loop out | Yes | No | Yes |
| IP outputs | 1 MPTS output | 8 MPTS output | 8 MPTS output |
| USB | N/A | Playing TS | Playing TS |
| DATA Ports | IP inputs/outputs via GE1 | IP inputs via DATA 1 IP outputs via DATA 2 | IP inputs via DATA 1 IP outputs via DATA2 |
| Reboot in Web after firmware upgrading | N/A | N/A | Yes |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227