FMUSER FUQ-44105R DVB-T Modulator 16 Tuner 256 IP 2 ASI Input 8 RF 8 IP Output GE-ASI Interface BISS Descrambling Multiplexing EN300744 Broadcasting Systems

Product overview

The FMUSER FUQ-44105R is a DVB-T modulator with 16 FTA tuner inputs, 256 IP inputs via GE1 & GE2, and 2 ASI ports, delivering 8 DVB-T RF outputs and 8 IP outputs. It processes signals through BISS descrambling (version B only), multiplexing, DVB-T modulation, PCR adjustment, and PSI/SI editing. Managed via web-based interface, this modulator is ideal for modern broadcasting systems, ensuring efficient signal distribution and integration.FU-44105T is also characterized with high integrated level, high performance and low cost. It supports dual power supply (optional). This is very adaptable to newly generation broadcasting system.

Key feature

Supports 16 FTA tuner inputs with standards including DVB-C, DVB-T, DVB-S/S2/S2X, ATSC, and ISDB-T.

Features 256 IP inputs and 2 ASI inputs via UDP and RTP protocols for flexible re-muxing.

Delivers 8 DVB-T RF outputs and 8 IP outputs with non-adjacent carriers and MPTS streams.

Includes BISS descrambling (version B only), multiplexing, DVB-T modulation, PCR adjustment, and PSI/SI editing.

Enables web-based management for remote control, updates, and monitoring.

Working Principle

Specifications

| Input | 16 DVB-C (DVB-T/(T)/-S/S2/S2X, ATSC, ISDB-T optional) FTA Tuner (DVB-S2X is only for FU-44105T version B) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 256 IP (GE1&GE2) input over UDP and RTP protocol |  |  |  |  |  |
|  | 2 ASI input, BNC interface |  |  |  |  |  |
| Tuner Section |  | DVB-C |  |  | Standard | J.83A(DVB-C), J.83B, J.83C |
|  |  |  |  |  | Frequency In | 60~890MHz |
|  |  |  |  |  | Constellation | 16/32/64/128/256 QAM |
|  |  | DVB-T/(T) |  |  | Frequency In | 60~890MHz |
|  |  |  |  |  | Bandwidth | 6/7/8 M bandwidth |
|  |  | ISDB-T |  |  | Frequency In | 60~890MHz |
|  | Version 1 | DVB-S/S2 |  |  | Frequency In | 950~2150MHz |
|  |  |  |  |  | Symbol rate | QPSK 1~45Mbauds 8PSK 1~45Mbauds |
|  |  |  |  |  | Code rate | 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 |
|  |  |  |  |  | Constellation | QPSK, 8PSK |
|  | Version 2 (For FU-44105T version B) | DVB-S |  |  | Frequency In | 950~2150MHz |
|  |  |  |  |  | Symbol rate | 0.5~45Msps |
|  |  |  |  |  | Signal Strength | - 65~-25dBm |
|  |  |  |  |  | FEC | 1/2, 2/3, 3/4, 5/6, 7/8 |
|  |  |  |  |  | Constellation | QPSK |
|  |  |  |  |  | Max input bitrate | ≤120 Mbps |
|  |  | DVB-S2 |  |  | Frequency In | 950~2150MHz |
|  |  |  |  |  | Symbol rate | QPSK/8PSK /16APSK: 0.5~45 Msps 32APSK: 0.5~40Msps; |
|  |  |  |  |  | FEC | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10 |
|  |  |  |  |  | Constellation | QPSK, 8PSK, 16APSK, 32APSK |
|  |  |  |  |  | Max input bitrate | ≤120 Mbps |
|  |  | DVB-S2X |  |  | Frequency In | 950-2150MHz |
|  |  |  |  |  | Symbol rate | QPSK/8PSK /16APSK: 0.5~45 Msps 8APSK/32APSK: 0.5~40Msps |
|  |  |  |  |  | FEC | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36 , 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10, 2/3-L, 32/45, 11/15, 7/9 |
|  |  |  |  |  | Constellation | QPSK, 8PSK, 8APSK, 16APSK, 32APSK |
|  |  |  |  |  | Max input bitrate | ≤120 Mbps |
|  |  | ATSC |  |  | Frequency In | 30~1000MHz |
|  |  |  |  |  | Bandwidth | 6M |
|  |  |  |  |  | Constellation | 8VSB |
| BISS Descrambling | Mode 1, Mode E (descramble individual program) (only for FU-44105T version B) |  |  |  |  |  |
| Multiplexing | Maximum PID Remapping |  | 256 output per channel |  |  |  |
|  | Function |  | PID remapping (automatically or manually) |  |  |  |
|  |  |  | A/V PID filtering (not for FU-44105T version B) |  |  |  |
|  |  |  | Accurate PCR adjusting |  |  |  |
|  |  |  | Generate PSI/SI table automatically |  |  |  |
| Modulation | Standard |  | EN300 744 |  |  |  |
|  | FFT |  | 2K 4K 8K |  |  |  |
|  | Bandwidth |  | 6M, 7M, 8M |  |  |  |
|  | Constellation |  | QPSK, 16QAM, 64QAM |  |  |  |
|  | Guard interval |  | 1/4, 1/8, 1/16, 1/32 |  |  |  |
|  | FEC |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |  |  |
|  | MER |  | ≥42 dB |  |  |  |
|  | RF out |  | 8 non-adjacent carrier output |  |  |  |
|  | RF frequency |  | 50~960MHz |  |  |  |
|  | RF output level |  | -20~+10dBm, 0.1dB step |  |  |  |
| Stream out | 8 DVB-T RF output (F type interface) |  |  |  |  |  |
|  | 8 IP (MPTS) output over UDP and RTP/RTSP (GE1 only), as mirror of carriers |  |  |  |  |  |
| System | Network management (WEB) |  |  |  |  |  |
|  | Chinese and English language |  |  |  |  |  |
|  | Ethernet software upgrade |  |  |  |  |  |
| General | Dimension(W*D*H) |  |  | 482mm×300mm×44.5mm |  |  |
|  | Temperature |  |  | 0~45℃(Operation) ; -20~80℃(Storage) |  |  |
|  | Power |  |  | AC 100V±1050/60Hz; AC 220V±10%, 50/60HZ |  |  |

Order Guide

|  | FU-44105T | FU-44105T (version B) | FU-44105C |
| --- | --- | --- | --- |
| DVB-S2X tuner in | × | √ | √ |
| Biss descrambling | × | √ | × |
| Scrambling | × | × | √ |
| A/V PID filtering | √ | × | × |
| IP Input | 256 | 256 | 512 |
| IP Output | 8 MPTS | 8 MPTS | 1 MPTS |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227