FMUSER FUQ-4412C QAM Modulator ASI-IP Input DVB-C 16-256QAM EN300 429-ITU-T J.83A-B-C PID Filtering NIT Insertion for Broadcast Satellite Product overview

The FMUSER FUQ-4412C is a QAM modulator supporting ASI and IP inputs for RF output. It processes transport streams with QAM modulation. Features include PID filtering, NIT insertion, and intelligent TS management. Ideal for broadcasting and interactive services.

Key feature

Delivers RF output in 30–960 MHz range with 1 kHz stepping and attenuation control.

Specifications

| Input | 1 channel ASI input, BNC interface, 75Ω |  |
| --- | --- | --- |
|  | 1 channel IP (MPTS/SPTS) input over UDP (Unicast/Multicast) (100M Port) |  |
| Modulation | QAM Channel | 1 |
|  | Standard | EN300 429/ITU-T J.83A/B/C, GB/T170 |
|  | Symbol Rate | 5.0~7.0Msps，1ksps stepping |
|  | Constellation | 16/32/64/128/256 QAM |
|  | FEC | RS (204, 188) |
| RF Output | Connector | F Type, 75Ω impedance |
|  | RF Range | 30~960MHz，1kHz Stepping |
|  | Output Attenuation | -30dBm~0dBm, 0.5dB Stepping |
| System | LCD/Keyboard operating and web NMS support |  |
|  | Ethernet software upgrading |  |
| General | Dimension(W*D*H) | 482mm×300mm×44mm |
|  | Weight | 2.3kg |
|  | Temperature | 0~45℃(Operation) ; -20~80℃(Storage) |
|  | Power | AC 110V±10%, 50/60Hz or AC 220V±10%, 50/60Hz |
|  | Consumption | 22W |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227