FMUSER FUQ-4415B DVB-T Modulator 128 IP Inputs 4 Non-Adjacent RF Outputs UDP-RTP DVB-T EN300744 Multiplexing Modulation for DTV Systems Product overview

The FMUSER FUQ-4415B is a DVB-T modulator with 128 IP inputs over UDP/RTP and outputs 4 DVB-T carriers and 4 MPTS streams. It integrates multiplexing and modulating functions for efficient signal processing. Features web-based NMS management for easy control. Ideal for DTV broadcasting systems, providing reliable transmission in modern headend setups.

Key feature

Enables web-based NMS management for monitoring and configuration.

Principle Chart

Specifications

| Input | 128 IP(SPTS/MPTS) input over UDP and RTP, RJ45, 1000M/100M, Unicast/Multicast |  |  |
| --- | --- | --- | --- |
| Multiplexing | Maximum PID Remapping | 256 per input channel |  |
|  | Function | PID Remapping |  |
|  |  | Accurate PCR adjusting |  |
|  |  | Generate PSI/SI table automatically |  |
| Modulation | DVB-T | Standard | EN300744 |
|  |  | FFT mode | 2K |
|  |  | Bandwidth | 6M, 7M, 8M |
|  |  | Constellation | QPSK, 16QAM, 64QAM |
|  |  | Guard Interval | 1/4, 1/8, 1/16, 1/32 |
|  |  | FEC | 1/2, 2/3, 3/4, 5/6, 7/8 |
|  |  | MER | ≥42 dB |
|  |  | RF frequency | 50~960MHz, 1KHz step |
|  |  | RF out | 1 RF port |
|  |  |  | 4*DVB-T non-adjacent carriers combined output , 40M total bandwidth limitation |
|  |  | RF output level | -28~ -3 dBm, 0.1db step |
| Stream Out | 4*MPTS over UDP and RTP out (RJ45, 1000M/100M Unicast/Multicast) |  |  |
| System | LCD/Keyboard operating and web-based NMS support |  |  |
|  | Ethernet software upgrading |  |  |
| General | Dimension (W*D*H) | 482mm×300mm×44.5mm |  |
|  | Temperature | 0~45℃(Operation) ; -20~80℃(Storage) |  |
|  | Power supply | AC 100V±1050/60Hz; AC 220V±10%, 50/60HZ |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227