FMUSER FUQ-4417J ISDB-T Modulator 24-48 Channels IP to RF Conversion 1024 IP Input ARIB STD-B31 Web-Based NMS Digital TV Broadcast Systems

Product Overview

The FMUSER FUQ-4417J is a high-density ISDB-T modulator converting IP streams to 24/48 RF carriers (50-960MHz) through 4xRJ45 and 2xSFP ports. It processes transport streams with PID remapping and PCR correction, featuring web-based control and 1U rack-mount design. Ideal for digital TV headends, cable network upgrades, and multiscreen distribution systems requiring simultaneous RF/IP output.

Key feature

Centralized management via web-based NMS with real-time monitoring

Inner Principle Chart - (24 carriers out per daughter card):

Carrier Setting Illustration

Specifications

| Input | Input | Max 1024 IP input through data ports 1 & 2， 100/1000M Ethernet Ports per daughter card (RJ45 and SFP interface alternative) |
| --- | --- | --- |
|  | Transport Protocol | TS over UDP/RTP, unicast and multicast, IGMP V2/V3 |
|  | Transmission Rate | Max 840Mbps for each GE input |
| Mux | Input Channel | 1024 IP streams |
|  | Output Channel | 24 (or 48) |
|  | Max PIDs | 256 per channel |
|  | Functions | PID remapping (auto/manually optional) |
|  |  | PCR accurate adjusting |
|  |  | PSI/SI table automatically generating |
| Modulation Parameters | Standard | ARIB STD-B31 |
|  | Bandwidth | 6M |
|  | Constellation | QPSK, 16QAM, 64QAM |
|  | Guard Interval | 1/32, 1/16, 1/8, 1/4 |
|  | Transmission Mode | 2K, 4K, 8K |
|  | Code rate | 1/2, 2/3, 3/4, 5/6, 7/8 |
|  | MER | ≥40dB |
|  | RF frequency | 50~960MHz, 1KHz step |
|  | RF output level | -20dBm~+10dBm(87~117dbµV), 0.1dB stepping |
|  | Output Channel | 24 (or 48) non-adjacent carriers output |
| RF Output | Interface | 2 F type port mirror out, 75Ω impedance - 24ch model 4 F type port (with 2 ports located in each daughter card), 75Ω impedance - 48ch model |
|  | ACLR | -50 dBc |
| IP output | 24 (or 48) IP output over UDP/RTP/RTSP through data ports 3&4, unicast/multicast, 100/1000M Ethernet Ports |  |
| System | Web-based NMS management |  |
| General | Demission | 480mm×327mm×44.5mm (WxLxH) |
|  | Weight | 5.5kg |
|  | Temperature | 0~45℃(operation), -20~80℃(storage) |
|  | Power Supply | AC 100V±10%, 50/60Hz or AC 220V±10%, 50/60Hz |

Order Guide

|  | 24ch Model | 48ch Model |
| --- | --- | --- |
| 1024 IP input to 24ch ISDB-T carriers out, 24 IP out (with single card) | x |  |
| 1024 x2 IP input to 24ch x2 ISDB-T carriers out, 24 x2 IP out (with two daughter cards) |  | x |
| 10G Ethernet | As per order | As per order |
| Dual Power Supply | As per order | As per order |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227