FMUSER FUQ-4417K ISDB-Tb Modulator 8-16 Non-Adjacent Carrier Output 512-1024 IP Input UDP-RTP-RTSP Output ARIB STD-B31 Advanced TS Processing DTV Broadcasting

* 3 GE ports for IP input and output

* Support 8 (or 16) multiplexed TS over UDP/RTP/RTSP output

* 8 (or 16) non-adjacent carriers output, compliant to ISDB-Tb (ARIB STD-B31)

* GE/SFP port alternative, with Data1 bi-directional for data in/out, and Data2-4 one-way for data in

* Supports 6 ASI input and 2 ASI output as mirror, TS file input through USB disk

* Support 8 (or 16) multiplexed TS over UDP/RTP/RTSP output

* 8 (or 16) ISDB-T non-adjacent carriers output, compliant to ARIB STD-B31 standard

Product Overview

The FMUSER FUQ-4417B converts IP streams into 8 or 16 non-adjacent carriers compliant with ARIB STD-B31. It performs modulation for ISDB-Tb broadcasting. Featuring web-based network management for centralized control. Ideal for DTV broadcasting systems and next-gen applications.

Key feature

Supports 8/16 non-adjacent carriers output compliant with ISDB-Tb (ARIB STD-B31).

Delivers 840Mbps throughput per GE input port with SFP/GE alternatives.

Features advanced signal processing including PID remapping, PSI/SI editing, and PCR adjusting.

Provides web-based network management for centralized control.

Offers multi-input support with 6 ASI inputs, USB TS file ingestion, and UDP/RTP/RTSP outputs.

Inner Principle Chart

(V 1, 8ch or 16ch out for option)

(V 2, 8ch or 16ch out for option)

Carrier Setting Illustration:

Specifications

| Input | Input | Max 512 (or 1024) IP input through 100/1000M Ethernet Port (SFP interface optional) over UDP/RTP, unicast and multicast, IGMP V2/V3, Max 840Mbps for each GE input |
| --- | --- | --- |
|  |  | 6 ASI input, BNC interface - for V2 only |
|  |  | TS file input through USB disk - for V2 only |
| Mux | Input Channel | 512 (or 1024) |
|  | Output Channel | 8 (or 16) |
|  | Max PIDs | 256 per channel |
|  | Functions | PID remapping (auto/manually optional) |
|  |  | PCR accurate adjusting |
|  |  | PSI/SI table automatically generating |
| Modulation Parameters | Standard | ARIB STD-B31 |
|  | Bandwidth | 6M |
|  | Constellation | QPSK, 16QAM, 64QAM |
|  | Guard Interval | 1/32, 1/16, 1/8, 1/4 |
|  | Transmission Mode | 2K, 4K, 8K |
|  | Code rate | 1/2, 2/3, 3/4, 5/6, 7/8 |
|  | MER | ≥40dB |
|  | RF frequency | 50~960MHz, 1KHz step |
|  | RF output level | -20dBm~+10dBm(87~117dbµV), 0.1dB stepping |
|  | Output Channel | 8 (or 16) non-adjacent carriers output |
| RF Output | Interface | 1 F type port, 75Ω impedance |
|  | ACLR | -50 dBc |
| TS output | 8 (or 16) IP output over UDP/RTP/RTSP, unicast/multicast 100/1000M Ethernet Ports |  |
|  | 2 ASI output, one as mirror - for V2 only |  |
| System | Web-based NMS management; LCD+Keyboard management - for V2 only |  |
| General | Demission | 480mm×327mm×44.5mm (WxLxH) |
|  | Weight | 5.5kg |
|  | Temperature | 0~45℃(operation), -20~80℃ (storage) |
|  | Power Supply | AC 100V±10%, 50/60Hz or AC 220V±10%, 50/60Hz |

Order Guide:

|  | V1 |  | V2 |  |
| --- | --- | --- | --- | --- |
|  | 8ch model | 16ch model | 8ch model | 16ch model |
| 512 IP input to 8ch ISDB-T carriers out, 8 IP out | x |  | x |  |
| 1024 IP input to 16ch ISDB-T carriers out, 16 IP out |  | x |  | x |
| 6 ASI input, and TS files input by USB disk |  |  | x | x |
| 2 ASI mirror out |  |  | x | x |
| LCD+Keyboard management |  |  | x | x |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227