FMUSER FUQ-4419BX ATSC Modulator 256 IP Inputs 8 RF Outputs GE Ports ATSC A-53 8VSB RS(208-188) Encoding Multiplexing Web-Based Control for Broadcasting and STB TestingProduct Overview

The FMUSER FUQ-4419BX is an ATSC modulator supporting up to 256 IP inputs via three GE ports and delivering 8 RF output channels. It performs multiplexing and modulation compliant with ATSC A/53, including 8VSB and RS(208,188) encoding, with web-based network management for control. Ideal for ATSC broadcasting network setup and set-top box testing.

Key feature

Supports high-capacity IP input with up to 256 streams via three GE ports

Provides robust ATSC modulation compliant with ATSC A/53, featuring 8VSB and RS(208,188) encoding

Enables advanced signal processing including PID remapping and PSI/SI editing

Features web-based network management for remote control and monitoring

Inner Principle Chart

Carrier Setting Illustration

Specifications

| Input | Input | Max 256 IP input through 3 (front-panel Data port, Data 1 and Data 2) 100/1000M Ethernet Port (SFP interface optional). Each Data1 or Data 2 port can input max 256 IP, while front-panel Data port can input max 128 IP |
| --- | --- | --- |
|  | Transport Protocol | TS over UDP/RTP, unicast and multicast, IGMP V2/V3 |
|  | Transmission Rate | Max 840Mbps for each input channel |
| Mux | Input Channel | 256 |
|  | Output Channel | 8 |
|  | Max PIDs | 180 per channel |
|  | Functions | PID remapping(auto/manually optional) |
|  |  | PCR accurate adjusting |
|  |  | PSI/SI table automatically generating |
| Modulation Parameters | Channel | 8 |
|  | Modulation Standard | ATSC A/53 |
|  | Constellation | 8VSB |
|  | Bandwidth | 6MHz |
|  | FEC | RS(208 188)+Trellis |
| RF Output | Interface | F typed output port for 8 non-adjacent carriers |
|  | RF Range | 50~960MHz, 1kHz stepping |
|  | Output Level | -20~+10dbm (for all carriers), 0.5db stepping |
|  | MER | ≥ 40dB |
|  | ACL | -55 dBc |
| TS output | 8 IP output over UDP/RTP/RTSP, unicast/multicast, 2 100/1000M Ethernet Ports |  |
| System | Web-based Network management |  |
| General | Demission | 420mm×440mm×44.5mm (WxLxH) |
|  | Weight | 3kg |
|  | Temperature | 0~45℃(operation), -20~80℃(storage) |
|  | Power Supply | AC 100V±10%, 50/60Hz or AC 220V±10%, 50/60Hz |
|  | Consumption | ≤20W |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227