FMUSER FUQ-4419U DVB-T Modulator 256-512-768 IP Input to 8-16-24 RF Output GE-SFP Interface TS over UDP-RTP QPSK-16QAM-64QAM ETSI EN300 744 PID Remapping PSI-SI Editing Digital TV Broadcasting Cable Headend IPTV Product Overview

The FMUSER FUQ-4419R is a DVB-T modulator that accepts up to 768 IP inputs via GE/SFP ports and outputs 8, 16, or 24 non-adjacent RF carriers (50-960MHz). It processes IP streams through multiplexing and modulation into DVB-T compliant signals. With high integration, low cost, and web-based network management, this device is ideal for digital TV broadcasting, cable headends, and IPTV systems.

Key feature

Supports up to 768 IP inputs via GE/SFP ports with max 840Mbps per input.

Features PID remapping, PSI/SI editing, and outputs DVB-T carriers compliant with ETSI EN300 744.

Enables web-based network management for easy configuration and monitoring.

Inner Principle Chart

Version I & II - For 8&16 carriers out):

Version III - For 24 carriers out)::

Carrier Setting Illustration

Specifications

| Input | Input | Max 256/512 IP input through 3 (front-panel Data port, Data 1 and Data 2) 100/1000M Ethernet Port (SFP interface optional). - For Version I & II Max 768 IP input through data ports 1 to 4 100/1000M Ethernet Ports (RJ45 and SFP interface alternative). –For Version III |
| --- | --- | --- |
|  | Transport Protocol | TS over UDP/RTP, unicast and multicast, IGMP V2/V3 |
|  | Transmission Rate | Max 840Mbps for each input channel |
| Mux | Input Channel | 256 IP streams - Version I 512 IP streams - Version II 768 IP streams - Version III |
|  | Output Channel | 8 (or 16, or 24) |
|  | Max PIDs | 180 per channel |
|  | Functions | PID remapping(auto/manually optional) |
|  |  | PCR accurate adjusting |
|  |  | PSI/SI table automatically generating |
| Modulation Parameters | Output Channel | 8 non-adjacent carriers output - Version I 16 non-adjacent carriers output - Version II 24 non-adjacent carriers output - Version III |
|  | Modulation Standard | ETSI EN300 744 |
|  | Constellation | QPSK/16QAM/64QAM |
|  | Bandwidth | 6/7/8 MHz |
|  | Trans mode | 2K/4K/8K |
|  | FEC | 1/2, 2/3, 3/4, 5/6, 7/8 |
| RF Output | Interface | F typed output port for 8 (or 16) non-adjacent carriers |
|  | RF Range | 50~960MHz, 1kHz stepping |
|  | Output Level | -20~+10dbm (for all carriers), 0.5db stepping |
|  | MER | ≥ 40dB |
|  | ACL | -55 dBc |
| TS output | 8 (or 16, or 24) IP output over UDP/RTP/RTSP, unicast/multicast, 2 100/1000M Ethernet Ports |  |
| System | Web-based Network management |  |
| General | Demission | 420mm×440mm×44.5mm (WxLxH) |
|  | Temperature | 0~45℃(operation), -20~80℃(storage) |
|  | Power Supply | AC 100V±10%, 50/60Hz or AC 220V±10%, 50/60Hz |

Order Guide:

|  | Version I | Version II | Version III |
| --- | --- | --- | --- |
| 256 IP input to 8ch DVB-T carriers out, 8 IP out | x |  |  |
| 512 IP input to 16ch DVB-T carriers out, 16 IP out |  | x |  |
| 768 IP input to 24ch DVB-T carriers out, 24 IP out |  |  | x |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227