FMUSER FUQ-4427 IP QAM Modulator 512 IP Inputs-16 QAM Outputs GE Ports DVB-C J.83 A-B Multiplexing Scrambling CATV Headend System Product Overview

The FMUSER FUQ-4427 is a 16-channel IP QAM modulator integrating multiplexing, scrambling, and modulating functions. It accepts up to 512 IP inputs via three GE ports and outputs 16 non-adjacent RF carriers compliant with DVB-C and ITU-T J.83 A/B standards. Managed through web-based network management, it enables easy configuration and monitoring. Ideal for CATV broadcasting and digital headend applications.

Key feature

Supports up to 512 IP inputs through three GE ports with 840Mbps per port.

Delivers 16 non-adjacent RF carriers compliant with DVB-C (EN 300 429) and ITU-T J.83 A/B standards.

Features DVB scrambling systems including ETR289, ETSI 101 197, and ETSI 103 197.

Enables web-based network management for configuration, monitoring, and control.

Supports PID remapping (up to 180 per channel), CA filtering, and PSI/SI editing.

Inner Principle Chart

Carrier Setting Illustration

Specifications

| Input | Input | Max 512 IP input through 3 (front-panel Data port, Data 1 and Data 2) 100/1000M Ethernet Port (SFP interface optional). Each Data1 or Data 2 port can input max 512 IP, while front-panel Data port can input max 128 IP |  |
| --- | --- | --- | --- |
|  | Transport Protocol | TS over UDP/RTP, unicast and multicast, IGMP V2/V3 |  |
|  | Transmission Rate | Max 840Mbps for each GE input |  |
| Mux | Input Channel | 512 |  |
|  | Output Channel | 16 |  |
|  | Max PIDs | 180 per channel |  |
|  | Functions | PID remapping(auto/manually optional) |  |
|  |  | PCR accurate adjusting |  |
|  |  | PSI/SI table automatically generating |  |
| Scrambling Parameters | Max simulscrypt CA | 4 |  |
|  | Scramble Standard | ETR289, ETSI 101 197, ETSI 103 197 |  |
|  | Connection | Local/remote connection |  |
| Modulation Parameters | DVB-C Modulator Sectioon | J.83A | Constellation : 16/32/64/128/256QAM |
|  |  |  | Bandwidth :8M |
|  |  | J.83B | Constellation : 64QAM/ 256QAM |
|  |  |  | Bandwidth :6M |
|  | QAM Channel | 16 non-adjacent carrier |  |
|  | Modulation Standard | EN300 429/ITU-T J.83A/B(DVB-C) |  |
|  | Symbol Rate | 5.0~7.0Msps, 1ksps stepping |  |
|  | FEC | RS (204, 188) |  |
| RF Output | Interface | 1 F typed output port for 16 carriers, 75Ω impedance |  |
|  | RF Range | 50~960MHz, 1kHz stepping |  |
|  | Output Level | -20dBm~+10dBm(87~117dbµV), 0.1dB stepping |  |
|  | MER | ≥ 40dB |  |
| TS output | 16 IP output over UDP/RTP/RTSP, unicast/multicast, 2 (Data1& Data2) 100/1000M Ethernet Ports |  |  |
| System | web-based Network management |  |  |
| General | Demission | 420mm×440mm×44.5mm (WxLxH) |  |
|  | Temperature | 0~45℃(operation), -20~80℃(storage) |  |
|  | Power Supply | AC 100V±10%, 50/60Hz or AC 220V±10%, 50/60Hz |  |

System Connection Sample (IP Based)

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227