FMUSER FUQ-4443 IP QAM Modulator 1024 IP Inputs 32 RF Outputs GE Ports with SFP DVB-C ITU-T J.83 A-B Multiplexing Scrambling Headend Broadcast System Product Overview

The FMUSER FUQ-4443 is an IP QAM modulator with 32-channel multiplexing, scrambling, and modulation, supporting up to 1024 IP inputs via GE ports at max 840Mbps each and outputting 32 non-adjacent carriers through dual RF ports (50-960MHz). It processes signals using PID remapping, PCR adjusting, and PSI/SI editing for optimization. This unit features web-based network management for remote control. Ideal for broadcast applications such as cable TV headends and secure broadcasting systems.

Key feature

Supports up to 1024 IP inputs via dual GE ports with optional SFP interface, max 840Mbps per port over UDP/RTP.

Outputs 32 non-adjacent QAM carriers via dual RF ports (50-960MHz), compliant with DVB-C (EN 300 429) and ITU-T J.83 A/B.

Integrates 32-channel multiplexing, scrambling, and modulation with PID remapping (up to 256 per channel), PCR adjusting, and PSI/SI editing.

Complies with DVB scrambling standards including ETR289, ETSI 101 197, and ETSI 103 197.

Features web-based network management for remote control and monitoring.

Inner Principle Chart

Carrier Setting Illustration

Specification

| Input | Input |  | 512×2 IP input, 2*100/1000M Ethernet Port(SFP interface optional) |  |
| --- | --- | --- | --- | --- |
|  | Transport Protocol |  | TS over UDP/RTP, unicast and multicast, IGMP V2/V3 |  |
|  | Transmission Rate |  | Max 840Mbps for each GE input |  |
| Mux | Input Channel |  | 1024 |  |
|  | Output Channel |  | 32 |  |
|  | Max PIDs |  | 256 per channel |  |
|  | Functions |  | PID remapping(auto/manually optional) |  |
|  |  |  | PCR accurate adjusting |  |
|  |  |  | PSI/SI table automatically generating |  |
| Scrambling Parameters | Max simulscrypt CA |  | 4 |  |
|  | Scramble Standard |  | ETR289, ETSI 101 197, ETSI 103 197 |  |
|  | Connection |  | Local/remote connection |  |
| Modulation Parameters | DVB-C Modulator Sectioon |  | J.83A | Constellation :16/32/64/128/256QAM |
|  |  |  |  | Bandwidth :8M |
|  |  |  | J.83B | Constellation :64/256QAM |
|  |  |  |  | Bandwidth :6M |
|  | QAM Channel |  | 32 non-adjacent carrier |  |
|  | Modulation Standard |  | EN300 429/ITU-T J.83A/B |  |
|  | Symbol Rate |  | 5.0~7.0Msps, 1ksps stepping |  |
|  | Constellation |  | 16, 32, 64 , 128, 256QAM |  |
|  | FEC |  | RS (204, 188) |  |
| RF Output | Interface |  | 2 F type output ports for 32 carriers, 75Ω |  |
|  | RF Range |  | 50~960MHz, 1kHz stepping |  |
|  | Output Level |  | -20dBm~+10dBm(87~117dbµV), 0.1dB stepping |  |
|  | MER |  | ≥ 40dB |  |
|  | ACLR |  | -60 dBc |  |
| TS output | 32 IP output over UDP/RTP/RTSP, unicast/multicast, 2*100/1000M Ethernet Ports (SFP) |  |  |  |
| System | Network management software (NMS) supporting |  |  |  |
| General | Demission | 420mm×440mm×44.5mm (WxLxH) |  |  |
|  | Weight | 3kg |  |  |
|  | Temperature | 0~45℃(operation), -20~80℃(storage) |  |  |
|  | Power Supply | AC 100V±10%, 50/60Hz or AC 220V±10%, 50/60Hz |  |  |
|  | Consumption | 15.4W |  |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227