FMUSER FUQ-4443 V2 IP QAM Modulator 1024 IP + 6 ASI Inputs 16-32 DVB-C Outputs IP-ASI-RF Interfaces ITU-T J.83 A-B Multiplexing Scrambling for Cable Headends Hotel IPTV

Product Overview

The FMUSER FUQ-4443 (V2) is an IP QAM modulator with inputs via 4 GE/SFP ports for up to 1024 IP streams and 6 ASI inputs. It processes signals by modulating them into 16 or 32 non-adjacent DVB-C carriers. Featuring web-based network management, this device is ideal for cost-effective DTV broadcasting systems.

Key feature

Enables web-based network management for remote monitoring and control.

Principle Chart

For 16 carriers out:

For 32 carriers out:

Carrier Setting Illustration:

Specifications

| Input | Input | 16/32 DVB-C carriers: Max 1024 IP input through Date1-Date4 100/1000M Ethernet Port (or SFP interface optional). Each Data port can input max 1024 IP. |  |
| --- | --- | --- | --- |
|  |  | 6 ASI input, BNC interface |  |
|  | Transport Protocol | TS over UDP/RTP, unicast and multicast, IGMP V2/V3 |  |
|  | Transmission Rate | Max 840Mbps for each GE input |  |
| Mux | Input Channel | 1024 |  |
|  | Output Channel | 16/32 |  |
|  | Max PIDs | 256 per channel |  |
|  | Functions | PID remapping(auto/manually optional) |  |
|  |  | PCR accurate adjusting |  |
|  |  | PSI/SI table automatically generating |  |
| Scrambling Parameters | Max simulscrypt CA | 6 |  |
|  | Scramble Standard | ETR289, ETSI 101 197, ETSI 103 197 |  |
|  | Connection | Local/remote connection |  |
| Modulation Parameters | DVB-C Modulator Section | J.83A | Constellation : 16/32/64/128/256QAM |
|  |  |  | Bandwidth :8M |
|  |  | J.83B | Constellation : 64QAM/ 256QAM |
|  |  |  | Bandwidth :6M |
|  | QAM Channel | 16/32 non-adjacent carrier |  |
|  | Modulation Standard | EN300 429/ITU-T J.83A/B(DVB-C) |  |
|  | Symbol Rate | 5.0~7.0Msps, 1ksps stepping |  |
|  | Constellation | 16, 32, 64 , 128, 256QAM |  |
|  | FEC | RS (204, 188) |  |
| RF Output | Interface | 1 F typed output port for 16/32 carriers, 75Ω |  |
|  | RF Range | 50~960MHz, 1kHz stepping |  |
|  | Output Level | -20dBm~+10dBm(87~117dbµV), 0.1dB stepping |  |
|  | MER | ≥ 40dB |  |
| TS output | 16/32 IP output over UDP/RTP/RTSP, unicast/multicast, Data1 100/1000M Ethernet Ports |  |  |
|  | 2 ASI output, one as mirror |  |  |
| System | Network management software (NMS) supporting |  |  |
| General | Demission | 420mm×440mm×44.5mm (WxLxH) |  |
|  | Temperature | 0~45℃(operation), -20~80℃(storage) |  |
|  | Power Supply | AC 100V±10%, 50/60Hz or AC 220V±10%, 50/60Hz |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227