FMUSER FUQ-4454B-C 4-Channel Encoder Modulator 4x CVBS-YPbPr-S-Video Input DVB-C Tuner & IP Re-Mux MPEG-2 Encoding QAM Modulation for CATV and IPTV Headends Product overview

The FUQ-4454B-C is a 1RU 4-in-1 mux-scrambling encoder modulator with 4× CVBS/YPbPr/S-Video inputs, 1-2× DVB-C tuners, and 2× IP interfaces. It performs MPEG-2 encoding, ETSI-compliant scrambling, and 16-256QAM modulation in a single pass. The unit features 30–999 MHz RF output with web/USB management and complies with DVB-C (EN300 429) and ITU-T J.83A/B/C. Ideal for CATV digital migration and IPTV edge QAM deployments.

Key feature

Modular upgradability: Features pluggable modules and dual web/USB control for future-proof operation.

Specifications

| Input | 1 or 2 DVB-C Tuner input for remux; 2 IP input for re-mux |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 4 CVBS/YPbPr/S-Video Input for encoding |  |  |  |  |
| Tuner Input | DVB-C | Standard | J.83A(DVB-C), J.83B, J.83C |  |  |
|  |  | Input Frequency | 30MHz-1000Mhz |  |  |
|  |  | Constellation | 16/32/64/128/256 QAM |  |  |
| Encoding | Video | Format | MPEG-2 MP@ML (4:2:0) |  |  |
|  |  | Interface | S-Video/YPbPr/CVBS*4 |  |  |
|  |  | Bitrate | 1-19.5Mbps |  |  |
|  |  | Resolution | 720*480_60i, 720*576_50i |  |  |
|  |  | Rate mode | VBR/CBR |  |  |
|  |  | Chroma | 4:2:0 |  |  |
|  |  | Aspect | 16:9,4:3, 1:1, 2.21:1 |  |  |
|  | Audio | Format | MPEG1 Layer II, AC3 |  |  |
|  |  | Sampling | 48KHz |  |  |
|  |  | Bitrate | 64kbps,96kbps,128kbps,192kbps,256kbps,320kbps |  |  |
| Multiplexing | Maximum PID Remapping | 128per input channel |  |  |  |
|  | Function | PID remapping (automatically or manually) |  |  |  |
|  |  | Accurate PCR adjusting |  |  |  |
|  |  | Generate PSI/SI table automatically |  |  |  |
| Scrambling | Maximum simulcrypt CA | 4 |  |  |  |
|  | Standard | ETSI 101 197, ETSI 103 197 |  |  |  |
|  | Connection | Local/remote connection |  |  |  |
| Modulation | QAM Channel | 4 |  |  |  |
|  | Standard | EN300 429/ITU-T J.83A/B/C |  |  |  |
|  | MER | ≥40db |  |  |  |
|  | RF frequency | 30~999MHz, 1KHz step |  |  |  |
|  | RF output level | -20~0dbm(87~107 dbµV),0.1db step |  |  |  |
|  | Symbol Rate | 5.0Msps~9.0Msps, 1ksps stepping |  |  |  |
|  | Constellation | 16/32/64/128/256QAM |  |  |  |
|  |  | J.83A |  | J.83B | J.83C |
|  | Constellation | 16/32/64/128/256QAM |  | 64/256 QAM | 64/256 QAM |
|  | Bandwidth | 8M |  | 6M | 6M |
| System | Local interface | LCD + control buttons |  |  |  |
|  | Remote management | Web NMS |  |  |  |
|  | Stream Out | 4*MPTS IP over UDP out (RJ45, 1000M, Unicast/Multicast) |  |  |  |
|  | Language | English |  |  |  |
|  | Software Upgrading | Web and USB port |  |  |  |
| General | Dimension (W*D*H) | 482mm×300mm×44.5mm |  |  |  |
|  | Weight | 3.7kg |  |  |  |
|  | Temperature | 0~45℃(Operation) ; -20~80℃(Storage) |  |  |  |
|  | Power | AC 100V±1050/60Hz; AC 220V±10%, 50/60HZ |  |  |  |
|  | Consumption | 25W |  |  |  |

Working Principle

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227