FMUSER FUQ-4454E QAM Modulator 8 Tuners-6 ASI-256 IP Inputs 4 RF Outputs DVB-C-EN300 429 ITU-T J.83 Mux Scrambling CATV Broadcasting

Product overview

The FUQ-4454E is a high-performance mux-scrambling QAM modulator featuring multiple input options including tuners, ASI, and IP ports, and 4 RF outputs. It processes signals through multiplexing, scrambling, and QAM modulation. With pluggable module design, UDP output support, and Web NMS, it ensures flexible control. Designed for modern CATV systems and broadcast applications, it delivers efficient signal distribution.

Key feature

Supports flexible input configurations with optional versions: Version 1 (8 tuners + 6 ASI + 256 IP) or Version 2 (4 tuners with CAM + 6 ASI)

Integrates 4-channel multiplexing, scrambling, and QAM modulation compliant with DVB-C and ITU-T J.83A/B/C

Delivers superior RF output with MER ≥40dB and adjustable frequency/level

Provides advanced output options including 4MPTS over UDP and BISS descrambling (Version 2)

Features user-friendly management via LCD display, keyboard, and Web NMS

Working Principle

Key feature

* Multiple Input ports optional:

8 DVB-C/T/(T)/ISDBT/S/S2/S2X/ATSC FTA Tuner + 6 ASI input ports+256 IP input (Version 1)

4 DVB-C/S/S2/ATSC Tuner with CAM+6 ASI input ports (Version 2)

* Support 4 channels multiplexing+4 channels scrambling+4 channels QAM modulating

* Fully support DVB-C (EN300 429) and ITU-T J.83A/B/C

* Fully support DVB general scrambling system description ETR289, simulcrypt standard ETSI 101 197 and ETSI 103 197

* Support accurate PCR adjusting

* Support PSI/SI editing and inserting

* Excellent RF output performance index, MER≥40db

* Support 4*MPTS output over UDP

* 4 CAM decrypt multiple programs from 4 Tuner input (Optional)

* Support BISS descrambling(only for Version 2)

* Support LCD display and keyboard

* Support Web management, Updates via web and USB

Specifications

| Input | 8 DVB-C/T/(T)/ISDBT/S/S2/S2X/ATSC FTA Tuner + 6 ASI input+256 IP input ------Version 1 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4 DVB-C/S/S2/ATSC Tuner with CAM + 6 ASI -------Version 2 |  |  |  |  |  |  |
| Tuner Section | For Version 1： Multi-mode tuners switchable (New Type) | DVB-C | Input Frequency |  | 60 ~890 MHz |  |  |
|  |  |  | Standard |  | J.83A, J.83C |  | J.83B |
|  |  |  | Constellation |  | 16/32/64/128/256 QAM |  | 64/256 QAM |
|  |  | DVB-T/(T) | Input Frequency |  | 60 ~890 MHz |  |  |
|  |  |  | Bandwidth |  | 5/6/7/8 M |  |  |
|  |  | ISDB-T | Input Frequency |  | 60 ~890 MHz |  |  |
|  | For Version 1 (New Type) | DVB-S | Input Frequency |  | 950~2150MHz |  |  |
|  |  |  | Symbol rate |  | 0.5~45Msps |  |  |
|  |  |  | Signal Strength |  | - 65- -25dBm |  |  |
|  |  |  | FEC |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |  |
|  |  |  | Constellation |  | QPSK |  |  |
|  |  |  | Max input bitrate |  | ≤120 Mbps |  |  |
|  |  | DVB-S2X | Input Frequency |  | 950~2150MHz |  |  |
|  |  |  | Symbol rate |  | QPSK/8PSK /16APSK :0.5~45 Msps 32APSK: 0.5~40Msps; |  |  |
|  |  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10 |  |  |
|  |  |  | Constellation |  | QPSK, 8PSK, 16APSK, 32APSK |  |  |
|  |  |  | Max input bitrate |  | ≤120 Mbps |  |  |
|  |  | DVB-S2X | Input Frequency |  | 950~2150MHz |  |  |
|  |  |  | Symbol rate |  | QPSK/8PSK /16APSK :0.5~45 Msps 8APSK: 0.5~40Msps 32APSK: 0.5~40Msps |  |  |
|  |  |  | FEC |  | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36 , 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 2/3-L, 32/45, 11/15, 7/9 |  |  |
|  |  |  | Constellation |  | QPSK, 8PSK, 8APSK, 16APSK, 32APSK |  |  |
|  |  |  | Max input bitrate |  | ≤120 Mbps |  |  |
|  | For Version 2 | DVB-C | Standard |  | J.83A(DVB-C), J.83B, J.83C |  |  |
|  |  |  | Input Frequency |  | 30~1000Mhz |  |  |
|  |  |  | Constellation |  | 16/32/64/128/256 QAM |  |  |
|  |  | DVB-S | Input Frequency |  | 950~2150MHz |  |  |
|  |  |  | Symbol rate |  | 2-45Msps |  |  |
|  |  |  | FEC |  | 1/2, 2/3, 3/4, 5/6, 7/8 |  |  |
|  |  |  | Signal Strength |  | -65~-25dBm |  |  |
|  |  |  | Constellation |  | QPSK |  |  |
|  |  | DVB-S2 | Input Frequency |  | 950~2150MHz |  |  |
|  |  |  | Symbol rate |  | QPSK/8PSK 1~45 Msps |  |  |
|  |  |  | FEC |  | 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 |  |  |
|  |  |  | Constellation |  | QPSK, 8PSK |  |  |
|  |  | ATSC | Input Frequency |  | 54~858MHz |  |  |
|  |  |  | Bandwidth |  | 6M |  |  |
|  |  |  | Constellation |  | 8VSB |  |  |
| Multiplexing | Maximum PID Remapping | 128per input channel |  |  |  |  |  |
|  | Function | PID remapping (automatically or manually) |  |  |  |  |  |
|  |  | Accurate PCR adjusting |  |  |  |  |  |
|  |  | Generate PSI/SI table automatically |  |  |  |  |  |
| Scrambling | Maximum simulcrypt CA | 4 |  |  |  |  |  |
|  | Standard | ETSI 101 197, ETSI 103 197 |  |  |  |  |  |
|  | Connection | Local/remote connection |  |  |  |  |  |
| Modulation | QAM Channel | 4 |  |  |  |  |  |
|  | Standard | EN300 429/ITU-T J.83A/B/C |  |  |  |  |  |
|  | MER | ≥40db |  |  |  |  |  |
|  | RF frequency | 30~999MHz, 1KHz step |  |  |  |  |  |
|  | RF output level | -10~0dbm,0.1db step ------Version 1 -20~0dbm,0.1db step ------Version 2 |  |  |  |  |  |
|  | Symbol Rate | 5.0Msps~9.0Msps, 1ksps stepping |  |  |  |  |  |
|  |  | J.83A |  | J.83B |  | J.83C |  |
|  | Constellation | 16/32/64/128/256QAM |  | 64/256 QAM |  | 64/256 QAM |  |
|  | Bandwidth | 8M |  | 6M |  | 6M |  |
| System | Local interface | LCD + control buttons |  |  |  |  |  |
|  | Remote management | Web NMS |  |  |  |  |  |
|  | Stream Out | 4*MPTS over UDP out (RJ45, 1000M, Unicast/Multicast) |  |  |  |  |  |
|  | Language | English and Chinese |  |  |  |  |  |
|  | Software Upgrading | Web and USB |  |  |  |  |  |
| General | Dimension (W*D*H) | 482mm×300mm×44.5mm |  |  |  |  |  |
|  | Temperature | 0~45℃(Operation) ; -20~80℃(Storage) |  |  |  |  |  |
|  | Power | AC 100V±1050/60Hz; AC 220V±10%, 50/60HZ |  |  |  |  |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227