FMUSER FUQ-4459AB-CD Quad-Channel DVB-T-DVB-C Modulator 128 IP Inputs 4 MPTS IP Outputs BISS Descrambling Web-Based NMS Control for Digital Broadcast NetworksProduct overview

The FUQ-4459AB/CD series are professional quad-channel modulators supporting 12× DVB-S2/T tuner inputs and 128 IP (UDP/RTP) streams. They perform MPTS multiplexing and DVB-T/C modulation (EN300744/J.83A/B) with BISS Mode 1/E descrambling. Featuring 1RU rack-mount design and web-based NMS control, these devices are ideal for multi-format headend systems, IP-to-RF conversion, and secure satellite-to-terrestrial redistribution.

Key feature

Complies with DVB-T EN300744 (TS) and DVB-C J.83A/B (CS) standards

FU-4459TS DVB-T Principle Chart

FU-4459CS DVB-C Principle Chart

Specifications

| Input | 12 DVB-S2 FTA Tuner input ports---Version I 12 DVB-T Tuner input ports---Version II 8 DVB-S2 FTA tuner+ 4 DVB-T tuner input---Version III 8 DVB-T FTA tuner+4 DVB-S2 tuner input---Version IV 128 IP(SPTS/MPTS) input over UDP and RTP, RJ45, 1000M/100M Unicast/Multicast |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Tuner Section | DVB-S | Input Frequency |  | 950-2150MHz |  |
|  |  | Symbol rate |  | 2-45Msps |  |
|  |  | Signal Strength |  | -65~-25dBm |  |
|  |  | FEC Demodulation |  | 1/2, 2/3, 3/4, 5/6, 7/8 QPSK |  |
|  | DVB-S2 | Input Frequency |  | 950-2150MHz |  |
|  |  | Symbol rate |  | QPSK 1~45Mbauds; 8PSK 2~30Mbauds |  |
|  |  | Code rate |  | 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 |  |
|  |  | Constellation |  | QPSK, 8PSK |  |
|  | DVB-T/(T) | Input Frequency |  | 30MHz ~999.999 MHz |  |
|  |  | Bandwidth |  | 6M, 7M, 8M |  |
| BISS descrambling | Mode 1, Mode E (Up to 850Mbps) (descramble individual program) |  |  |  |  |
| Multiplexing | Maximum PID Remapping |  | 256 per input channel |  |  |
|  | Function |  | PID remapping |  |  |
|  |  |  | A/V PID filtering (for FU-4459TS) |  |  |
|  |  |  | Accurate PCR adjusting |  |  |
|  |  |  | Generate PSI/SI table automatically |  |  |
| Modulation | DVB-T |  | Standard | EN300744 |  |
|  |  |  | FFT mode | 2K, |  |
|  |  |  | Bandwidth | 6M, 7M, 8M |  |
|  |  |  | Constellation | QPSK, 16QAM, 64QAM |  |
|  |  |  | Guard Interval | 1/4, 1/8, 1/16, 1/32 |  |
|  |  |  | FEC | 1/2, 2/3, 3/4, 5/6, 7/8 |  |
|  |  |  | MER | ≥42 dB |  |
|  |  |  | RF frequency | 50~960MHz, 1KHz step |  |
|  |  |  | RF out | 4*DVB-T adjacent carriers combined output, 40M total bandwidth limitation |  |
|  |  |  | RF output level | -28~ -3 dBm (79~105 dbµV), 0.1db step |  |
|  | DVB-C |  | Standard | J.83A (DVB-C), J.83B, |  |
|  |  |  | MER | ≥43dB |  |
|  |  |  | RF frequency | 50~960MHz, 1KHz step |  |
|  |  |  | RF output level | -25~ -1dbm (82~106 dbµV), 0.1db step |  |
|  |  |  | Symbol rate | 5.000~7.000Msps adjustable |  |
|  |  |  | RF Out | 4*DVB-C adjacent carriers combined output, 40M total bandwidth limitation |  |
|  |  |  |  | J.83A | J.83B |
|  |  |  | Constellation | 16/32/64/128/ 256 QAM | 64/ 256 QAM |
|  |  |  | Bandwidth | 8M | 6M |
| Stream out | 4*MPTS over UDP and RTP out , RJ45, 1000M/100M, Unicast/Multicast |  |  |  |  |
| System | web-based NMS support |  |  |  |  |
|  | Ethernet software upgrading |  |  |  |  |
| General | Dimension (W*D*H) |  | 482mm×300mm×44.5mm |  |  |
|  | Temperature |  | 0~45℃(Operation) ; -20~80℃(Storage) |  |  |
|  | Power |  | AC 100V±1050/60Hz; AC 220V±10%, 50/60HZ |  |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227