FMUSER FUQ-4459B DVB-C Modulator 8 Tuner Inputs 128 IP Inputs UDP-RTP 4 RF Outputs DVB-C-DVB-C-T-S-S2-S2X-ISDB-T-ATSC Multi-Mode Scrambling BISS Descrambling Web-Based NMS for Broadcast and CATV Its production is stopped temporarily, and the date for resuming production is not fixed yet.

Product overview

The FMUSER FUQ-4459D is a DVB-C modulator with 8 tuner inputs, 128 IP inputs via UDP/RTP, and 4 non-adjacent carrier outputs. It integrates multiplexing, scrambling, and modulation functions, supporting BISS descrambling. Managed through web-based NMS, this device is ideal for digital broadcasting networks, enabling flexible signal processing and secure content delivery.

Key feature

Supports multi-mode tuner inputs for DVB-C, DVB-T, DVB-S/S2/S2X, ATSC, and ISDB-T with CAM slots.

Handles up to 128 IP streams via UDP/RTP protocols.

Features integrated multiplexing, scrambling, and DVB-C modulation for 4 carrier outputs.

Enables BISS descrambling for secure decryption.

Facilitates remote management via web-based NMS.

Principle Chart

Specifications

| Input | 4 Tuner (DVB-C/T/(T)/S/S2/S2X/ISDBT optional) + 4 Tuner with CAM (DVB-C/T/(T)/S/S2/S2X/ATSC/ISDBT optional) 128 IP(SPTS/MPTS) input over UDP and RTP, RJ45, 1000M/100M , Unicast/Multicast |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Tuner Section | Multi-mode tuners switchable (New) | DVB-C |  |  | Input Frequency | 60~890 MHz |  |  |
|  |  |  |  |  | Standard | J.83A, J.83C |  | J.83B |
|  |  |  |  |  | Constellation | 16/32/64/128/256 QAM |  | 64/256 QAM |
|  |  | DVB-T/(T) |  |  | Input Frequency | 60~890 MHz |  |  |
|  |  |  |  |  | Bandwidth | 6/7/8 M |  |  |
|  |  | ISDB-T |  |  | Input Frequency | 60~890 MHz |  |  |
|  | Version 1 | DVB-S |  |  | Input Frequency | 950-2150MHz |  |  |
|  |  |  |  |  | Symbol rate | 0.5~45Msps |  |  |
|  |  |  |  |  | Signal Strength | -65~-25dBm |  |  |
|  |  |  |  |  | FEC | 1/2, 2/3, 3/4, 5/6, 7/8 QPSK |  |  |
|  |  | DVB-S2 |  |  | Input Frequency | 950 ~2150MHz |  |  |
|  |  |  |  |  | Symbol rate | QPSK: 1~45 Msps; 8PSK: 0.5~45 Msps |  |  |
|  |  |  |  |  | FEC | 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 |  |  |
|  |  |  |  |  | Constellation | QPSK, 8PSK |  |  |
|  | Version 2 (New) | DVB-S |  |  | Input Frequency | 950 ~2150MHz |  |  |
|  |  |  |  |  | Symbol rate | 0.5~45Msps |  |  |
|  |  |  |  |  | Signal Strength | - 65- -25dBm |  |  |
|  |  |  |  |  | FEC | 1/2, 2/3, 3/4, 5/6, 7/8 |  |  |
|  |  |  |  |  | Constellation | QPSK |  |  |
|  |  |  |  |  | Max input bitrate | ≤129Mbps |  |  |
|  |  | DVB-S2 |  |  | Input Frequency | 950 ~2150MHz |  |  |
|  |  |  |  |  | Symbol rate | QPSK/8PSK /16APSK :0.5~45 Msps 32APSK: 0.5~40Msps; |  |  |
|  |  |  |  |  | FEC | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10 32APSK: 3/4, 4/5, 5/6, 8/9, 9/10 |  |  |
|  |  |  |  |  | Constellation | QPSK, 8PSK, 16APSK, 32APSK |  |  |
|  |  |  |  |  | Max input bitrate | ≤129 Mbps |  |  |
|  |  | DVB-S2X |  |  | Input Frequency | 950 ~2150MHz |  |  |
|  |  |  |  |  | Symbol rate | QPSK/8PSK /16APSK :0.5~45 Msps 8APSK/32APSK: 0.5~40Msps |  |  |
|  |  |  |  |  | FEC | QPSK: 1/2, 3/5, 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 13/45, 9/20, 11/20 8PSK: 3/5, 2/3, 3/4, 5/6, 8/9, 9/10 8APSK: 5/9-L, 26/45-L 16APSK: 2/3, 3/4, 4/5, 5/6, 8/9, 9/10, 1/2-L, 8/15-L, 5/9-L, 26/45, 3/5, 3/5-L, 28/45, 23/36 , 2/3-L, 25/36, 13/18, 7/9, 77/90 32APSK: 3/4, 4/5, 5/6, 8/9, 2/3-L, 32/45, 11/15, 7/9 |  |  |
|  |  |  |  |  | Constellation | QPSK, 8PSK, 8APSK, 16APSK, 32APSK |  |  |
|  |  |  |  |  | Max input bitrate | ≤129 Mbps |  |  |
|  |  | ATSC-T |  |  | Input Frequency | 54~858MHz |  |  |
|  |  |  |  |  | Bandwidth | 6M |  |  |
|  |  |  |  |  | Constellation | 8VSB |  |  |
| Descrambling | CAM/CI |  |  |  | 4 sets |  |  |  |
| BISS Descrambling | Mode 1, Mode E (descramble individual program) |  |  |  |  |  |  |  |
| Multiplexing | Maximum PID Remapping |  | 256 per input channel |  |  |  |  |  |
|  | Function |  | A/V PID filtering |  |  |  |  |  |
|  |  |  | PID Remapping |  |  |  |  |  |
|  |  |  | Accurate PCR adjusting |  |  |  |  |  |
|  |  |  | Generate PSI/SI table automatically |  |  |  |  |  |
| Scrambling | Maximum simulcrypt CA |  | 4 |  |  |  |  |  |
|  | Standard |  | ETR289, ETSI 101 197, ETSI 103 197 |  |  |  |  |  |
|  | Connection |  | Local/remote connection |  |  |  |  |  |
| Modulation | DVB-C |  | Standard |  | J.83A (DVB-C), J.83B, |  |  |  |
|  |  |  | MER |  | ≥43dB |  |  |  |
|  |  |  | RF frequency |  | 50~960MHz, 1KHz step |  |  |  |
|  |  |  | RF output level |  | -25~ -1dbm(83~107 dbµV), 0.1db step |  |  |  |
|  |  |  | Symbol rate |  | 5.000~7.000Msps adjustable |  |  |  |
|  |  |  | RF Out |  | 4*DVB-C non-adjacent carriers combined output, 40M total bandwidth limitation |  |  |  |
|  |  |  | Constellation |  | J.83A |  | J.83B |  |
|  |  |  |  |  | 16/32/64/128/256QAM |  | 64/256 QAM |  |
|  |  |  | Bandwidth |  | 8M |  | 6M |  |
| Stream out | 4*MPTS over UDP and RTP out (RJ45, 1000M/100M Unicast/Multicast) |  |  |  |  |  |  |  |
| System | web-based NMS support |  |  |  |  |  |  |  |
|  | Ethernet software upgrading |  |  |  |  |  |  |  |
| General | Dimension |  |  | 482mm×300mm×44.5mm (W*D*H) |  |  |  |  |
|  | Temperature |  |  | 0~45℃(Operation) ; -20~80℃(Storage) |  |  |  |  |
|  | Power |  |  | AC 100V±1050/60Hz; AC 220V±10%, 50/60HZ |  |  |  |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227