FMUSER FUQ-4459E Modulator 1536 IP Input - 48 QAM Output 3 GE Ports DVB-C Multiplexing Scrambling CA Headend TV SystemsProduct Overview

The FMUSER FUQ-4459E is a multiplexing, scrambling, and modulating device with max 1536 IP inputs via three GE ports and 48 QAM outputs through dual RF ports. It processes signals through multiplexing, scrambling, and modulation, including PID remapping and RS (204,188) encoding. Featuring web-based management, this device is ideal for CATV headend systems and broadcast video distribution.

Key feature

- Handles max 1536 IP streams through three GE ports (SFP optional).

- Supports 6 simulcrypt CAs and 12 scrambling channels for security.

- Features PID remapping (up to 512 per channel), RS (204,188) encoding, and accurate PCR adjusting.

- Delivers 48 non-adjacent QAM carriers (50MHz–960MHz) and 48 IP outputs over UDP/RTP/RTSP.

- Provides web-based network management for configuration.

Inner Principle Chart

Carrier Setting Illustration

Specifications

| Input | Input |  | Max 1536 IP in thru 3 GE ports [Data 1 & Data 2 bi-directional ports, Data port (located on front panel), each port can input max 512 IP] 100/1000M self-adaption Ethernet Port (SFP interface optional) |  |
| --- | --- | --- | --- | --- |
|  | Transport Protocol |  | TS over UDP/RTP, unicast and multicast, IGMP V2/V3 |  |
|  | Transmission Rate |  | Max 840Mbps for each GE input |  |
| Mux | Input Channel |  | 1536 |  |
|  | Output Channel |  | 48 |  |
|  | Max PIDs Remapping |  | 512 per output channel |  |
|  | Functions |  | PID remapping (auto/manually optional) |  |
|  |  |  | PCR accurate adjusting |  |
|  |  |  | PSI/SI table automatically generating |  |
| Scrambling Parameters | Max simulcrypt CA |  | 6 |  |
|  | Scramble Standard |  | ETR289, ETSI 101 197, ETSI 103 197 |  |
|  | Scramble Channel |  | 12 |  |
|  | Connection |  | Local/remote connection |  |
| Modulation Parameters | DVB-C Modulator Sectioon |  | J.83A | Constellation :16/32/64/128/256QAM |
|  |  |  |  | Bandwidth :8M |
|  |  |  | J.83B/C | Constellation :64/256QAM |
|  |  |  |  | Bandwidth :6M |
|  | QAM Channel |  | 48 non-adjacent carrier |  |
|  | Modulation Standard |  | EN300 429/ITU-T J.83A/B/C |  |
|  | Symbol Rate |  | 3600~7000Ksps, 1ksps stepping 5057Ksps (J.83B, 64QAM) 5361Ksps (J.83B, 256QAM) |  |
|  | Constellation |  | 16, 32, 64 , 128, 256QAM |  |
|  | FEC |  | RS (204, 188) |  |
| RF Output | Interface |  | 2 F type output ports for 48 carriers, 75Ω Carrier 1~24 out thru RF1, 25~48 thru RF2 |  |
|  | RF Range |  | 50~960MHz, 1kHz stepping |  |
|  | Output Level |  | -20dBm~+10dBm(87~117dbµV), 0.1dB stepping |  |
|  | MER |  | ≥ 40dB |  |
| TS output | 48 IP output over UDP/RTP/RTSP, unicast/multicast, 2*100/1000M self-adaption, Ethernet Ports (SFP). IP 1~24 out thru Data1, IP 25~48 out thru Data2. |  |  |  |
| System | Network management software (NMS) supporting |  |  |  |
| General | Demission | 420mm×440mm×44.5mm (WxLxH) |  |  |
|  | Weight | 3kg |  |  |
|  | Temperature | 0~45℃(operation), -20~80℃(storage) |  |  |
|  | Power Supply | AC 100V±10%, 50/60Hz or AC 220V±10%, 50/60Hz |  |  |
|  | Consumption | 15.4W |  |  |

About Us

FMUSER is a company specializing in broadcasting and television transmission systems and integrating hotel IPTV systems. The company was established in 2009 and has served over 180 countries worldwide. More than 20,000 television stations, radio stations, and hotel clients from different.

Contact:Tomleequan    Email：sales@fmuser.com    whatsapp: +8613922702227